3GPP TSG-RAN4 Meeting #115	        R4-2507651 St Julians, Malta, 19th May – 23rd May 2025

Source: 	Qualcomm Inc.

Title: 	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth

Agenda Item:	7.8.2

Document for:	Discussion

WI Code:	NR_FR1_7MHz_BW-Core

# Introduction

RAN4 agreed that the signalling labels NS_12, NS_13 and NS_15 should be supported in band n26 for a 7 MHz channel bandwidth [1], it was also agreed to verify the signalling label NS_14 impact on 7 MHz channel bandwidth. In this proposal, we present a simulation study performed in band n26 with a power class 3 UE highlighting A-MPR behavior.

# Background

The WI objective is to support 7 MHz channel bandwidth in bands n5 and n26 for which the network signalling labels are defined as shown below. It was agreed in RAN4; there’s no impact on A-MPR due to NS_100 [2].

| NR band | Value of additionalSpectrumEmission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| n5 | NS_100 |  |  |  |  |
| n26 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |

All of the remaining four network signalling labels applicable to band n26 are applied for 7 MHz channel bandwidth and their impact on A-MPR is presented.

Additional requirements to be met by the UE when NS_12 is indicated in the cell are tabulated below.

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 7 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Additional requirements to be met by the UE when NS_13 is indicated in the cell are tabulated below.

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 7 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Additional requirements to be met by the UE when NS_15 is indicated in the cell are tabulated below.

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 7MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# Simulation Study

For analyzing the impact on Tx performance, we simulated the impact on achievable output power. A power class 3 (PC3) transmitter was calibrated to just meet the ACLR with DFT-s-OFDM waveform with Start_RB = 0 and L_CRB = 100 and MPR = 1 dB. Other parameters in the simulation set up are as follows :

- Channel Bandwidth = 7 MHz

- SCS = 15 kHz

- Maximum # of RBs = 35

- Band of Operation : n26 (Uplink : 814 – 849 MHz)

- Carrier Frequency : Varied based observed NS label

- LO leakage = 28 dBc

- Image = 28 dBc

- CIM3 = 60 dBc

- Network Signalling Labels : NS_12, NS_13, NS_14 and NS_15

- Waveforms : CP-OFDM and DFT-s-OFDM

- Modulation : QPSK, 16 QAM, 64 QAM and 256 QAM

Preliminary simulated results are presented below, further corroboration with bench measurements will be performed.

NS_12 :

Fc = 817.5 MHz

Carrier frequency of 817.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. The results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Figure 1: A-MPR behavior for NS_12 @ Fc = 817.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 5 dB and ≤ 7 dB respectively.

Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 5 dB and ≤ 7 dB respectively.

Proposal 2: A-MPR impact due to NS_12 is summarized in Tables 1 and 2 for all the remaining set of waveforms.

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7 MHz | Fc ≤ 817.5 | ≤2.88 | >0 | A3 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 | ≤ 5.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 9.5 |  | ≤ 7 |

NS_13 :

Fc = 820.5 MHz

Carrier frequency of 820.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. The results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Figure 2: A-MPR behavior for NS_13 @ Fc = 820.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Observation 2: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_13 is ≤ 4.5 dB and ≤6 dB respectively.

Proposal 3: A-MPR for NS_13 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 4.5 dB and ≤ 6 dB respectively.

Proposal 4: A-MPR impact due to NS_13 is summarized in Tables 3 and 4 for all the remaining set of waveforms.

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 7MHz | Fc = 820.5 | ≤1.8 | >0 | TBD |
|  |  | >1.8 | >2.16 | TBD |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

NS_14 :

Fc = 827.5 MHz

Carrier frequency of 827.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth and no impact on A-MPR is observed.

Observation 3: There’s no impact on A-MPR due to NS_14.

Proposal 5: A-MPR is not impacted for signalling label NS_14.

NS_15 :

Similar analysis is performed for NS_15 with three carrier frequencies over the entire allocation space of 7 MHz channel bandwidth. The A-MPR results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Fc = 845.5 MHz

Figure 3: A-MPR behavior for NS_15 @ Fc = 845.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Fc = 841.5 MHz

To be added

Figure 4: A-MPR behavior for NS_15 @ Fc = 841.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Fc = 817.5 MHz

To be added

Figure 5: A-MPR behavior for NS_15 @ Fc = 817.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Based on these simulation results Table 3 and Table 4 are updated with the chosen carrier frequency.

Observation 4: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for a carrier frequency of 845.5 MHz for NS_15 is ≤ 5.5 dB and ≤7 dB respectively.

Proposal 6: A-MPR for NS_15 for DFT-s-OFDM and CP-OFDM QPSK waveforms for a carrier frequency of 845.5 MHz shall be ≤ 5.5 dB and ≤ 7 dB respectively.

Proposal 7: A-MPR impact due to NS_15 is summarized in Tables 5 and 6 for all the remaining set of waveforms.

Table 5: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7 MHz | Fc = 845.5 MHz | TBD | TBD | TBD |
|  | 841.5 ≤ Fc < 845.5 | TBD | TBD | TBD |
|  | Fc < 841.5 | TBD | TBD | TBD |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |

Table 6: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  |

# Conclusions

In this contribution A-MPR analysis with network signalling labels NS_12 and NS_15 are presented in band n26 for 7 MHz channel bandwidth. Following observations and proposals are made:

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 5 dB and ≤ 7 dB respectively.

Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 5 dB and ≤ 7 dB respectively.

Proposal 2: A-MPR impact due to NS_12 is summarized in Tables 1 and 2 for all the remaining set of waveforms.

Observation 2: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_13 is ≤ 4.5 dB and ≤6 dB respectively.

Proposal 3: A-MPR for NS_13 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 4.5 dB and ≤ 6 dB respectively.

Observation 3: There’s no impact on A-MPR due to NS_14.

Proposal 5: A-MPR is not impacted for signalling label NS_14.

Observation 4: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for a carrier frequency of 845.5 MHz for NS_13 is ≤ 5.5 dB and ≤7 dB respectively.

Proposal 6: A-MPR for NS_15 for DFT-s-OFDM and CP-OFDM QPSK waveforms for a carrier frequency of 845.5 MHz shall be ≤ 5.5 dB and ≤ 7 dB respectively.

Proposal 7: A-MPR impact due to NS_15 is summarized in Tables 5 and 6 for all the remaining set of waveforms.

# References

[1] 	R4-2505098, WF on UE and BS RF requirements for 7MHz channel bandwidth, Ericsson

[2] 	R4-2502870, WF on requirements for 7MHz channel bandwidth, Ericsson