3GPP TSG-RAN WG4 Meeting # 115 	    R4-2507663

Malta, MT, 19 – 24 May 2025

Agenda Item:	7.21.5

Source: 	Ericsson

Title:	On core requirements for AI/ML based positioning

Document for:	Approval

Introduction

This contribution discusses the core requirements for AI/ML based positioning use cases in the revised WID [1]. Issues related to the core requirement for use case 1 are particularly discussed.

Discussion

Revised WID for AI/ML for NR air interface considers 3 positioning use cases identified as use case 1, use case 3a, and use case 3b [1]. In use case 1 the AI/ML model is deployed at the UE to support the AI/ML based positioning. The output of the AI/ML model in this use case is the location of the UE. In use case 3a the AI/ML model is deployed at the gNB to support the AI/ML based positioning. The output of the AI/ML model in this use case is the measurements that are used by the LMF for UE positioning. In use case 3b, the model is deployed at the LMF to support AI/ML based positioning. Like use case 1, output of the AI/ML model in this use case is the location of the UE in use case 3b. Use cases 3a and 3b are network-based use cases where the measurements are performed and used by the network node(s) for UE positioning. Time to perform measurement and time to report the measurement to the location server can be effectively handled via implementation. There is no need to define delay requirements to be met for AI/ML based positioning use cases 3a and 3b. Use case 1 on the other hand concerns a scenario where a UE can be configured to estimate/predict its location and report it to the network node, potentially within a certain time threshold, making this use case relevant for core requirement discussion.

To define the measurement delay requirement for the UE capable of supporting positioning use case 1, existing measurement period requirement for positioning measurements shall be used as a baseline. Requirements for RRC_CONNECTED mode where the measurements for positioning inference are performed by the UE inside the measurement gap should be prioritized.

Proposal 1: Existing measurement period requirement for positioning measurements is used as a baseline to define the measurement delay requirement for the UE capable of supporting positioning use case 1.

Proposal 2: Requirement for RRC_CONNECTED mode where the measurements for positioning inference are performed by the UE inside the measurement gap is prioritized.

The existing measurement period requirement for positioning measurements considers the impact of timing error group (TEG), carrier specific scaling factor (CSSF), Rx beam sweeping factor, and the number of samples on the measurement delay requirement. To adapt the existing measurement period requirement for use case 1, impact of these parameters on the measurement delay for inference in use case 1 needs to be identified.

In the legacy procedure TEG is configured only for the UE-assisted positioning measurements and has no relevance to the UE based positioning method. The same principle should also be applied to use case 1. No impact from the TEG should therefore be considered on the measurement period requirement for measurements performed by UE in use case 1 for positioning inference. CSSF on the other hand refers to the factor to account for the delay when UE performs measurement on PRS resources in different positioning frequency layers and shall be taken into account in the measurement period requirement for use case 1.

Proposal 3: No impact from TEG is considered on the measurement period requirement for measurements performed by UE in use case 1 for positioning inference.

Proposal 4: Impact of CSSF is taken into account for the measurement period requirement for use case 1.

Rx beam sweeping performed by UE in FR2 for PRS measurement is another factor that will have an impact on the measurement period for use case 1. Depending on the UE capability, UE can perform PRS measurement by employing Rx beam sweeping factor of 1, 2, 4, 6, or 8. Since the number of Rx beams performed by the UE to receive DL PRS resources is transparent in UE based solutions, it is not feasible to evaluate the impact of Rx beam sweeping on measurement period requirement for measurements performed by UE in use case 1 for positioning inference.

Observation 1: Number of Rx beams performed by the UE to receive DL PRS resources is transparent in UE based solutions.

The number of samples for positioning measurements is another factor that has an impact on the measurement delay requirement and the accuracy requirement. Since the issue on the feasibility of defining the accuracy requirement for use case 1 is yet to settled [2], the impact of number of samples on the measurement delay requirement for use case 1 cannot be evaluated at this point of time.

Observation 2: Issue on the feasibility of defining the accuracy requirement for use case 1 is yet to resolved in RAN4 and therefore the impact of number of samples on the measurement delay requirement for use case 1 cannot be evaluated at this point of time.

Summary

In this contribution, issues related to the core requirement for use case 1 in WID for AI/ML based positioning are discussed. The observations and proposals below summarize the discussions presented in this paper.

Proposal 1: Existing measurement period requirement for positioning measurements is used as a baseline to define the measurement delay requirement for the UE capable of supporting positioning use case 1.

Proposal 2: Requirement for RRC_CONNECTED mode where the measurements for positioning inference are performed by the UE inside the measurement gap is prioritized.

Proposal 3: No impact from TEG is considered on the measurement period requirement for measurements performed by UE in use case 1 for positioning inference.

Proposal 4: Impact of CSSF is taken into account for the measurement period requirement for use case 1.

Observation 1: Number of Rx beams performed by the UE to receive DL PRS resources is transparent in UE based solutions.

Observation 2: Issue on the feasibility of defining the accuracy requirement for use case 1 is yet to resolved in RAN4 and therefore the impact of number of samples on the measurement delay requirement for use case 1 cannot be evaluated at this point of time.

References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated.