# 3GPP TSG-RAN WG4 Meeting #115	R4-2507665

# Malta, MT, 19 – 24 May, 2025

Agenda Item:	7.23.2

Source: 	Ericsson

Title: 	TP for TR 38.744

Document for:	Approval

- Introduction

In this contribution, text proposal on scenarios for RRM measurement prediction and evaluation of alternative 1 and alternative 2 for ground truth extraction in FR1 testing to be captured in TR 38.744 are provided.

- Text proposal

< Start of text proposal >

6.2	Interoperability, testability, and RRM requirements

6.2.1 RRM requirements impact for measurement prediction

6.2.1.1 General

6.2.1.1.1	Scenarios

Scenarios valid for AI/ML aided mobility is captured in Table 1.

Table 1. Scenarios for AI/ML aided mobility.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

During the study item impact on RAN4 requirement was evaluated taking scenarios 2, 3, and 4 as candidate scenarios for temporal and frequency domain predictions. Discussion on scenarios 1, 5, and 6 during the work item phase is not precluded.

6.2.1.2 Potential impact to RRM requirements

6.2.2 RRM requirements impact for measurement event prediction

6.2.2.1 General

6.2.2.2 Potential impact to RRM requirements

6.2.3 Testability and Interoperability

6.2.3.1 Testing goal

6.2.3.2 Testing setup

6.2.3.2.1	Ground truth extraction in testing in FR1

RAN4 has identified two alternatives of ground truth extraction in testing in FR1. The analysis of the identified alternatives is provided in Table 2.

Table 2. Analysis of alternatives for testing in FR1.

| Issues | Alternative 1 | Alternative 2 |
| --- | --- | --- |
| Testing method | Conducted testing |  |
| Ground truth | - Calculated based on known parameters/conditions such as transmission power, reception power, knowledge of propagation condition. | - Based on the measurements reported by UE under certain condition. - Conditions under which the UE reported values can be used as ground truth in FR1 testing for RRM measurement prediction needs to be studied/identified. |
| Availability of timing information related to measurement/prediction | - Reporting framework for the AI/ML aided RRM measurement reporting is yet to be concluded. - It is unclear whether timing information related to the measurements and/or prediction can be assumed known to the TE. - Lack of timing related to measurement/prediction at TE will have an impact on both alternatives. |  |
| Impact of L3 filtering | - If the requirements for predicted cell level measurement are tested without L3 filtering, then there is no impact of L3 filtering on ground truth extraction in FR1 testing. |  |
| Number of samples for L1-filtering | - Ground truth in the test is calculated based on the number of samples for which the requirements are defined for. | - Number of samples used by the UE during prediction and ground truth measurement reporting is assumed to be same. |
| Channel variation | - No issues as test run for ground truth extraction is not needed. | - Test needs to run at least 2 times while ground truth extraction is based on alternative 2. - In one of the test runs UE is configured to report the predicted measurements. - In the second test run, the UE needs to be configured to report the ground truth measurement. - Channel condition may change between test runs thus impacting the ground truth extraction for the time instant of the predicted measurement in alternative 2. |
| NOTE: Ground truth value in testing should refer to the true value of the measurement for which the prediction is performed by the UE. |  |  |

<End of text proposal>