3GPP TSG-RAN WG4 Meeting #115	    R4-2507669

Malta, MT, 19 – 24 May 2025

Agenda Item:	7.23.4

Source: 	Ericsson

Title:	On testing of AI/ML based mobility

Document for:	Approval

Introduction

This contribution discusses issues related to testing for AI/ML aided mobility. The issues related to testing of intra-cell scenario and inter-cell scenario, ground truth extraction in testing, and the relevance of propagation condition for modeling the impact of UE trajectory in the test are discussed.

Discussion

Testing for Intra-cell Scenario

In the last meeting number of cells to be considered in the testing setup was discussed. For intra-cell scenario it was agreed to consider two-cell setup, where one of the cells is the serving cell and another cell is the target cell for intra-cell RRM measurement/event prediction. Whether more than 2 cells need to be considered for the inter-cell scenario was agreed for further study.

Agreement from RAN4#114bis [1]:

- RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction.

- FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

Intra-cell scenarios characterize deployment of AI/ML model at UE to predict the RRM measurements in temporal domain, where the measurements performed within observation window (OW) are used by the UE to perdict measurements valid for time instances inside the prediction window (PW) for the same cell as shown in Figure 1. The UE may report the measurements performed inside the OW and/or the measurements predicted to be valid for the instances inside the PW depending on the measurement prediction condition/configuration received from the network. Testing in intra-cell scenario should therefore be to validate that the measurements reported by the UE for the target cell in the test meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Proposal 1: Testing in intra-cell scenario should validate the measurements reported by the UE for the target cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Intra-cell temporal prediction case-A.

Intra-cell temporal prediction case-B.

Figure 1. Measurement timeline for intra-cell temporal prediction scenarios.

Testing for Inter-cell Scenario

Inter-cell scenario characterizes deployment of AI/ML model to predict RRM measurement of a cell in one carrier frequency by exploiting the measurements performed by UE on another cell in another carrier frequency. The carrier frequency of measured cell and the predicted cell belong to the same frequency region. For testing of requirements in inter-cell scenario, 2-cell setup will be required, where cell 1 in the test setup can be considered as the cell for measurement and cell 2 in the test setup can be considered as the target cell for which the UE predicts the measurement for. Testing in inter-cell scenario should therefore be performed to validate the measurements reported by the UE in the test:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 2: 2-cell setup is used to validate requirements for inter-cell prediction.

- Cell 1 in the test can be considered as the cell for measurement.

- Cell 2 in the test can be considered as the target cell for measurement prediction.

Proposal 3: Testing in inter-cell scenario should validate the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Ground Truth Extraction in Test

In previous meetings ground truth for the predicted measurements in the testing was discussed. It was agreed that the ground truth extraction in testing may differentiate between FR1 and FR2. For testing in FR2 it has been agreed that the ground truth of L3 RSRP is the reported L3 RSRP measurement under sufficient high SNR condition. Extraction of ground truth for testing in FR1 is pending agreement in RAN4.

Agreements from RAN4#114 [2]:

- The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

- Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered.

  - Option 1-1-FR2: The ground truth of L3 RSRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

- Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

- Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

- Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as:

  - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

  - Impact of L3 filtering

  - Number of samples for L1 filtering

  - Timing window, if it exists, where UE can’t measure the ground truth in Alt2

  - channel condition

  - Avoid UE cheat in the test

  - S(I)NR variations in the test at different time instances (for Alt2)

  - Other aspects are not precluded

- Companies are encouraged to provide analysis on all or some of the aspects in the list

Agreement from RAN4#114bis [1]:

- During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase.

  - At least capture the definition of the two Alternatives in the TR.

Ground truth value in testing should refer to the true value of the measurement for which the prediction is performed by the UE. Alternative 1 and alternative 2 agreed in RAN4#114 mainly differs in the ways ground truth value is determined in testing. In alternative 1 ground truth value is determined based on the transmission power and channel condition that are known to the TE. Ground truth values in this alternative can in principle be calculated for any time instant in the test for which the prediction of RRM measurement is performed/reported by the UE. This approach is feasible given that in RAN4#112 it was agreed to consider conducted testing as baseline for FR1 testing, where TE and DUT are connected through a well calibrated cable in the test [3]. In alternative 2, ground truth value is the UE measured value under certain conditions that is reported to the TE to be used to validate the accuracy of the predicted measurements by the UE. Since UE reported values cannot be error free, the conditions under which the UE reported values can be used as ground truth in testing needs to be studied and identified before alternative 2 can be considered for ground truth extraction in FR1 testing for RRM measurement prediction.

Observation 1: Ground truth value in testing should refer to the true value of the measurement for which the prediction is performed by the UE.

Observation 2: Alternative 1 is feasible given that in RAN4#112 it was agreed to consider conducted testing as baseline for FR1 testing.

Observation 3: Conditions under which the UE reported values can be used as ground truth in FR1 testing is unclear in alternative 2.

Proposal 4: RAN4 to study and identify conditions under which the UE reported values can be used as ground truth in alternative 2 for FR1 testing for RRM measurement prediction.

In the discussions to follow, alternative 1 and alternative 2 are further analayzed based on the aspects that were agreed to be considered while evaluating alternative 1 and alternative 2 for FR1 testing

Availability of timing information related to measurement/prediction

Reporting framework for the AI/ML aided RRM measurement reporting is yet be concluded. It is unlikely that details of the reporting framework will be settled by RAN2 by the end of the study item phase. Therefore, at this point of time, whether the timing information related to the measurements and/or prediction can be assumed known to the TE is unclear. Lack of timing related to measurement/prediction at TE will have an impact on both alteratives.

Observation 4: Reporting framework for the AI/ML aided RRM measurement reporting is yet to be concluded.

Observation 5: It is unclear whether timing information related to the measurements and/or prediction can be assumed known to the TE.

Observation 6: Lack of timing related to measurement/prediction at TE will have an impact on both alteratives.

Impact of L3 filtering

In testing for legacy RRM measurements, accuracy requirements are tested for the measurements without L3 filtering. The same approach can also be considered for the predicted RRM measurements. If the requirements for predicted cell level measurement are tested without L3 filtering, then there is no impact of L3 filtering on ground truth extraction in FR1 testing. This applies to both alternatives under discussion.

Observation 7: If the requirements for predicted cell level measurement are tested without L3 filtering, then there is no impact of L3 filtering on ground truth extraction in FR1 testing.

Number of sample for L1-filtering

L1 filtering is UE implementation specific and is not known to the TE. This issue however does not limit the applicability of alternative 1 for ground truth extraction in testing. Ground truth in the test can be calculated based on the number of samples for which the requirements are defined for. Regardless of the number of samples used by the UE to predict the RRM measurement, UE shall be considered passing the test provided the requirement for prediction accuracy is met.

Observation 8: Based on alternative 1, ground truth in the test can be calculated based on the number of samples for which the requirements are defined for.

Observation 9: Regardless of the number of samples used by the UE to predict the RRM measurement, UE shall be considered passing the test provided the requirement for prediction accuracy is met.

Channel variation in test runs

Unlike in alternative 1, where the ground truth can be calculated based on the known parameters before running the test, alternative 2 requires at least two runs of tests. In one of the runs UE is configured to report the predicted measurements. In the second run, the UE needs to be configured to report the ground truth. To assess accuracy of the predicted measurements, it is imperative that the channel condition remains same in two runs of the tests such that the predicted measurements for a time instant can be compared with the UE reported ground truth value for the same time instant. Given that the testing needs to be done for dynamically varying channel condition, realizing the same channel condition in multiple runs is a challenge that may result into a bottle neck for implementing alternative 2 for ground truth extraction in testing.

Observation 10: Test needs to run at least 2 times while ground truth extraction is based on alternative 2.

- In one of the test runs UE is configured to report the predicted measurements.

- In the second test run, the UE needs to be configured to report the ground truth measurement.

Observation 11: Channel condition may change between test runs thus impacting the ground truth extraction for the time instant of the predicted measurement in alternative 2.

Based on the observations and the analysis of different aspects related to testing presented above, alternative 1 is our preferred approach for ground truth extraction. We would however like to acknowledge that some issues that are of relevance before testing framework for FR1 can be concluded are yet to be settled and may not be finalized before the end of the study item. We are therefore open to keep both alternatives alive and have more discussion in the WI phase when issues related to availability of timing information related to measurement/prediction, and the point in the measurement model where the requirement will be applied for RRM measurement prediction are addressed.

Table 1. Analysis of alternatives for testing in FR1.

| Issues | Alternative 1 | Alternative 2 |
| --- | --- | --- |
| Testing method | Conducted testing |  |
| Ground truth | - Calculated based on known parameters/conditions such as transmission power, reception power, knowledge of propagation condition. | - Based on the measurements reported by UE under certain condition. - Conditions under which the UE reported values can be used as ground truth in FR1 testing for RRM measurement prediction needs to be studied/identified. |
| Availability of timing information related to measurement/prediction | - Reporting framework for the AI/ML aided RRM measurement reporting is yet to be concluded. - It is unclear whether timing information related to the measurements and/or prediction can be assumed known to the TE. - Lack of timing related to measurement/prediction at TE will have an impact on both alteratives. |  |
| Impact of L3 filtering | - If the requirements for predicted cell level measurement are tested without L3 filtering, then there is no impact of L3 filtering on ground truth extraction in FR1 testing. |  |
| Number of samples for L1-filtering | - Ground truth in the test is calculated based on the number of samples for which the requirements are defined for. | - Number of samples used by the UE during prediction and ground truth measurement reporting is assumed to be same. |
| Channel variation | - No issues as test run for ground truth extraction is not needed. | - Test needs to run at least 2 times while ground truth extraction is based on alternative 2. - In one of the test runs UE is configured to report the predicted measurements. - In the second test run, the UE needs to be configured to report the ground truth measurement. - Channel condition may change between test runs thus impacting the ground truth extraction for the time instant of the predicted measurement in alternative 2. |
| NOTE: Ground truth value in testing should refer to the true value of the measurement for which the prediction is performed by the UE. |  |  |

Proposal 5: RAN4 to keep both alternatives open and have more discussion in the WI phase when issues related to availability of timing information related to measurement/prediction, and the point in the measurement model where the requirement will be applied for RRM measurement prediction are addressed.

Proposal 6: Capture analysis of alternatives for ground truth extraction in FR1 testing in Table 1 in TR38.744.

UE Trajectory in Test

In the last meeting modelling of time varying characteristics per cell/site to incorporate controlled randomness in the radio environment due to UE trajectories was discussed. The issue is relevant and needs to be addressed to have a complete overview of the testing setup for the validation of the RRM measurement prediction requirements. The issue in our understanding concerns to the aspects in addition to the propagation condition that need to be considered to validate the RRM measurement prediction requirements. Since the propagation model for testing has not been discussed/concluded yet, it is our proposal to first agree on the propagation model for testing and then come back to this issue.

Agreement from RAN4#114bis [1]:

- To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories.

  - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation

  - RAN4 to prioritize discussion on FR1.

Proposal 7: RAN4 to conclude propagation condition for the validation of the RRM measurement prediction requirements before discussing the issues related to modelling of time varying characteristics per cell/site to incorporate controlled randomness in the radio environment due to UE trajectories.

Summary

In this paper, issues related to testing of intra-cell scenario and inter-cell scenario, ground truth extraction in testing, and the relevance of propagation condition for modeling the impact of UE trajectory in the test are discussed. The proposals and observations below summarize the discussion presented in this paper.

Proposal 1: Testing in intra-cell scenario should validate the measurements reported by the UE for the target cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Proposal 2: 2-cell setup is used to validate requirements for inter-cell prediction.

- Cell 1 in the test can be considered as the cell for measurement.

- Cell 2 in the test can be considered as the target cell for measurement prediction.

Proposal 3: Testing in inter-cell scenario should validate the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Observation 1: Ground truth value in testing should refer to the true value of the measurement for which the prediction is performed by the UE.

Observation 2: Alternative 1 is feasible given that in RAN4#112 it was agreed to consider conducted testing as baseline for FR1 testing.

Observation 3: Conditions under which the UE reported values can be used as ground truth in FR1 testing is unclear in alternative 2.

Proposal 4: RAN4 to study and identify conditions under which the UE reported values can be used as ground truth in alternative 2 for FR1 testing for RRM measurement prediction.

Observation 4: Reporting framework for the AI/ML aided RRM measurement reporting is yet to be concluded.

Observation 5: It is unclear whether timing information related to the measurements and/or prediction can be assumed known to the TE.

Observation 6: Lack of timing related to measurement/prediction at TE will have an impact on both alteratives.

Observation 7: If the requirements for predicted cell level L3 measurement is agreed to be applied before L3 filtering, then there is no impact of L3 filtering on ground truth extraction in FR1 testing.

Observation 8: Based on alternative 1, ground truth in the test can be calculated based on the number of samples for which the requirements are defined for.

Observation 9: Regardless of the number of samples used by the UE to predict the RRM measurement, UE shall be considered passing the test provided the requirement for prediction accuracy is met.

Observation 10: Test needs to run at least 2 times while ground truth extraction is based on alternative 2.

- In one of the test runs UE is configured to report the predicted measurements.

- In the second test run, the UE needs to be configured to report the ground truth measurement.

Observation 11: Channel condition may change between test runs thus impacting the ground truth extraction for the time instant of the predicted measurement in alternative 2.

Proposal 5: RAN4 to keep both alternatives open and have more discussion in the WI phase when issues related to availability of timing information related to measurement/prediction, and the point in the measurement model where the requirement will be applied for RRM measurement prediction are addressed.

Proposal 6: Capture analysis of alternatives for ground truth extraction in FR1 testing in Table 1 in TR38.744.

Table 1. Analysis of alternatives for testing in FR1.

| Issues | Alternative 1 | Alternative 2 |
| --- | --- | --- |
| Testing method | Conducted testing |  |
| Ground truth | - Calculated based on known parameters/conditions such as transmission power, reception power, knowledge of propagation condition. | - Based on the measurements reported by UE under certain condition. - Conditions under which the UE reported values can be used as ground truth in FR1 testing for RRM measurement prediction needs to be studied/identified. |
| Availability of timing information related to measurement/prediction | - Reporting framework for the AI/ML aided RRM measurement reporting is yet to be concluded. - It is unclear whether timing information related to the measurements and/or prediction can be assumed known to the TE. - Lack of timing related to measurement/prediction at TE will have an impact on both alteratives. |  |
| Impact of L3 filtering | - If the requirements for predicted cell level measurement are tested without L3 filtering, then there is no impact of L3 filtering on ground truth extraction in FR1 testing. |  |
| Number of samples for L1-filtering | - Ground truth in the test is calculated based on the number of samples for which the requirements are defined for. | - Number of samples used by the UE during prediction and ground truth measurement reporting is assumed to be same. |
| Channel variation | - No issues as test run for ground truth extraction is not needed. | - Test needs to run at least 2 times while ground truth extraction is based on alternative 2. - In one of the test runs UE is configured to report the predicted measurements. - In the second test run, the UE needs to be configured to report the ground truth measurement. - Channel condition may change between test runs thus impacting the ground truth extraction for the time instant of the predicted measurement in alternative 2. |
| NOTE: Ground truth value in testing should refer to the true value of the measurement for which the prediction is performed by the UE. |  |  |

Proposal 7: RAN4 to conclude propagation condition for the validation of the RRM measurement prediction requirements before discussing the issues related to modelling of time varying characteristics per cell/site to incorporate controlled randomness in the radio environment due to UE trajectories.

References

- R4-2504901, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).