# 3GPP TSG-RAN WG4 Meeting #115		R4-2507670

St. Julian’s, Malta, 19th to 23rd May 2025

Agenda Item:	7.13.3.3

Source:	Ericsson

Title:	On Transmitter Spurious Emission Co-existence discussion

Document for:	Approval

# 1	Introduction

LS from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence Tx spurious emission requirement levels for BS. It was acknowledged by RAN4 in the response LS [2].

The co-existence spurious emission requirement was originally derived in 1999 for UTRA. The technical background at that time was very much focused on the introduction of UTRA FDD/TDD operating at 2 GHz. The assumptions used as references to the original requirement definition can be found in a contribution [3] presented at RAN4#113 in Orlando. Over the years RAN4 have defined new operating bands below and above 2 GHz using the same requirement level. In current specification, bands are defined within the range 410 MHz up to 7125 MHz with the same requirement level, except for a few occasions (e.g. protection of GSM). Therefore, it is now time to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

The agreement is captured in a moderator summary as an outcome of the discussion related to WID update for Rel-19 BS RF evolution WI in RP-243288 [4].

The objective can be summarized as:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

In this contribution we share initial simulation evaluations based on the agreements from the last meeting.

# 2	Discussion

In this contribution, we present simulation assumptions and simulation methodology used to evaluate the transmitter co-existence spurious emission requirement for BS defined to protect the downlink of BS/ UE of another network in the same geographical area.

## 2.1	Simulation Assumptions

### 2.1.1	Co-existence simulation scenarios

Table 2.2-1 summarizes the initial proposed simulation scenarios.

Table 2.1.1-1: Summary of simulation scenarios

| No. | Aggressor Simulating frequency | Victim Simulating frequency | Direction | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 GHz/ 3.5 GHz | 7 GHz | BS DL to UE UL | Urban macro | First priority |
| 2 | 2 GHz/ 3.5 GHz | 7 GHz | BS DL to BS UL | Urban macro | First priority |
| 3 | 2 GHz/ 3.5 GHz | 7 GHz | BS DL to UE UL | Micro | Second priority |
| 4 | 2 GHz/ 3.5 GHz | 7 GHz | BS DL to BS UL | Micro | Second priority |
| Note: RAN4 to prioritize Macro deployments. |  |  |  |  |  |

## 2.1.2	Network layout model

### 2.1.2.1	Urban macro

The details on urban macro network layout model are listed in Table 2.1.2.1-1 below.

Table 2.1.2.1-1 : Network assumptions for Urban Macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | For BS-to-BS operation, network grid shift is 50 &100% |
| Inter-site distance |  | 500 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | 100% Outdoor |  |
|  | Indoor UE ratio | 0% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1.5 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Figure 2.1.2.1-1: Network layout with Grid shift

## 2.1.3	Propagation Model

### 2.1.3.1	Pathloss

The pathloss models are summarized in Table 7.4.1-1 from TR 38.901.

### 2.1.3.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1. from TR 38.901.

### 2.1.3.3	O-to-I penetration loss

The pathloss incorporating O2I building penetration loss is described in Clause 7.4.3.1 from TR 38.901.

## 2.1.4	Antenna configuration

### 2.1.4.1	Modelling of antenna gain for Out-of-band emissions

In the way forward from previous meeting, it was unclear how the gains from the out-of-band emissions will be modelled in the beamforming systems and was left for discussions in future meeting.

As stated in TR 37.840 Clause 5.4.4.1.4 [5] and TR 38.922 Clause 7.3.2.1.1 [6], the unwanted signals can under different circumstances be either regarded as fully correlated, ρ = 1, or un-correlated, ρ = 0. As the type of unwanted signal is implementation specific and hence unknown, a correlation matrix is used to explore different correlation levels.

Assuming uncorrelated signals can be used as a baseline.

In the uncorrelated case (ρ = 0), the array gain is effectively lost, and the radiation pattern becomes equivalent to that of a single antenna element or sub-array, rather than a directional beamformed pattern experience the array factor directivity. This is because the randomness in the inputs prevents coherent summation across the array due to decorrelation effects from transmission lines, filters out-of-band characteristics and mixture of emission contributions (such as wide-band noise and IM).

When signals are uncorrelated (ρ = 0), the antenna array does not exhibit array factor beamforming gain.

RAN4 should model unwanted emissions as uncorrelated signals to simplify the system level evaluation.

### 2.1.4.2	Macro AAS BS antenna

In the previous meeting, it was agreed to re-use the parameters from the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz documented in TR 38.922.

- For the aggressor frequency 2 GHz and 3.5 GHz, the antenna parameters are re-used from 4400 to 4800 MHz frequency range found in Table 4.4.1.2-1 TR 38.922.

- For the victim frequency 7 GHz, the antenna parameters are re-used from 7125 to 8400 MHz frequency range found in Table 6.5.1.2-1 TR 38.922.

### 2.1.4.3	Macro Non-AAS BS antenna

For the non-AAS antenna at the aggressor frequency 2 GHz and 3.5 GHz, the Table 5.4.4.2.1-1 in TR 37.840 states parameters for typical passive antenna types shared below.

| Antenna type | A1 | A5 | A10 | A15 | B5 | B10 | B15 | D5 | D10 | D15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No of radiation elements per column | 1 | 5 | 10 | 15 | 5 | 10 | 15 | 5 | 10 | 15 |
| No of columns | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 4 | 4 | 4 |
| Max array gain for a single column / dBi | 8.7 | 15 | 18 | 19.5 | 14.5 | 17 | 18.5 | 14.5 | 17 | 18.5 |
| Max antenna gain / dBi | 8.7 | 15 | 18 | 19.5 | 17 | 19.5 | 21 | 20 | 22.5 | 24 |
| Vertical radiating element spacing d/ | - | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| Horizontal radiating element spacing d/ | - | - | - | - | 0.6 | 0.6 | 0.6 | 0.5 | 0.5 | 0.5 |
| Vertical 3dB beamwidth of single element / deg | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 |
| Horizontal 3dB beamwidth of single element / deg | 65 | 65 | 65 | 65 | 80 | 80 | 80 | 80 | 80 | 80 |
| Losses of cable network / dB | 0.5 | 0.8 | 1.0 | 1.2 | 0.8 | 1.0 | 1.2 | 0.8 | 1.0 | 1.2 |

However, it was not discussed which antenna type to follow for the simulation assumptions. The antenna type A10 with 10 element single column antenna model could be used as a starting point for the simulation work. It is expected to produce results similar to a real passive antenna (with nulls and side-lobes). Further, it should be noted that A10 has a narrower horizontal beamwidth and 0.9 lambda vertical spacing as compared to AAS antenna assumptions.

Passive Antenna type A10 from TR 37.840 with 10 element single column antenna model has a narrower horizontal beamwidth and 0.9 lambda vertical spacing as compared to AAS antenna assumptions with antenna parameters of 90 degrees horizontal beamwidth and 0.7 lambda vertical spacing.

The Macro Non-AAS antenna configuration listed in Table 2.1.4.3-1 can be used for simulation evaluation, as it is expected to produce realistic results similar to a real passive antenna.

Table 2.1.4.3-1: Macro Non-AAS BS antenna

|  | Non- AAS Urban Macro |  |
| --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) |
| Parameter |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 10x1 | 10x1 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1x1 | 1x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | N/A |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 38 | 38 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 |

## 2.2	Simulation Methodology

The simulation methodology is described below –

Step 1 – Generate Aggressor and Victim networks based on the deployment parameters, defined for the Network layout. The aggressor network operates at 2/3.5 GHz, while the victim network operates at 7 GHz. The propagation of the spurious emissions happens at 7 GHz.

Step 2 – UEs are randomly generated inside the two networks deployment.

Step 3 – The BF weights are adjusted to point to the LOS direction between the serving BSs and served UE, in the two networks.

Step 4 – Apply BF weights to the subarray ports.

Step 5 – Calculate the Coupling Loss between the aggressor BS antenna, dimensioned to operate at 2/3.5 GHz, and the victim UE or BS antenna, which are operating at 7 GHz.

- For the BS-to-BS link, include the RX BF gain at the victim BS receiver.

- At the aggressor BS, do not include the beamforming gain in the antenna gain, but only the subarray gain, to account for uncorrelation of unwanted signals.

## 2.3	Simulation Results

### 2.3.1	Coupling loss approach with AAS vs Non-AAS

The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive antennas with 17 dBi antenna gain.

In Release-99 approach to evaluate the MCL, it was reasonable to assume non-AAS passive fixed beam antennas because of the predictable and stable nature of the beam’s directions and unavailability of AAS antennas.

With the introduction of AAS BS in frequencies up to 7125 MHz, the impact of beams pointing towards the victim UE or BS, larger antenna gain, sub-array antenna structures and more relevant propagation models that are available today should also be considered in the coupling loss evaluation.

To derive the coupling loss, we should derive the relevant CDF of the coupling loss between the aggressor BS and the victim UE or BS considering the impact of adaptive beamforming, adjusting the beam patterns to maximize the reception and transmission of signal. The beam could be steered to a different location in each scenario based on the channel and UE locations.

Release-99 approach to evaluate the MCL for AAS antennas has large variations, due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal.

This MCL value, however, is at the queue of the CDF, which is very sensitive to the simulation and can fluctuate across different simulation runs because the beams are dynamically adjusting, which might lead to variability in observed MCL values.

Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99th percentile of the coupling loss CDF, instead of the minimum value.

### 2.3.2	BS-to-UE coupling with AAS Macro at 3.5 GHz and Isotropic UE at 7 GHz

Figure 2.3.2-1 depicts the CDF of the coupling loss between the aggressor BSs deployed at 3.5 GHz and the victim UE associated to a 7 GHz network deployment.

Proposed detailed assumptions can be found in Section 2 along with the simulation methodology. The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 3.5 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – only subarray gain is considered

  - Propagation model – TR 38.901

  - Minimum BS - UE distance (2D) – 35 m

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 7 GHz

  - Isotropic UE with 0 dBi gain and without beamforming

  - Propagation model – TR 38.901

  - UE NF is 9 dB

Figure 2.3.2-1 Coupling loss CDF from aggressor BS Macro to victim UE

It can be observed that the directionality of the BS beamforming towards the served UEs allows for spatial separation towards victim UEs in the other network, and consequently to more relaxed coupling loss values compared to previous R99 analysis.

### 2.3.3	BS-to-BS coupling with AAS Macro at 3.5 GHz and AAS Macro at 7 GHz

Figures 2.3.3.1-1 and 2.3.3.2-1 depicts the coupling loss CDF between the aggressor AAS Macro BS operating at 3.5 GHz, and the victim AAS Macro BS receiver operating at 7 GHz.

Proposed detailed assumptions can be found in Section 2 along with the simulation methodology. The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 3.5 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – only subarray gain is considered

  - Propagation model – TR 38.901

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 7 GHz

  - Antenna model – AAS with 8x16 array and 3x1 subarray antenna configuration

  - Gain at Rx – both Beamforming gain and subarray gain are considered

  - Propagation model – TR 38.901

  - Antenna height – 25m

  - BS NF is 6 dB (based on n104)

The BS Tx Beamforming gain is not included in the coupling loss to model the uncorrelation of the signals, while the BS Rx beamforming gain is included at the receiver.

Beamforming gain is not considered at BS Tx side as the unwanted emissions are modelled as uncorrelated signals, whereas the Beamforming gain is included at BS Rx side.

Results are shown for two grid shift values - 100% and 50%. Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective. Co-location related work in terms of coupling loss is handled in separate discussions in RAN4.

Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective.

#### 2.3.3.1	Network grid shift is 100%

Figure 2.3.3.1-1 Coupling loss CDF from aggressor BS Macro to victim BS Macro, when GS is 100%

As discussed in Section 2.3.1, the coupling loss value is at the queue of the CDF, which is very sensitive to the simulation and can fluctuate across different simulation runs because the beam is dynamically adjusting, which might lead to variability in observed MCL values. Due to this, we suggest looking at around 99th percentile of the CDF, instead of looking at the Minimum coupling loss.

It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile.

#### 2.3.3.2	Network grid shift is 50%

Figure 2.3.3.2-1 Coupling loss CDF from aggressor BS Macro to victim BS Macro, when GS is 50%

As discussed in Section 2.3.1, the coupling loss value is at the queue of the CDF, which is very sensitive to the simulation and can fluctuate across different simulation runs because the beam is dynamically adjusting, which might lead to variability in observed MCL values. Due to this, we suggest looking at around 99th percentile of the CDF, instead of looking at the Minimum coupling loss.

It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Assuming uncorrelated signals can be used as a baseline.

Observation 2	When signals are uncorrelated (ρ = 0), the antenna array does not exhibit array factor beamforming gain.

Observation 3	Passive Antenna type A10 from TR 37.840 with 10 element single column antenna model has a narrower horizontal beamwidth and 0.9 lambda vertical spacing as compared to AAS antenna assumptions with antenna parameters of 90 degrees horizontal beamwidth and 0.7 lambda vertical spacing.

Observation 4	In Release-99 approach to evaluate the MCL, it was reasonable to assume non-AAS passive fixed beam antennas because of the predictable and stable nature of the beam’s directions and unavailability of AAS antennas.

Observation 5	Release-99 approach to evaluate the MCL for AAS antennas has large variations, due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal.

Observation 6	Beamforming gain is not considered at BS Tx side as the unwanted emissions are modelled as uncorrelated signals, whereas the Beamforming gain is included at BS Rx side.

Observation 7	Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective.

Observation 8	It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile.

Observation 9	It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 should model unwanted emissions as uncorrelated signals to simplify the system level evaluation.

Proposal 2	The Macro Non-AAS antenna configuration listed in Table 2.1.4.3-1 can be used for simulation evaluation, as it is expected to produce realistic results similar to a real passive antenna.

Proposal 3	Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99th percentile of the coupling loss CDF, instead of the minimum value.

# References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[3]	  R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

[4]	RP-243288, “Moderator's summary of discussion about R19-RAN4-BSRF”, Huawei

[5]	3GPP TR 37.840

[6]	3GPP TR 38.922