3GPP TSG-RAN WG4 Meeting #115	R4-2507699

Malta, MT, May 19 – 23, 2025

# Title:	Finalizing the 1UL HPUE MSD Simplification

# Agenda Item:	7.1.2.1.1

# Source:	Skyworks Solutions, Inc., Nokia, Samsung

# Document for:	Approval

- Introduction

This contribution wraps up the previous meeting way forward (WF) agreement [1] and general aspect to prepare the introduction of these new requirements into TS 38.101-1. A companion draft CR is proposed [3].

- Discussion

  - Background

At RAN4 meeting #114-bis, the following agreements 1 and 2 were reached for the HPUE inter-band CA simplification of 1 UL band 1UL CC MSD requirements [1].

  - Proposal for companion draft CR

    - Look-up table granularity

In the WF [1] agreement 1, some companies expressed some concerns about the input granularity proposed in the example look-up table (LUT), i.e., about the number of entries and about the step size for PC31Tx MSD requirements less than 1dB. We capture in Observation 1 one possible LUT simplification.

Observation 1: The 1UL band 1UL CC HPUE MSD simplification MSDmax LUTs can be simplified as shown in Table 7.3A.2.x-x.

Table 7.3A.2.x-x: MSDmax look-up-table for MSD due to harmonic, harmonic mixing, cross-band isolation.

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ - ≤0.9 | 0.7 | 1.1 | 2.5 |
| 1.0≤ - ≤1.9 | 1.2 | 2.1 | 4.1 |
| 2.0≤ - ≤2.9 | 1.6 | 2.7 | 5.1 |
| 3.0≤ - ≤3.9 | 1.8 | 3.3 | 5.9 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 |
| ≥10 | 3 | 6 | 9 |

    - Eligibility criteria

We reproduce in Observation 2 the criteria from proposal 1 of [2].

Observation 2: The following eligibility criteria are sufficient for 1UL band 1UL CC HPUE MSD simplification.

The 1UL HPUE MSD LUT simplification applies to Rel-19 CA band combinations, for which:

- A valid 1UL HPUE configurations is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and,

- A default power class 1UL MSD test point is specified, and,

- A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4a-2, Table 7.3A.4-4b, Table 7.3A.6-1a-1, Table 7.3A.6-1a-2, or Table 7.3A.6-1b.

    - Draft CR proposal

The following proposal wraps up observation 1 and 2 for consideration of companion draftCR [3].

Proposal: Consider adopting the text highlighted in blue to the core requirements of sub-clause 7.3A.2.3 [3].

-------------------------------------------------------------- start of change -----------------------------------------------------------

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

In case the applicable exceptions to reference sensitivity (MSD) for power class 2 or power class 1.5 are not specified in clause 7.3A.4, 7.3A.5 or 7.3A.6 then,

PCx MSD = PC31Tx MSD + MSD

where,

- x = 21Tx, 22Tx, or 1.52Tx,

- PC31TX MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1,

and for MSD due to harmonic, harmonic mixing, and cross-band isolation,

- MSD is specified in Table 7.3A.2.3-1 with three look-up-tables denoted “MSDmax” of 3, 6, and 9dB,

- the correspondence between the MSDmax look-up tables and the source of 1UL band 1UL CC interference and the 1UL band 1UL CC power-class is specified in Table 7.3A.2.3-2,

when the following criteria are met:

- a PC3 reference sensivity exception requirement is specified either in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1, and,

- PC2 achieved with one Tx antenna connector “PC21TX”, or PC2 achieved with two Tx antenna connector “PC22Tx”, or PC1.5 achieved with two Tx antenna connectors “PC1.52Tx” is specified as a valid UL configuration in Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1n, and,

- the UL band is neither band n46, band n96 or band n102.

Table 7.3A.2.3-1: MSDmax look-up-table for MSD due to harmonic, harmonic mixing, cross-band isolation

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ - ≤0.9 | 0.7 | 1.1 | 2.5 |
| 1.0≤ - ≤1.9 | 1.2 | 2.1 | 4.1 |
| 2.0≤ - ≤2.9 | 1.6 | 2.7 | 5.1 |
| 3.0≤ - ≤3.9 | 1.8 | 3.3 | 5.9 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 |
| ≥10 | 3 | 6 | 9 |

Table 7.3A.2.3-2: 1UL band 1UL CC MSDmax correspondence look-up-table.

| Source of interference | MSDmax (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |
| Harmonic mixing | PC21Tx | PC22Tx | PC1.52Tx |
| Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

-------------------------------------------------------------- end of change -----------------------------------------------------------

- Conclusion

This paper presents our views to help introduce the new 1UL band 1UL CC HPUE inter-band CA MSD simplification scheme for Rel-19.

Proposal: Consider adopting the text highlighted in blue to the core requirements of sub-clause 7.3A.2.3 [3].

-------------------------------------------------------------- start of change -----------------------------------------------------------

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

In case the applicable exceptions to reference sensitivity (MSD) for power class 2 or power class 1.5 are not specified in clause 7.3A.4, 7.3A.5 or 7.3A.6 then,

PCx MSD = PC31Tx MSD + MSD

where,

- x = 21Tx, 22Tx, or 1.52Tx,

- PC31TX MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1,

and for MSD due to harmonic, harmonic mixing, and cross-band isolation,

- MSD is specified in Table 7.3A.2.3-1 with three look-up-tables denoted “MSDmax” of 3, 6, and 9dB,

- the correspondence between the MSDmax look-up tables and the source of 1UL band 1UL CC interference and the 1UL band 1UL CC power-class is specified in Table 7.3A.2.3-2,

when the following criteria are met:

- a PC3 reference sensivity exception requirement is specified either in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1, and,

- PC2 achieved with one Tx antenna connector “PC21TX”, or PC2 achieved with two Tx antenna connector “PC22Tx”, or PC1.5 achieved with two Tx antenna connectors “PC1.52Tx” is specified as a valid UL configuration in Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1n, and,

- the UL band is neither band n46, band n96 or band n102.

Table 7.3A.2.3-1: MSDmax look-up-table for MSD due to harmonic, harmonic mixing, cross-band isolation

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ - ≤0.9 | 0.7 | 1.1 | 2.5 |
| 1.0≤ - ≤1.9 | 1.2 | 2.1 | 4.1 |
| 2.0≤ - ≤2.9 | 1.6 | 2.7 | 5.1 |
| 3.0≤ - ≤3.9 | 1.8 | 3.3 | 5.9 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 |
| ≥10 | 3 | 6 | 9 |

Table 7.3A.2.3-2: 1UL band 1UL CC MSDmax correspondence look-up-table.

| Source of interference | MSDmax (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |
| Harmonic mixing | PC21Tx | PC22Tx | PC1.52Tx |
| Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

-------------------------------------------------------------- end of change -----------------------------------------------------------

- References

- R4-2505143 WF on HPUE MSD simplification, 3GPP TSG-RAN WG4 Meeting #114-bis, Wuhan, China, Skyworks Solutions, Inc., Nokia, Murata, Samsung, Qualcomm, Huawei, MediaTek, Vivo, ZTE, Apple.

- R4-2419699 1UL HPUE NR-CA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2507645 [NR_ENDC_RF_Ph4-Core] DraftCR to TS38.101-1-j10: Introduction of the 1UL band 1CC HPUE MSD requirements simplification, 3GPP TSG-RAN WG4 Meeting #115, Malta, MT, Skyworks solutions, Inc., Nokia.