3GPP TSG-RAN WG4 Meeting #115	R4-2507709

Malta, MT, May 19 – 23, 2025

# Title:	PRD updates on rules for 1UL MSD configurations

# Agenda Item:	6.3.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Discussion

- Introduction

This contribution proposes to introduce exception to the RAN4 PRD rules on configuration parameters for one uplink (1UL) MSD requirements, i.e. MSD due to harmonic, harmonic mixing and cross-band isolation.

- Discussion

  - Background

RAN4 rules and MSD table templates have significantly changed since Rel-15. The motivation by driving companies was mainly to eliminate inconsistent MSD requirements, and to reduce the number of MSD test points to the strict minimum required. This complex and time-consuming task showed that RAN4 needed to define new rules on how to specify the MSD test points. However, very often, and once agreed and captured in respective TRs (such as TR 38.846), these new rules also revealed that several corrections were needed, including cross-alignment of MSD test points between NR-CA and their EN-DC counterparts, as well as NR-CA with their SUL counterparts. These corrections required heavy involvement to correct, clean-up and align the MSD requirements at the end of each Release. For example, MSD requirements due to harmonic, and harmonic mixing were corrected at the end of Rel-18 by three companies [1,2]. Changing these rules have not always been easy to follow by RAN4 contributing companies but were however necessary.

Despite this work, some inconsistencies and missing rules have recently been identified [3,4]. We do not feel like it is appropriate to once again revisit all MSD test points at the end of Rel-19. Rather than proposing new updated rules that would trigger additional corrections, we propose to introduce some exceptional relaxations to the previously agreed PRD rules. Allowing a few exceptions is not ideal. But the reality is that there remains a few, exceptional, inconsistent MSD test points in Rel-19. Some of these exceptions could be accounted due to “historical reasons”, other are due to missing rules. In any case, the number of such test points is very small. We believe these few exceptions will not degrade the quality of the technical requirements. To the contrary some of these rules account for pragmatic considerations, such as downlink (DL) CBW that are not currently supported by any network or chipsets etc…

Note that there is still ongoing work to keep on correcting MSD test points which do not comply to the “mainstream” PRD rules [3].

Observation: Despite huge effort to clean-up MSD test points at the end of each Release, there remains a few inconsistent MSD test points which can be either accounted due to “historical” reasons, or due to lack of clear rules.

This contribution proposes to integrate the following exceptions to the RAN4 PRD rules.

Proposal: Consider introducing the following updates to the RAN4 PRD rules:

Based on proposal 1 [3]:

- Do not consider the following CBW as valid “smallest” CBW for MSD requirements

  - 3MHz except when band n106 is the DL affected band

  - 5MHz when band n41 or band n90 is the DL affected band

  - 7MHz for all bands

- Case of smallest CBW restricted to Scell operation:

  - exceptionally, do not mandate to specify MSD requirements with these CBW, i.e. proponents to decide on a case-by-case basis if these CBW are appropriate for MSD requirements.

Based on proposal 7 [4]:

- Relax the PRD rules regarding the so-called “second optional” MSD test point for cross-Band isolation to allow configuring the UL CBW with other CBW than the maximum supported by the highest BCS.

- Conclusion

This paper presents our views on updating the RAN4 PRD rules on valid configurations to specify the 1UL MSD requirements. This paper is for discussion only and if agreeable would be formally re-submitted at RAN4 meeting #116.

Observation: Despite huge effort to clean-up MSD test points at the end of each Release, there remains a few inconsistent MSD test points which can be either accounted due to “historical” reasons, or due to lack of clear rules.

Proposal: Consider introducing the following updates to the RAN4 PRD rules:

Based on proposal 1 [3]:

- Do not consider the following CBW as valid “smallest” CBW for MSD requirements

  - 3MHz except when band n106 is the DL affected band

  - 5MHz when band n41 or band n90 is the DL affected band

  - 7MHz for all bands

- Case of smallest CBW restricted to Scell operation:

  - exceptionally, do not mandate to specify MSD requirements with these CBW, i.e. proponents to decide on a case-by-case basis if these CBW are appropriate for MSD requirements.

Based on proposal 7 [4]:

- Relax the PRD rules regarding the so-called “second optional” MSD test point for cross-Band isolation to allow configuring the UL CBW with other CBW than the maximum supported by the highest BCS.

- References

- R4-2414437 CR for EN-DC Harmonic Mixing clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, The Netherlands, Qualcomm France, Skyworks Solutions, Inc, Nokia.

- R4-2414384 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, The Netherlands, Skyworks Solutions, Inc, Qualcomm France, Nokia.

- R4-2507700 Impact of Rel-19 BCS4_5 new greatest-smallest CBW on MSD, 3GPP TSG-RAN WG4 Meeting #115, Malta, MT, Skyworks Solutions, Inc.

- R4-2503240 Cross-band isolation exception cases and corrections, 3GPP TSG-RAN WG4 Meeting #114-bis, Wuhan, China, Skyworks Solutions, Inc.