3GPP TSG-RAN WG4 Meeting #115	R4-2507712

# Malta, Malta, 19th May – 23rd May 2025

Agenda item:		6.15.1

Source: 		Aalyria

Title:	 	Possible approach on L-band NS value for RA protection

Document for:		Discussion

## 1	Introduction

There has been a long-standing ongoing discussion on how to address the Radio Astronomy (RA) protection requirements for the L-band frequency bands, with frequencies ranging 1518-1559 MHz DL and 1626.5-1660.5 MHz UL and 1668-1675 MHz UL, in the 3GPP specifications. Such requirements are stipulated in a number of regulatory sources, such as ETSI (e.g. ETSI EN 301 681), FCC and ITU.

One specific requirement refers to the unwanted emissions in the Carrier Off-state.  This paper attempts at proposing a possible approach for the definition of useful Network Signalling (NS) that an NTN operator could leverage to signal the applicability of this requirement in a given portion of the network, when in proximity of RA stations.

## 2	Discussion

As presented earlier in [3], ETSI EN 301 681 harmonized standard specifies requirements for the protection of radioastronomy bands. In section 4.2.6 of the document, the protection of radioastronomy services is mandated by disabling devices in the vicinity of radioastronomy stations recorded in the ITU master international frequency register.

To protect the radio astronomy service in the 1 660 MHz to 1 660,5 MHz band and the 1 668,0 MHz to 1 670,0 MHz band from emissions produced by MESs the transmissions in these frequency bands shall be capable of being disabled in the vicinity of RA stations recorded in the ITU Master International Frequency Register.

Requirements are also set out for the activation of this disable function:

“An MES which is in the carrier-on state shall not continue transmissions for a period of time longer than 1s after receipt of a transmission disable command.”

Spectrum emission requirements are further specified when a transmission disable command is issued, in the form of carrier-off state emissions:

Table 5.2.5.4.1.2-1 EN 301 681 Carrier-off state requirements

| Frequency (MHz) | EIRP (dBW) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 30 to 1 000 | -87 | 100 kHz | peak hold |
| 1 000 to 1 525 | -87 | 100 kHz | peak hold |
| 1 525 to 1 559 | -97 (see note 3) | 100 kHz | average |
| 1 559 to 1 610 | -70 | 1 MHz (see note 1) | average (see note 2) |
| 1 610 to 12 750 | -87 | 100 kHz | peak hold |
| NOTE 1: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 2: The average measurement method defined in clause 5.2.2.3 shall apply except that a measurement time of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 3: If MES antenna gain is higher than 8 dBi, the unwanted emission limit in the band 1 525 MHz to 1 559 MHz shall be considered to be -90 dBW in 100 kHz. |  |  |  |

The emissions limit for 1610-12750MHz captures the carrier-off requirements for protection of radioastronomy bands within 1660-1660.5MHz and 1668-1670MHz, mandating a limit of -87dBW/100kHz, measured in any time period as a peak hold measurement.

Observation 1: Based on ETSI EN 301 681, when disabling transmission in proximity of Radioastronmy (RA) stations, MES are required to comply with a Carrier-off state emission requirement of up to -87dBW/100kHz in the L-band and adjacent RA bands.

The current 3GPP TS 38.101-5 specifies the “Transmit OFF power” as follows

### 6.3.2	Transmit OFF power

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1 ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.2-1.

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth | (MHz) | 5, 10, 15, 20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| Transmit OFF power | (dBm) | -50 |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

For a 5 MHz CHBW, leading to a Measurement Bandwidth (MBW) of 4515 kHz, this yields a value of -80 dBW/4515 kHz, which translates to –63.5 dBW/100 kHz, which is much less stringent than the requirement specified in EN 301 681.

Observation 2: The required Carrier-off state emission requirement of -87dBW/100kHz in EN 301 681 is much more stringent than the current equivalent Transmit OFF power requirement in 3GPP TS 38.101-5, which equates to –63.5 dBW/100 kHz.

Given that a UE is not expected to be aware about the locations of RA stations, but is assumed to follow the network guidance as per Operator’s planning, there may be a need to specify an additional NS that the network could signal based on Operator’s discretion, to enforce the additional carrier-off state emission requirement when UE are not scheduled by the network in proximity to an RA station.

Proposal 1: Consider an optional NS value to indicate the additional more stringent Transmit OFF power requirement corresponding to the ETSI “Carrier-off state” emission requirements applicable in proximity to an RA stations, to allow the UE to take necessary measures.

Proposal 2: Specify the additional emission requirements associated with the new NS value as per the following table from ETSI EN 301 681.

Table 5.2.5.4.1.2-1 EN 301 681 Carrier-off state requirements

| Frequency (MHz) | EIRP (dBW) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 30 to 1 000 | -87 | 100 kHz | peak hold |
| 1 000 to 1 525 | -87 | 100 kHz | peak hold |
| 1 525 to 1 559 | -97 (see note 3) | 100 kHz | average |
| 1 559 to 1 610 | -70 | 1 MHz (see note 1) | average (see note 2) |
| 1 610 to 12 750 | -87 | 100 kHz | peak hold |
| NOTE 1: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 2: The average measurement method defined in clause 5.2.2.3 shall apply except that a measurement time of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 3: If MES antenna gain is higher than 8 dBi, the unwanted emission limit in the band 1 525 MHz to 1 559 MHz shall be considered to be -90 dBW in 100 kHz. |  |  |  |

Proposal 3: Capture the necessary requirements in the TR 38.863

## 3	Conclusion

Observation 1: Based on ETSI EN 301 681, when disabling transmission in proximity of Radioastronmy (RA) stations, MES are required to comply with a Carrier-off state emission requirement of up to -87dBW/100kHz in the L-band and adjacent RA bands.

Observation 2: The required Carrier-off state emission requirement of -87dBW/100kHz in EN 301 681 is much more stringent than the current equivalent Transmit OFF power requirement in 3GPP TS 38.101-5, which equates to –63.5 dBW/100 kHz.

Proposal 1: Consider an optional NS value to indicate the additional more stringent Transmit OFF power requirement corresponding to the ETSI “Carrier-off state” emission requirements applicable in proximity to an RA stations, to allow the UE to take necessary measures.

Proposal 2: Specify the additional emission requirements associated with the new NS value as per the following table from ETSI EN 301 681.

Table 5.2.5.4.1.2-1 EN 301 681 Carrier-off state requirements

| Frequency (MHz) | EIRP (dBW) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 30 to 1 000 | -87 | 100 kHz | peak hold |
| 1 000 to 1 525 | -87 | 100 kHz | peak hold |
| 1 525 to 1 559 | -97 (see note 3) | 100 kHz | average |
| 1 559 to 1 610 | -70 | 1 MHz (see note 1) | average (see note 2) |
| 1 610 to 12 750 | -87 | 100 kHz | peak hold |
| NOTE 1: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 2: The average measurement method defined in clause 5.2.2.3 shall apply except that a measurement time of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 3: If MES antenna gain is higher than 8 dBi, the unwanted emission limit in the band 1 525 MHz to 1 559 MHz shall be considered to be -90 dBW in 100 kHz. |  |  |  |

Proposal 3: Capture the necessary requirements in the TR 38.863

## 4	References

- RP-241856, Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges, Inmarsat, Viasat, Thuraya

- R4-2420465, WF on requirements for NT IoT NTN bands, Moderator(Inmarsat)

- R4-2419555, Input to UE RF requirements for NTN L-bands n253, n521, n250, Inmarsat, Viasat