3GPP TSG-RAN WG4 Meeting #115	R4-2507717

Malta, MT, 19th – 23th May, 2025

Source:	Nokia

Title:	Discussion on NR NTN SAN requirement for Ku bands

Agenda item:	7.11.6

Document for:	Approval

Introduction

For WI on the introduction of NTN Ku Band [1], RAN4#114-bis has agreed the following Way-forward [4] regarding the SAN requirement for FR1-NTN and FR2-NTN Ku bands.

Issue 2-1-1: General issues for Ku band SAN RF requirements Agreement: - FFS on whether to introduce a new SAN type for FR1-NTN Ku bands which is based on SAN type 2-O RF requirements. Sub-topic 2-2: Specific topics for Ku band SAN RF requirements Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power Agreement: - For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands - [+-3.4] dB EIRP accuracy, [+-3] dB TRP accuracy (re-using FR2-NTN) as baseline - Other options are not precluded - Other NTN bands will not be impacted Issue 2-2-2: OTA RE power control dynamic range for the Ku band SAN Agreement - The OTA RE power control dynamic range is not required for FR2-NTN Ku-band SAN. - Further discuss whether OTA RE power control dynamic range (i.e. RE power boosting) is needed for FR1-NTN Ku-band SAN for the smaller channel bandwidth. Issue 2-2-3: OTA total power dynamic range for the Ku band SAN Agreements: - Agree on adding the following OTA SAN total power dynamic range requirement for new channel bandwidths of FR1-NTN Ku bands. Table 2-2-3.3 SAN channel; Total power dynamic range (dB). bandwidth (MHz); 15 kHz SCS; 30 kHz SCS. 25; 21.2; 18.1. 35; 22.7; 19.6. 50; 24.3; 21.2. 70; N/A; 22.7. 100; N/A; 24.3. Issue 2-2-5: OTA sensitivity for the Ku band SAN - Proposals: - Proposal 1 (ZTE): It is proposed not to specify OTA sensitivity for NTN Ku bands. - Observation 1 (Nokia): OTA sensitivity and Rx dynamic range requirements can be omitted for Ku bands with FR1 numerologies. Agreement: - Proposal 1 (ZTE): It is proposed not to specify OTA SAN sensitivity for NTN Ku bands. Issue 2-2-7: OTA dynamic range for the Ku band SAN - Proposals: - Proposal 1 (ZTE): Not to specify OTA dynamic range for NTN Ku bands. Agreement: - Proposal 1 (ZTE): Not to specify OTA SAN dynamic range for NTN Ku bands.

In this contribution, we discuss how we harmonize the requirement between FR1-NTN and FR2-NTN Ku bands.

Discussion

Issue 2-1-1: General issues for Ku band SAN RF requirements Agreement: - FFS on whether to introduce a new SAN type for FR1-NTN Ku bands which is based on SAN type 2-O RF requirements.

RF requirement for SAN is specified in TS 38.108, where three requirement types 1-H, 1-O and 2-O are currently specified. It has been already agreed that 1-H is out of scope [2]. One possible option that was earlier assumed was to reuse the existing 1-O requirement for FR1-NTN Ku bands (i.e., n248 and n247 in [5]) and 2-O requirement for FR2-NTN Ku bands (i.e., n509 and n508 in [5]).

However, there has been a request from some satellite operators to align the requirement between FR1-NTN and FR2-NTN as they use the same frequency range only with different numerologies. It is likely that they both are supported in a similar SAN hardware design thus the requirement is better harmonized among two.

The existing 1-O requirement originally comes from the requirement of TN FR1 bands. It was first derived for bands around 2 GHz (e.g., some fundamental assumptions are originated from IMT-2000) and they have been extended up to 7 GHz band by now. On the other hand, the existing 2-O requirement comes from the requirement for TN FR2 bands, which was first derived for 28 GHz band. The transmissions and reception requirements are generally more stringent for FR1 than FR2. It is likely that the existing 1-O requirement (applicable up to 7GHz today in TN) is too stringent for NTN Ku bands, as SAN RF design would be more similar to TN FR2 than FR1.

The same framework of SAN type 2-O RF requirement for FR2-NTN Ku bands (n509 and n508) is applied to FR1-NTN Ku bands (i.e., n248 and n247).

Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power Agreement: - For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands - [+-3.4] dB EIRP accuracy, [+-3] dB TRP accuracy (re-using FR2-NTN) as baseline - Other options are not precluded - Other NTN bands will not be impacted

As discussed above, FR2-NTN framework is more appropriate for Ku bands, we propose reusing FR2-NTN tolerance, i.e., ±3.4 dB EIRP accuracy and ±3 dB TRP accuracy as baseline.

The tolerance of the radiated transmit power and SAN output power are ±3.4 dB and ±3 dB, respectively, for both the FR1-NTN and FR2-NTN Ku bands.

Issue 2-2-2: OTA RE power control dynamic range for the Ku band SAN Agreement - The OTA RE power control dynamic range is not required for FR2-NTN Ku-band SAN. - Further discuss whether OTA RE power control dynamic range (i.e. RE power boosting) is needed for FR1-NTN Ku-band SAN for the smaller channel bandwidth.

As discussed above, we propose FR2-NTN framework is appropriate for Ku bands. Thus, we propose OTA RE power control dynamic range is not specified for FR1-NTN Ku bands.

OTA RE power control dynamic range is not specified for FR1-NTN Ku bands.

Conclusion

The following Observations and Proposals were made:

Proposal 1: The same framework of SAN type 2-O RF requirement for FR2-NTN Ku bands (n509 and n508) is applied to FR1-NTN Ku bands (i.e., n248 and n247).

Proposal 2: The tolerance of the radiated transmit power and SAN output power are ±3.4 dB and ±3 dB, respectively, for both the FR1-NTN and FR2-NTN Ku bands.

Proposal 3: OTA RE power control dynamic range is not specified for FR1-NTN Ku bands.

- References

- RP-250799 Revised WID: Introduction of Ku bands for NR NTN, RAN#107

- R4-2502288 WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114

- R4-2502290 Way Forward for [114][313] NR_NTN_Ku_Band_General, RAN4#114

- R4-2504720 Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114-bis

- R4-2504761 Way Forward for [114bis][313] NR_NTN_Ku_Band_General, RAN4#114-bis