# 3GPP TSG-RAN WG4 Meeting #115		        R4-2507720

# St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	7.31.3.1

Source: 	Huawei, HiSilicon

Title:	On remaining open issues in NR NTN Ph3

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in the last meeting, listing a number of open issues.

## Issue 1-1: Output power enhancement for NR NTN PC3

Agreements:

- If defined, additional output power enhancement will be optional for UE to support.

  - FFS on need of new UE capability

- Evaluate potential to further enhance output power for RAN4#115:

  - Amount of output power enhancement [0.5 to 1 dB]

    - Less than 0.5 dB improvement is not specified

  - Initially consider all RB allocations and modulation orders

    - Consider both MPR reduction and MOP increase

      - FFS how to capture MOP increase in the specification

    - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders

  - Evaluate both HD-FDD and FD-FDD

    - Consider at least insertion loss impact

    - FFS whether to apply same enhancement to both HD-FDD and FD-FDD

  - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously

    - FFS whether simultaneous support is specified

    - Consider whether PC3 or PC2 ACLR requirement should apply

  - FFS Applicability to PC3 non-RedCap NR NTN

Additionally, an LS from RAN1 is asking whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

# 2	Discussion

As described in the LS [2], RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. This is similar to the phase continuity requirements for DRMS bundling that has been defined in RAN4, for which the side conditions are duplicated below:

The above requirements are applicable when all the following conditions are met within the measurement time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	The TPMI precoder does not change.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

## -     For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions.

Among the side conditions, it’s required that “There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE”. Hence, the power consistency is also ensured.

- It is feasible to reuse the phase continuity requirements as for DMRS bundling as for OCC2 and OCC4.

As shown below, the Rel-18 power boost mechanism for TN applies to BPSK and QPSK for the inner RB allocations.

When enabled, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass], i.e. the MOP is higher than 23dBm .

- When enabled, the Rel-18 TN power boost mechanism increases the MOP by 1dB for PC3 for the inner RB allocations with BPSK or QPSK modulation.

Considering the strong demand for higher output power for NTN UEs, we think:

- It’s feasible to port the Rel-18 TN power boost mechanism to TN for PC3 for the inner RB allocations with BPSK or QPSK modulation.

For HD-FDD operations, the insertion loss of the RFFE could be reduced. Consequently, the output power of the wanted signal as well as the unwanted emissions can both increase. Such power boost is applicable for all types of modulations and waveforms.

- The output power of the wanted signal as well as the unwanted emissions can increase simultaneously as a result of insertion loss reduction.

From the perspective of MPR requirements, the power boost might not be feasible for RB allocations which need back-off due to SEM limits. However, for RB allocations whose MPR requirements are gated by ACLR, IBE or EVM, the power boost could be possible.

- Power boost resulting from insertion loss reduction might not be feasible for RB allocations which need back-off due to SEM limits, regardless whether it’s implemented by MOP increase or MPR reduction.

- New scheme for indicating and controlling power boost resulting from insertion loss reduction can be considered, and the PC3 ACLR requirement should apply if the PC3 MPR requirement is reused.

# 3	Conclusion

We have studied the remaining open issues and made the following observations and proposals:

Proposal 1:It is feasible to reuse the phase continuity requirements as for DMRS bundling as for OCC2 and OCC4.

Observation 1:When enabled, the Rel-18 TN power boost mechanism increases the MOP by 1dB for PC3 for the inner RB allocations with BPSK or QPSK modulation.

Proposal 2:It’s feasible to port the Rel-18 TN power boost mechanism to TN for PC3 for the inner RB allocations with BPSK or QPSK modulation.

Observation 2:The output power of the wanted signal as well as the unwanted emissions can increase simultaneously as a result of insertion loss reduction.

Proposal 3:Power boost resulting from insertion loss reduction might not be feasible for RB allocations which need back-off due to SEM limits, regardless whether it’s implemented by MOP increase or MPR reduction.

Proposal 4:New scheme for indicating and controlling power boost resulting from insertion loss reduction can be considered, and the PC3 ACLR requirement should apply if the PC3 MPR requirement is reused.

# References

- R4-2504708 Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm, RAN4#114bis

- R4-2505305 LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, RAN1, RAN4#115