# 3GPP TSG-RAN WG4 Meeting #115		        R4-2507721

# St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	7.10.3.3.1

Source: 	Huawei, HiSilicon

Title:	On IoT NTN HPUE requirements

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in the last meeting to progress the HPUE requirements IoT NTN. Among many open issues listed in [1], we will focus on the PC2 MPR requirements for NB-IoT based IoT-NTN in the sequel.

# 2	Discussion

There’re two candidates for calibrate the PA operation point, as shown below.

Issue 1-1-1: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC2

- WF (Ad-hoc agreement):

  - Tentative PA Calibration points

    - Option 1:

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc

      - CIM3: 60 dBc

      - [NR ACLR: 31dBc]

    - Option 2:

      - DFT-s-OFDM 1RE (Single Tone)

      - [Single Tone] [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

      - [2] or [3] dB post PA loss

      - 0dB MPR

      - Carrier leakage 28dBc

      - IQ image:28 dBc

      - CIM3: 60 dBc

  - Both options can be considered

  - Both Testing and simulation may be considered.

Option 1 assumes a wide-band NR PA, which might be feasible for hand-held UEs sharing hardware between NR and NB-IoT. However, some non-handheld UEs dedicated for IoT operations are expected to work on battery power supply for years. The PAs in those devices are optimized for narrow-band signal transmissions and low power consumption.

- PA calibration option 1 uses wideband NR signal and is suitable for handheld UEs; option 2 uses narrowband signal and is suitable for IoT devices optimized for power consumption.

We conducted separate simulations with both options. With option 2, we calibrated the PA model with 0dB MPR for all single tone transmissions. The MPR results for multi-tone transmissions are shown in Table 1.

Table 1: Simulated MPR for QPSK (power class 2)

| Tone spacing | Number of tones | Allocated tones | Gating factor | Option 1 MPR [dB] | Option 2 MPR [dB] |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | IBE | 0.3 | 0 |
|  |  | 3-5 | IBE | 0.3 | 0 |
|  |  | 6-8 | IBE | 0.3 | 0 |
|  |  | 9-11 | IBE | 0.3 | 0 |
|  | 6 | 0-5 | IBE | 0.4 | 0 |
|  |  | 6-11 | IBE | 0.4 | 0 |
|  | 12 | 0-11 | OBW | 2.3 | 0.6 |

- Take the MPR results in Table 1 into account when defining the PC2 MPR requirements for NB-IoT based NTN.

# 3	Conclusion

Regarding the PC2 MPR requirements for NB-IoT based NTN, the following proposals are made:

Observation 1: PA calibration option 1 uses wideband NR signal and is suitable for handheld UEs; option 2 uses narrowband signal and is suitable for IoT devices optimized for power consumption.

Proposal 1:Take the MPR results in Table 1 into account when defining the PC2 MPR requirements for NB-IoT based NTN.

# References

- R4-2505114 WF on HPUE RF requirements for IoT-NTN, VIVO, RAN4#114bis