3GPP TSG-RAN WG4 Meeting #115	R4-2507724

St Julian's, Malta, May 19th – 23rd 2025

Agenda Item:	7.25.2

Source:	Ericsson

Title:	SBFD general aspects

Document for:	Approval

# 1	Introduction

In this contribution, we present our views and proposals on general aspects for SBFD capable BS regarding band support for SBFD capable BS, SBFD sub-band configuration, and specification structure. Comprehensive analysis of each SBFD BS RF TX/RX requirement and new requirements for SBFD capable BS are detailed in separate submissions [1-3].

# 2	Discussion

2.1	SBFD subband configuration

### Sub-topic 1-1: Declaration and flexibility for SBFD operation ##### Issue 1-1-1: Declaration and flexibility for SBFD operation Agreement: - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. ### Sub-topic 1-2: DL/UL subband configurations support for SBFD ##### Issue 1-2-1: Restriction on the number of UL subband sizes supported Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. ##### Issue 1-2-2: Restriction on the number of DL subband sizes supported Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

The left-over issue for the two sub-topics/three issues above are the same and focused on FR1 MR SBFD-capable BSs, and it is agreed that a power level threshold should be established: Above this threshold, analogue filters are expected to be used, and the restrictions above are applicable as FR1 WA BS; Below this threshold, analogue filters are not expected to be used, and the restrictions above are not applicable.

Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs, and FFS the value of X.

Considering the BS rated carrier TRP output power for BS type 1-O defined in table 9.3.1-1 in TS 38.104 [4], we propose using 33 dBm either as TRP or total conducted power as the power level threshold.

In the WF, it is recommended to study the value for different number of TX chains. In our view, the reason that 33 dBm is introduced for FR1 MR is that the RX array starts requiring analogue filters to manage the transmit power level from the TX array. So, what matters here is the total amount of power that the RX needs to handle, which is related to the total number of transmitters and carriers in the system. Therefore, no need to define a value per TX chain.

Regarding the introduction of power level threshold for FR1 MR, what matters is the total amount of power that the RX needs to handle, which is related to the total number of transmitters and carriers in the system.

Do not define a value per TX chain as the power level threshold for FR1 MR.

Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

2.2	Specification structure and requirement set applicability for SBFD

In the WF [5] approved during RAN4#114bis, it is agreed to adopt the following clause structure for Chapter 12 for requirements for SBFD-capable BS and use the following RAN4 requirement set applicability for SBFD-capable BS and capture it in clause 12.1.

12	Requirements for SBFD-capable BS	Error! Bookmark not defined.

12.1	General	Error! Bookmark not defined.

12.2	Conducted transmitter characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.2.1	General	Error! Bookmark not defined.

12.2.2	Base station output power for SBFD	Error! Bookmark not defined.

12.2.3	Output power dynamics for SBFD	Error! Bookmark not defined.

12.2.4	Transmit ON/OFF power for SBFD	Error! Bookmark not defined.

12.2.5	Transmitted signal quality for SBFD	Error! Bookmark not defined.

12.2.6	Unwanted emissions for SBFD	Error! Bookmark not defined.

12.2.7	Transmitter intermodulation for SBFD	Error! Bookmark not defined.

12.2.8	Transient period between SBFD and non-SBFD	Error! Bookmark not defined.

12.2.9	In-channel adjacent subband leakage for SBFD	Error! Bookmark not defined.

12.3	Conducted receiver characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.3.1	General	Error! Bookmark not defined.

12.3.2	Reference sensitivity level for SBFD	Error! Bookmark not defined.

12.3.3	Dynamic range for SBFD	Error! Bookmark not defined.

12.3.4	In-band selectivity and blocking for SBFD	Error! Bookmark not defined.

12.3.5	Out-of-band blocking for SBFD	Error! Bookmark not defined.

12.3.6	Receiver spurious emissions for SBFD	Error! Bookmark not defined.

12.3.7	Receiver intermodulation for SBFD	Error! Bookmark not defined.

12.3.8	In-channel selectivity for SBFD	Error! Bookmark not defined.

12.3.9	In-channel adjacent subband blocking for SBFD	Error! Bookmark not defined.

12.4          Conducted performance requirements for SBFD-capable BS

12.5	Radiated transmitter characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.5.1	General	Error! Bookmark not defined.

12.5.2	Radiated transmit power for SBFD	Error! Bookmark not defined.

12.5.3	OTA base station output power for SBFD	Error! Bookmark not defined.

12.5.4	OTA output power dynamics for SBFD	Error! Bookmark not defined.

12.5.5	OTA transmit ON/OFF power for SBFD	Error! Bookmark not defined.

12.5.5.1	General	Error! Bookmark not defined.

12.5.5.2	OTA transmitter OFF power for SBFD	Error! Bookmark not defined.

12.5.5.3	OTA transient period for SBFD	Error! Bookmark not defined.

12.5.6	OTA transmitted signal quality for SBFD	Error! Bookmark not defined.

12.5.7	OTA unwanted emissions for SBFD	Error! Bookmark not defined.

12.5.8	OTA transmitter intermodulation for SBFD	Error! Bookmark not defined.

12.5.9	OTA transient period between SBFD and non-SBFD	Error! Bookmark not defined.

12.5.10	OTA in-channel adjacent subband leakage for SBFD	Error! Bookmark not defined.

12.6	Radiated receiver characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.6.1	General	Error! Bookmark not defined.

12.6.2	OTA sensitivity for SBFD	Error! Bookmark not defined.

12.6.3	OTA reference sensitivity level for SBFD	Error! Bookmark not defined.

12.6.4	OTA dynamic range for SBFD	Error! Bookmark not defined.

12.6.5	OTA in-band selectivity and blocking for SBFD	Error! Bookmark not defined.

12.6.6	OTA out-of-band blocking for SBFD	Error! Bookmark not defined.

12.6.7	OTA receiver spurious emissions for SBFD	Error! Bookmark not defined.

12.6.8	OTA receiver intermodulation for SBFD	Error! Bookmark not defined.

12.6.9	OTA in-channel selectivity for SBFD	Error! Bookmark not defined.

12.6.10	OTA in-channel adjacent subband blocking for SBFD	Error! Bookmark not defined.

12.7	   Radiated performance requirements for SBFD-capable BS

- Table 12.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6.2 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6.5 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | 12.3.7 |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.8 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.9 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7.2 | 12.5.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7.3 | 12.5.7.3 |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | 12.5.7.4 | 12.5.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7.5 | 12.5.7.5 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.9 | 12.5.9 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | 12.6.8 | 12.6.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.9 | 12.6.9 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.10 | 12.6.10 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

The draft CR will be based on the latest available Rel-19 version. A new clause needs to be added for EEIRP in clause 12 and clause numbers after that need to be corrected accordingly, which are highlighted below (including the correction of a mistake in the latest release on OTA operating band unwanted emission).

9.9	OTA spatial emission has already been introduced in the latest release (V19.0.0). The clause numbers need to be moved backward accordingly.

Use the following updated clause structure and Requirement set applicability for SBFD-capable BS.

12	Requirements for SBFD-capable BS	Error! Bookmark not defined.

12.1	General	Error! Bookmark not defined.

12.2	Conducted transmitter characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.2.1	General	Error! Bookmark not defined.

12.2.2	Base station output power for SBFD	Error! Bookmark not defined.

12.2.3	Output power dynamics for SBFD	Error! Bookmark not defined.

12.2.4	Transmit ON/OFF power for SBFD	Error! Bookmark not defined.

12.2.5	Transmitted signal quality for SBFD	Error! Bookmark not defined.

12.2.6	Unwanted emissions for SBFD	Error! Bookmark not defined.

12.2.7	Transmitter intermodulation for SBFD	Error! Bookmark not defined.

12.2.8	Transient period between SBFD and non-SBFD	Error! Bookmark not defined.

12.2.9	In-channel adjacent subband leakage for SBFD	Error! Bookmark not defined.

12.3	Conducted receiver characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.3.1	General	Error! Bookmark not defined.

12.3.2	Reference sensitivity level for SBFD	Error! Bookmark not defined.

12.3.3	Dynamic range for SBFD	Error! Bookmark not defined.

12.3.4	In-band selectivity and blocking for SBFD	Error! Bookmark not defined.

12.3.5	Out-of-band blocking for SBFD	Error! Bookmark not defined.

12.3.6	Receiver spurious emissions for SBFD	Error! Bookmark not defined.

12.3.7	Receiver intermodulation for SBFD	Error! Bookmark not defined.

12.3.8	In-channel selectivity for SBFD	Error! Bookmark not defined.

12.3.9	In-channel adjacent subband blocking for SBFD	Error! Bookmark not defined.

12.4          Conducted performance requirements for SBFD-capable BS

12.5	Radiated transmitter characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.5.1	General	Error! Bookmark not defined.

12.5.2	Radiated transmit power for SBFD	Error! Bookmark not defined.

12.5.3	OTA base station output power for SBFD	Error! Bookmark not defined.

12.5.4	OTA output power dynamics for SBFD	Error! Bookmark not defined.

12.5.5	OTA transmit ON/OFF power for SBFD	Error! Bookmark not defined.

12.5.5.1	General	Error! Bookmark not defined.

12.5.5.2	OTA transmitter OFF power for SBFD	Error! Bookmark not defined.

12.5.5.3	OTA transient period for SBFD	Error! Bookmark not defined.

12.5.6	OTA transmitted signal quality for SBFD	Error! Bookmark not defined.

12.5.7	OTA unwanted emissions for SBFD	Error! Bookmark not defined.

12.5.8	OTA transmitter intermodulation for SBFD	Error! Bookmark not defined.

12.5.9	OTA spatial emission for SBFD	Error! Bookmark not defined.

12.5.10	OTA transient period between SBFD and non-SBFD	Error! Bookmark not defined.

12.5.11	OTA in-channel adjacent subband leakage for SBFD	Error! Bookmark not defined.

12.6	Radiated receiver characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.6.1	General	Error! Bookmark not defined.

12.6.2	OTA sensitivity for SBFD	Error! Bookmark not defined.

12.6.3	OTA reference sensitivity level for SBFD	Error! Bookmark not defined.

12.6.4	OTA dynamic range for SBFD	Error! Bookmark not defined.

12.6.5	OTA in-band selectivity and blocking for SBFD	Error! Bookmark not defined.

12.6.6	OTA out-of-band blocking for SBFD	Error! Bookmark not defined.

12.6.7	OTA receiver spurious emissions for SBFD	Error! Bookmark not defined.

12.6.8	OTA receiver intermodulation for SBFD	Error! Bookmark not defined.

12.6.9	OTA in-channel selectivity for SBFD	Error! Bookmark not defined.

12.6.10	OTA in-channel adjacent subband blocking for SBFD	Error! Bookmark not defined.

12.7	   Radiated performance requirements for SBFD-capable BS

- Table 12.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6.2 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6.5 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | 12.3.7 |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.8 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.9 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7.2 | 12.5.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7.3 | 12.5.7.3 |
| OTA operating band unwanted emission |  | 9.7.4 | 9.7.4 |  | 12.5.7.4 | 12.5.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7.5 | 12.5.7.5 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA spatial emission |  | 9.9 | 9.9 |  | 12.5.9 | 12.5.9 |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.11 | 12.5.11 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | 12.6.8 | 12.6.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.9 | 12.6.9 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.10 | 12.6.10 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

2.3	Conformance testing for SBFD-capable BS

Test model

In RAN4#113, it was agreed to reuse the existing FRCs for conformance testing of the SBFD UL subband. While reusing the existing FRCs for UL subbands is a logical starting point, DL test models must be designed for scalability. Current DL test models (e.g., as shown in Table 4.9.2.2-1 TS 38.104 [4]) do not consider flexible UL subband configurations, which are essential for SBFD operation. Therefore, a scalable test model for the DL subband, which is parameterized by the UL subband bandwidth, UL subband center frequency or its relative offset from the DL, etc., is needed to accurately evaluate SBFD performance under scalable conditions.

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

The test model for SBFD DL subbband needs to be scalable.

New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

In addition, an important technical concern is the impact of scalable test model on signal statistics. As DL subbands are scaled (e.g., by varying bandwidth or power), their signal statistics may change, potentially affecting the PA testing. To ensure accurate and comprehensive testing, quantitative assessment of signal statistics variations under different subband configurations/DL subband sizes and power levels.

Signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed.

Declarations

With the introduction of SBFD feature in 3GPP, the specific SBFD-related declarations need to be incorporated in TS 38.141-1/-2 for perform conformance testing of SBFD-capable BS. Below is a summary of the declarations that have been agreed upon in past discussions during the Rel-19 meetings.

- Declaration of TX power for SBFD.

- Declaration of bands supported for SBFD.

- Declaration of channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104.

- Declaration of SBFD subband configuration(s) per supported channel bandwidth, including

  - SBFD pattern (DUD or DU/UD),

  - size of DL subband(s),

  - size of UL subband,

  - size of guardband(s) between subbands.

Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.

Observation 1	Regarding the introduction of power level threshold for FR1 MR, what matters is the total amount of power that the RX needs to handle, which is related to the total number of transmitters and carriers in the system.

Observation 2	9.9 OTA spatial emission has already been introduced in the latest release (V19.0.0). The clause numbers need to be moved backward accordingly.

Observation 3	The test model for SBFD DL subbband needs to be scalable.

Observation 4	New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

Observation 5	Signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed.

Observation 6	Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

Proposal 1	Do not define a value per TX chain as the power level threshold for FR1 MR.

Proposal 2	Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

Proposal 3	Use the following updated clause structure and Requirement set applicability for SBFD-capable BS.

# References

R4-2507726, Impact on SBFD BS RF TX requirements, Ericsson

R4-2507727, Impact on SBFD BS RF RX requirements, Ericsson

R4-2507725, New SBFD BS RF requirements, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2504751, Way forward for [114bis][306] NR_duplex_evo_General