3GPP TSG RAN WG 4 Meeting #115		R4-2507732

St Julian, Malta, May 19th – May 23rd, 2025

Title:	Proposal for LS reply to RAN1 on uplink pre-compensation for NB-IoT NTN TDD mode

Source: 	Iridium, THALES

Document for:	Discussion and Agreement

Agenda Item:	7.33.2

- Introduction

In February meetings 2025, RAN1 sent LS to RAN4 asking about validity of RAN1 working assumption on NB-IoT NTN TDD uplink precompensation [1]. This is a proposal for LS Reply to RAN1 on that topic.

- Proposed Draft LS on NB-IoT NTN TDD pre-compensation for NB-IoT NTN TDD mode

Title:	Reply LS on pre-compensation for NB-IoT NTN TDD mode

Response to:	R1-2503142 LS on precompensation for NB-IoT NTN TDD mode

Release:	Release-19

Work Item:	IoT_NTN_TDD-Core

Source:	3GPP WG RAN4

To:	3GPP WG RAN1

Cc:

Contact Person:

#### Name:		Andjela Savoia

E-mail Address:	andjela.savoia@iridium.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1    Overall description

RAN4 would like to thank RAN1 for the LS regarding uplink precompensation for NB-IoT NTN TDD mode. RAN4 would like to provide the following feedback:

- For NB-IoT NTN TDD, UE may adjust its time/frequency pre-compensation before, at the beginning of, and/or during each set of consecutive uplink sub-frames.

- Different options should not be precluded from implementation.

RAN4 discussed this topic in RAN4#115 meeting in May 2025. RAN4 confirms there is no issue with RAN1 working assumption. Furthermore, RAN4 agreed to the following:

- Do not change the current frequency error requirement from TS 36.102 for NB-IoT NTN TDD:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

- Add a note a new phrase/paragraph to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. For band 249, UE may adjust its frequency pre-compensation before, at the beginning of, and/or during each set of consecutive uplink sub-frames.”

Therefore, for NB-IoT NTN TDD, UE may adjust its time/frequency pre-compensation before, at the beginning of, and/or during each set of consecutive uplink sub-frames.

# 2.   Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to take the above information into account.

# 3.    Dates of next RAN1 Meetings:

TSG-RAN WG1 Meeting #122			25 Aug – 29 Aug 2025	                  Bangalore, India

TSG-RAN WG1 Meeting #122-bis		13 Oct – 17 Oct 2025		    Prague, CZ

- References

R1-2503142 	LS on precompensation for NB-IoT NTN TDD mode,	RAN1, 	Qualcomm Inc.

R4-2504719 	Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF,	 Thales