3GPP TSG-RAN4 Meeting #115	R4-2507734

# Malta, Malta, 19th - 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2841 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ATG-Core) Rel-18 Clarifications on some ATG requirements applicability per connector |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 6.1J indicates that the transmitter characterisitcs are specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array. Current power control and transmit signal quality requirements for ATG do not properly clarify how to consider multiple omni-directional antenna connectors or multiple transceiver array boundary connectors in case of UE with an antenna array. In RAN4#112, WF R4-2414322 was approved including the following statement: |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify power control requirements to apply to the sum of output power at each transmit antenna connector in case of UE using omnidirectional antenna(s) or to the sum of output power at each transceiver array boundary (TAB) connectors in case of UE using antenna array for ATG UE - Clarify applicability of transmit signal quality requirements per antenna connector or per transceiver array boundary connector (adding references to equivalent Tx diversity requirements in case of mulitple connectors). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core specification will remain unclear for ATG UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J.4, 6.4J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << START OF CHANGES 1>>>

### 6.3J.4	Power control for ATG

The power control requirements specified in clause 6.3.4 are applicable to the sum of output power at each transmit antenna connector for UE with omnidirectional antenna(s) or to the sum of output power at each transceiver array boundary (TAB) connectors for UE with antenna array for ATG UE.

## <<< END OF CHANGES 1>>>

## << START OF CHANGES 2>>>

## 6.4J	Transmit signal quality for ATG

### 6.4J.1	Frequency error for ATG

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency per layer at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [7].

NOTE 1:	the ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

### 6.4J.2	Transmit modulation quality for ATG

The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array, except for the phase continuity requirements for DMRS bundling in 6.4.2.5. The requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

## <<< END OF CHANGES 2>>>