3GPP TSG-RAN WG4 Meeting #115	R4-2507736

Malta, MT, 19th -23rd May, 2025

Source:	Qualcomm Incorporated

Title:	4 Tx ΔPPowerClass for SRS antenna switching

Agenda item:	5.6.1

Document for:	Approval

# 1.	Introduction

In RAN4#114 there was a discussion on how configure ΔPPowerClass for 4Tx for SRS antenna switching [1,2]. In this paper we present our views on this issue.

# 2. 	Discussion

In RAN4 there was a discussion on how to configure ΔPPowerClass for SRS antenna switching for the various 4Tx architecures agreed in rel-18. The issue was that according to the current standard for t1ry ΔPPowerClass can only be assigned a value of 3 dB as indicated below.

ΔPPowerClass =

3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r8’;

In rel-18 the following 4Tx architectures were studied: 4X23dBm, 4X26dBm, 2X23+2X26dBm. In certain 4 Tx architectures there is a possibility that SRS antenna switching may be performed with a 23 dBm PA in which case the ΔPPowerClass value would be 6 dB which is not permissible according to the current standard. Unlike for 2Tx architectures the variable ΔTRxSRS was excluded in the calculation of sounding power for 4Tx architectures as it introduces up to 3dB of ambiguity in the SRS Tx power. The exclusion of ΔTRxSRS means that SRS sounding requires 2 sets of ΔPPowerClass values if both 26dBm and 23dBm PAs are to be used for sounding at their maximum output powers.

In [1,2] it was proposed that the ΔPPowerClass value be set to 6dB for t1ry, 3dB for t2ry and 0dB otherwise. While this will not preclude any of the above mentioned 4Tx architectures from being used it would mean that the PAs having 26dBm output power would have to reduce their SRS transmission power by 3dB. We think that this power reduction would lead to degraded performance and so would like to create 2 capabilities to support both the 23dBm and 26dBm Tx architectures. These capabilities will enable PAs with different power classes to sound at their maximum output power. Our proposal is as follows:

Capability#1 for Tx architectures which use PC2 for SRS transmission, ΔPPowerClass _PC2. This capability has the following ΔPPowerClass values:

ΔPPowerClass  = 3dB for t1ry

ΔPPowerClass  = 0dB for t2ry

ΔPPowerClass  = 0dB for t4ry

Capability#2 for Tx architectures which use PC3 for SRS transmission, ΔPPowerClass _PC3. This capability has the following ΔPPowerClass values:

ΔPPowerClass  = 6dB for t1ry

ΔPPowerClass  = 3dB for t2ry

ΔPPowerClass  = 0dB for t4ry

Proposal: Introduce 2 capabilities ΔPPowerClass _PC2 and ΔPPowerClass _PC3 which have the following ΔPPowerClass values depending on the SRS transmission power:

Capability#1: ΔPPowerClass _PC2 for PC2 SRS transmission power have the following ΔPPowerClass values:

ΔPPowerClass  = 3dB for t1ry

ΔPPowerClass  = 0dB for t2ry

ΔPPowerClass  = 0dB for t4ry

Capability#2: ΔPPowerClass _PC3 for PC3 SRS transmission power have the following ΔPPowerClass values:

ΔPPowerClass  = 6dB for t1ry

ΔPPowerClass  = 3dB for t2ry

ΔPPowerClass  = 0dB for t4ry

We have also submitted a related LS [3] to this meeting detailing the above proposal

# Conclusion

In this paper we discuss our views on how configure ΔPPowerClass for 4Tx for SRS antenna switching and make the following proposal.

Proposal: Introduce 2 capabilities ΔPPowerClass _PC2 and ΔPPowerClass _PC3 which have the following ΔPPowerClass values depending on the SRS transmission power:

Capability#1: ΔPPowerClass _PC2 for PC2 SRS transmission power have the following ΔPPowerClass values:

ΔPPowerClass  = 3dB for t1ry

ΔPPowerClass  = 0dB for t2ry

ΔPPowerClass  = 0dB for t4ry

Capability#2: ΔPPowerClass _PC3 for PC3 SRS transmission power have the following ΔPPowerClass values:

ΔPPowerClass  = 6dB for t1ry

ΔPPowerClass  = 3dB for t2ry

ΔPPowerClass  = 0dB for t4ry

# References

[1] R4-2502187, On DeltaP_PowerClass for SRS-AS for 4Tx, February 17-21, 2025

[2] R4-2500377, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, February 17-21, 2025

[3] R4-2507713, Capabilities for Delta PPowerClass for 4Tx SRS transmission, May 19-23, 2025