3GPP TSG-RAN WG4 Meeting #115 	R4-2507739

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.30.2

Source: 	Ericsson

Title:	On RRM requirements for XR

Document for:	Approval

# Introduction

The RAN4#115 WF is in [9]. The RAN4#114 WF is in [7], RAN4#113 WF is in [6]. The RAN4#112-bis WF is in [5]. The RAN4#112 agreements and WF are in [4]. The RAN4-related objectives of the Rel-19 XR Phase 3 WI are captured in [1].

The following agreements were made in RAN4#114bis [9]:

- Deployment scenarios

  - Skipping requirements applies for NR-SA

- L1/L2 triggered mobility measurements and procedures

  - Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping.

- Types of measurement gaps

  - Gaps which shall not be cancelled

    - MUSIM

    - Positioning

  - For other gap types:

    - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19.

    - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

- Time offset requirement for measurement gap skipping

  - Not define additional condition for the optional capability of 3 ms.

- General on UAI

  - Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting.

  - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

    - R ∈ {0, 20, 40, 60}%

Below we further discuss the issues related to RAN4 requirements impact for UE operating XR.

# Discussion

## Types of MGs

In [6], it was agreed that Type 1 MGs are used as a baseline, while the relevance for cancellation of other MG types and MG purposes needs further discussion in RAN4. In [9], it was agreed that other types of measurement gaps can be further discussed, but the discussion in RAN4#115 should start with Type 2 measurement gaps (gaps configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17). As of the concurrent measurement gaps, they are not prioritized for the discussion while create some additional issues, e.g., due to the 5 ms time offset the network would additionally be required to separate well enough the concurrent gap patterns in time.

- Proposal 1: The requirements with MG cancellation are not applicable for UEs configured with concurrent measurement gaps.

## On the impact of UL XR operation

The UL XR operation can also be scheduled during the cancelled MGs, therefore the impact of scheduling the UL XR transmissions during the cancelled MGs needs to be discussed due to the potential impact on the RAN4 requirements.

At least the following issues need RAN4 discussion and their relation to the cancelled MGs:

Correct UL timing:

- the UE needs to have the correct UL timing prior staring the UL XR operation, which may result in delayed UL operation and lost resources of the cancelled MGs.

A guard period,

- which is needed for DL-to-UL switching for UL XR operation within the target MG, to avoid UL-to-DL interference.

RF switching:

- needed for the UE to switch from DL to UL and back.

- Proposal 2: For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG.

- Proposal 3: For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG.

- Proposal 4: When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion.

## Semi-static MG cancellation schemes

In the last 3GPP meeting, RAN2 has agreed to not further discuss semi-static MG cancellation schemes, and RAN1 confirmed the earlier RAN1 agreement on the DCI approach (see below).

Agreement Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.. Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

- Proposal 5: RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

## RRM requirements enhancements for XR

NR-U like approach was agreed for defining measurement period for measurements impacted by MG cancellation under XR operation.

### Inter-frequency cell detection

- Proposal 6: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

| Condition | FR1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8+XPSS/SSS,gaps) DRX cycle CSSFinter |

| Condition NOTE1 | FR2-1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |

- Proposal 7: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=0.5  Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 8: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded:

Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

### SSB time index detection

- Proposal 9: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, (3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (3+Xind,gaps) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, (MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |

- Proposal 10: The maximum numbers of extensions of inter-frequency SSB time index detection period are for

FR1:

Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xind,gaps,max=1.5  MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=0.5  MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 11: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded:

Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

### Inter-frequency measurements

- Proposal 12: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, (8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8 + Xmeas) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, (Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |

- Proposal 13: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xmeas=1.5  Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

Xmeas=0.5  Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 14: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded:

upon exceeding Xmeas,max the UE shall restart the measurement.

### Applicability conditions

Type 1 MGs are configured by several configuration parameters, including MGL and MGRP and can be per-UE or per-FR. Not all MG configurations are equally important for XR, when MGs are cancelled to reallocate resource to XR.

- Proposal 15: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 16: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

### UE power class

Considering the WI scope, practical XR scenarios, and scenarios when the same UE can be configured with measurement gaps or inter-frequency measurements, at least some of the UE power classes are not relevant in the requirements related to measurement gap cancellation for XR. The most typical UE power class which would be relevant for such requirements is UE power class 3.

- Proposal 17: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: The requirements with MG cancellation are not applicable for UEs configured with concurrent measurement gaps.

- Proposal 2: For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG.

- Proposal 3: For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG.

- Proposal 4: When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion.

- Proposal 5: RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

- Proposal 6: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

| Condition | FR1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8+XPSS/SSS,gaps) DRX cycle CSSFinter |

| Condition NOTE1 | FR2-1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |

- Proposal 7: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=0.5  Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 8: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded:

Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

- Proposal 9: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, (3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (3+Xind,gaps) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, (MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |

- Proposal 10: The maximum numbers of extensions of inter-frequency SSB time index detection period are for

FR1:

Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xind,gaps,max=1.5  MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=0.5  MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 11: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded:

Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

- Proposal 12: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, (8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8 + Xmeas) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, (Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |

- Proposal 13: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xmeas=1.5  Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

Xmeas=0.5  Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 14: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded:

Upon exceeding Xmeas,max the UE shall restart the measurement.

- Proposal 15: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Proposal 16: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

- Proposal 17: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.

[5] R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, Oct. 2024.

[6] R4-2420103, WF on RRM requirements for XR_Ph3, Qualcomm, Nov. 2024.

[7] R4-2502700, WF on RRM requirements for XR_Ph3, Nokia, Feb. 2025.

[8] R4-2503921, draftCR to 38.133, Inter-frequency measurement requirements with XR, Ericsson, Apr. 2025.

[9] R4-2504911, WF on RRM requirements for XR_Ph3, Nokia, Apr. 2025.