3GPP TSG-RAN WG4 Meeting #115 	R4-2507741

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.20.3

Source: 	Ericsson

Title:	On RRM requirements for NavIC L1 SPS AGNSS support in NR & LTE

Document for:	Approval

# Introduction

In [1], a WI was agreed on introducing in Rel-19 3GPP specifications the support for A-GNSS with NavIC L1 band SPS, applicable for both NR and LTE. The WI has the core and performance part objectives in [1].

Core part objectives:

- Specify signalling support for NavIC L1 band SPS in NR specifications by introducing necessary assistance data information (e.g. Clock Model for NavIC, Navigation Model for NavIC, and Almanac for NavIC L1 SPS) based on the existing GNSS framework, including enhancements to Ionospheric model, NavIC system time and Ephemeris. [RAN2].

- Specify signalling support for NavIC L1 band SPS in LTE specifications similar to NR [RAN2].

Performance part objectives:

- Specify UE performance requirements for support of NavIC L1 SPS for NR & LTE using existing A-GNSS minimum performance requirements framework [RAN4].

The expected output on the RAN4-related performance part is as below [1]:

| 38.171 | Specify UE performance requirements for support of NavIC L1 SPS for NR using existing A-GNSS minimum performance requirements | TSG#109 |
| --- | --- | --- |
| 36.171 | Specify UE performance requirements for support of NavIC L1 SPS for LTE using existing A-GNSS minimum performance requirements | TSG#109 |

The RAN4 work is expected to be finalized, according to the time plan, in RAN4#116 (Aug. 2025) and reported to the RAN Plenary at RAN#109 (Sep. 2025).

In this contribution, we discuss the potential updates to TS 38.171 and TS 36.171, according to the WI objective [1].

# Discussion

The A-GNSS support for first generation NavIC L5 band SPS in LTE specifications was introduced in Release 16. In release 17, the NavIC L5 support was specified for NR. India has now introduced second generation of NavIC satellite services. Compared to the first-generation satellite series, the second-generation satellite series includes L1 navigation band and incorporates indigenously developed rubidium atomic lock. Hence, some support of NavIC bands (L5) in 3GPP specifications already exists, for both NR and LTE, but it needs to be extended to also accommodate the new L1 band.

The RAN2 updates to TS 37.355 include the additions of:

- GNSS clock model for NavIC L1 (Model 9 added)

GNSS-ClockModel ::= CHOICE {

standardClockModelList	StandardClockModelList,				-- Model-1

nav-ClockModel			NAV-ClockModel,					-- Model-2

cnav-ClockModel			CNAV-ClockModel,					-- Model-3

glonass-ClockModel			GLONASS-ClockModel,				-- Model-4

sbas-ClockModel			SBAS-ClockModel,					-- Model-5

...,

bds-ClockModel-r12			BDS-ClockModel-r12,				-- Model-6

bds-ClockModel2-r16		BDS-ClockModel2-r16,				-- Model-7

navic-ClockModel-r16		NavIC-ClockModel-r16,				-- Model-8

navic-ClockModel2-r19		NavIC-ClockModel2-r19				-- Model-9

}

- GNSS orbit model for NavIC L1 (Model 9 added)

GNSS-OrbitModel ::= CHOICE {

keplerianSet				NavModelKeplerianSet,			-- Model-1

nav-KeplerianSet		NavModelNAV-KeplerianSet,		-- Model-2

cnav-KeplerianSet		NavModelCNAV-KeplerianSet,		-- Model-3

glonass-ECEF			NavModel-GLONASS-ECEF,			-- Model-4

sbas-ECEF				NavModel-SBAS-ECEF,				-- Model-5

...,

bds-KeplerianSet-r12	NavModel-BDS-KeplerianSet-r12,	-- Model-6

bds-KeplerianSet2-r16	NavModel-BDS-KeplerianSet2-r16,	-- Model-7

navic-KeplerianSet-r16	NavModel-NavIC-KeplerianSet-r16,	-- Model-8

navic-KeplerianSet2-r19	NavModel-NavIC-KeplerianSet2-r19 -- Model-9

}

- GNSS almanac choice for NavIC (Model 9 added)

GNSS-AlmanacElement ::= CHOICE {

keplerianAlmanacSet				AlmanacKeplerianSet,		-- Model-1

keplerianNAV-Almanac			AlmanacNAV-KeplerianSet,	-- Model-2

keplerianReducedAlmanac			AlmanacReducedKeplerianSet,	-- Model-3

keplerianMidiAlmanac			AlmanacMidiAlmanacSet,		-- Model-4

keplerianGLONASS				AlmanacGLONASS-AlmanacSet,	-- Model-5

ecef-SBAS-Almanac				AlmanacECEF-SBAS-AlmanacSet,-- Model-6

...,

keplerianBDS-Almanac-r12		AlmanacBDS-AlmanacSet-r12,	-- Model-7

keplerianNavIC-Almanac-r16		AlmanacNavIC-AlmanacSet-r16,	-- Model-8

keplerianNavIC-Almanac2-r19		AlmanacNavIC-AlmanacSet2-r19	-- Model-9

}

- GNSS ionospheric model for NavIC

GNSS-IonosphericModel ::= SEQUENCE {

klobucharModel			KlobucharModelParameter		OPTIONAL,	-- Need ON

neQuickModel			NeQuickModelParameter		OPTIONAL,	-- Need ON

...,

[[	klobucharModel2-r16		KlobucharModel2Parameter-r16	OPTIONAL	-- Need ON

]],

[[	neQuickModel2-r19		NeQuickModel2Parameter-r19	OPTIONAL	-- Need ON

]]

}

Based on the above, the following changes are necessary for introducing the support of GNSS NavIC L1 band for NR in TS 38.171:

- Section 4.8, Table 4.8-1 (Relative signal power levels for each signal type for each GNSS) needs to be updated with the corresponding power level for L1.

- In Annex E.2, Table E.4, the GNSS-IonosphericModel IE need to be updated to account for L1.

- In Annex E.2, Table E.7, the GNSS clock and orbit model need to be added for L1.

- In Annex E.2, Table E.10, the GNSS Almanac choice needs to be added for L1.

And the corresponding necessary changes for GNSS NavIC L1 band for LTE in TS 36.171 are:

- Section 4.8, Table 4.1 (Relative signal power levels for each signal type for each GNSS) needs to be updated with the corresponding power level for L1.

- In Annex E.2, Table E.4, the GNSS-IonosphericModel IE need to be updated to account for L1.

- In Annex E.2, Table E.7, the GNSS clock and orbit model need to be added for L1.

- In Annex E.2, Table E.10, the GNSS Almanac choice needs to be added for L1.

# Summary

- Proposal 1: The following changes are necessary for introducing the support of GNSS NavIC L1 band for NR in TS 38.171:

  - Section 4.8, Table 4.8-1 (Relative signal power levels for each signal type for each GNSS) needs to be updated with the corresponding power level for L1.

  - In Annex E.2, Table E.4, the GNSS-IonosphericModel IE need to be updated to account for L1.

  - In Annex E.2, Table E.7, the GNSS clock and orbit model need to be added for L1.

  - In Annex E.2, Table E.10, the GNSS Almanac choice needs to be added for L1.

- Proposal 2: The following changes are necessary for introducing the support of GNSS NavIC L1 band for LTE in TS 36.171:

  - Section 4.8, Table 4.1 (Relative signal power levels for each signal type for each GNSS) needs to be updated with the corresponding power level for L1.

  - In Annex E.2, Table E.4, the GNSS-IonosphericModel IE need to be updated to account for L1.

  - In Annex E.2, Table E.7, the GNSS clock and orbit model need to be added for L1.

  - In Annex E.2, Table E.10, the GNSS Almanac choice needs to be added for L1.

# References

[1] RP-243247, Revised WID on Introduction of A-GNSS support for NavIC L1 SPS in NR and LTE, Reliance Jio, Dec. 2024.