3GPP TSG-RAN WG4 Meeting #115				     			         		R4-2507742

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.1.2.2.1	Power domain enhancements for single carrier	[NR_ENDC_RF_Ph4-Core]

Title: 	RB and BW for 2x and 1.5x extension and optional smaller granularity extension

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b, we provided further details on our proposal to signal extended emission BW for UEs based on RB extension [1]. Further agreements were captured in way forward [2] on IBE and RB extension. In this contribution, we simplify the approach on defining guard bands and propose two additional extended bandwidths that are less than 0.5x extensions.

# Discussion

## 2.1 Way forward input

Several agreements related to the extended UE bandwidth were captured in the way forwards [2]. The key agreements and open issues are summarized here in italics.

#### Issue 1-1-1: RB based or CBW based extension

- Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB

- If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5

- The extended RBs are aligned with common RB grid

#### Issue 1-1-2: Fixed or flexible extension

Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature

- In addition, consider a limited number of additional fixed values with capability

#### Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW

| Option1. Individual evaluation of the extended RBs & Horizontal line | Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line |
| --- | --- |
| Case1 | Case3 |

- Case 1 and case 3 could be further down selected

- GB of UE CBW may or may not be scheduled to other UEs

- Extended IBE can be applied in the GB of UE CBW

  - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW

#### Issue 1-3-2: UE capability

- An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature

- To discuss the following aspects

  - Whether to have separate UE capabilities for scenario 1 and scenario 2

  - Whether to indicate multiple values of fixed extension capability

## 2.2 Bandwidth extension for 2x or 1.5x

In the RAN4#114b meeting, it was agreed that the half bandwidth extension can be asymmetric or symmetric and is determined by 0.5x NRB extension per side and the extended RBs are RB aligned with the UE RBs from the UE RBs edge.

Still, this does not clarify where the emissions to be met starts and the resulting guard band. In [1], we proposed a way to derive the guard band based on the NRB extension, but this resulted in the extended bandwidth being numbers that are not rounded (at least to half a MHz).

Since we recognize that this may be complex and result in 2x extensions not falling on existing CBW, we propose an hybrid approach:

- The RBs are extended by ceil(NRB/2) in a symmetric way (2x extension) or asymmetric way (1.5x extension)

- The BW is extended by CBW/2 in a symmetric way (2x extension) or asymmetric way (1.5x extension)

This approach is described versus existing UE CBW in detail in Table 1 which provides:

- In the first 4 columns: the SCS, CBW, NRB and guard-band of the existing CBW

- In the next three columns Ceil(NB/2) and the 2x and 1.5xNRB values

- In the next two columns the 2x and 1.5x extended bandwidths

- In the next two columns the resulting 2x and 1.5x guard-bands

- In the last two columns the guard bands that were proposed and checked with measurements in [1].

Table 1: NRB, emission BW and guard bands for 2x and 1.5x extended bandwidth

Observations:

- Except for the 3MHz case:

  - The NRB/2 and CBW/2 based guard band is larger than proposed the guard-band and verified by measurements in [1].

  - The guard band of 2x/1.5x extended BW is larger than the corresponding existing CBW cases.

  - Thus, the extended BW emissions can verified by verified by testing the existing CBW.

Proposal on 2x and 1.5x extension and associated guard band:

- The maximum bandwidth extension is based on adding ceil(NRB/2) symmetrically or asymmetrically to the initial UE CBW NRB:

  - A symmetrically extended NRB (NRBext) = NRB+2*ceil(NRB/2)

  - An asymmetrically extended NRB (NRBext) = NRB+ceil(NRB/2)

- The corresponding extended bandwidth is based on CBW/2 extensions:

  - A symmetrically extended bandwidth (BWext) = 2xCBW

  - An asymmetrically extended bandwidth (BWext) = CBW+CBW/2

- Thus, the resulting minimum guard band on the extended side is:

  - A symmetrically extended minimum guard-band (GBext) = 2xCBW-(NRB+2*Ceil(NRB/2))-SCS/2

  - An asymmetrically extended minimum guard-band (GBext) = 1.5xCBW-(NRB+Ceil(NRB/2))-SCS/2

    - The side that is not extended retains its initial CBWminimum guard-band

- Based on this guard-band approach we propose that RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band as our measurements and relaxed extended guard band suggests.

- In all cases, it is the BS responsibility to ensure that the RB extension enables the use of the extended guard band to define the out-of-band emissions based on BWext and initial CBW based OOBE regions.

  - In cases where a nearby band needs to be protected by the UE, the BS ensures that the RB extension and corresponding guard band is set in accordance with the protected band region by making sure that the protected band is within the spurious emission range (-30dBm/MHz) of the extended BW OOBE region.

## 2.3 Flexible RB Extension

In our contribution [1] we provided all the cases that could benefit from extension of less than NRB/2 and CBW/2.

In the spirit of further simplifying this optional approach and adapting to the proposed 2x and 1.5x extensions in this paper, we propose the following approach:

- Extension of less than 0.5x are only applicable to one side, the other side seeing no extension or 0.5x extension.

- The <0.5x extensions is based on 5/10/15/20MHz extensions for 15KHz SCS and 5/10/20/30MHz extensions for 30kHz SCS, and the associated NRB extension is the NRB associated with the corresponding existing CBW NRB of the same SCS.

- For any initial CBW, only two <0.5x extensions are possible as mapped in Table 2 where:

  - The first three columns provide the existing SCS, CBW and NRB

  - The next 5 columns map the applicable <0.5x extension per CBW and SCS

  - The last 5 columns provide the associated RB extension.

Table 2: Mapping of <0.5x bandwidth extensions and their additional RBs

| SCS [kHz] | UE BW |  | less than 0.5x extension option |  |  |  |  | less than 0.5x extension added RBs |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CBW | NRB | 5MHz | 10MHz | 15MHz | 20MHz | 30MHz | 5MHz | 10MHz | 15MHz | 20MHz | 30MHz |
| 15 | 3 | 15 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 5 | 25 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 7 | 35 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 10 | 52 | =0.5x | NA | NA | NA | NA | =0.5x | NA | NA | NA | NA |
|  | 15 | 79 | y | NA | NA | NA | NA | 25 | NA | NA | NA | NA |
|  | 20 | 106 | y | =0.5x | NA | NA | NA | 25 | =0.5x | NA | NA | NA |
|  | 25 | 133 | y | y | NA | NA | NA | 25 | 52 | NA | NA | NA |
|  | 30 | 160 | y | y | =0.5x | NA | NA | 25 | 52 | =0.5x | NA | NA |
|  | 35 | 188 | NA | y | y | NA | NA | NA | 52 | 79 | NA | NA |
|  | 40 | 216 | NA | y | y | =0.5x | NA | NA | 52 | 79 | =0.5x | NA |
|  | 45 | 242 | NA | NA | y | y | NA | NA | NA | 79 | 106 | NA |
|  | 50 | 270 | NA | NA | y | y | NA | NA | NA | 79 | 106 | NA |
| 30 | 5 | 11 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 10 | 24 | =0.5x | NA | NA | NA | NA | =0.5x | NA | NA | NA | NA |
|  | 15 | 38 | y | NA | NA | NA | NA | 11 | NA | NA | NA | NA |
|  | 20 | 51 | y | =0.5x | NA | NA | NA | 11 | =0.5x | NA | NA | NA |
|  | 25 | 65 | y | y | NA | NA | NA | 11 | 24 | NA | NA | NA |
|  | 30 | 78 | y | y | NA | NA | NA | 11 | 24 | NA | NA | NA |
|  | 35 | 92 | y | y | NA | NA | NA | 11 | 24 | NA | NA | NA |
|  | 40 | 106 | y | y | NA | =0.5x | NA | 11 | 24 | NA | =0.5x | NA |
|  | 45 | 119 | NA | y | NA | y | NA | NA | 24 | NA | 51 | NA |
|  | 50 | 133 | NA | y | NA | y | NA | NA | 24 | NA | 51 | NA |
|  | 60 | 162 | NA | y | NA | y | =0.5x | NA | 24 | NA | 51 | =0.5x |
|  | 70 | 189 | NA | NA | NA | y | y | NA | NA | NA | 51 | 78 |
|  | 80 | 217 | NA | NA | NA | y | y | NA | NA | NA | 51 | 78 |
|  | 90 | 245 | NA | NA | NA | y | y | NA | NA | NA | 51 | 78 |
|  | 100 | 273 | NA | NA | NA | y | y | NA | NA | NA | 51 | 78 |

Observation:

- <0.5x extension only applies from 15MHz CBW and above

- There are only two possible <0.5x extension per CBW and SCS

- The max RB extension is 106 for 15kHz vs 135 for the 0.5x extension for 50MHz

- The max RB extension is 78 for 30kHz vs 137 for the 0.5x extension for 100MHz

Since the RB added is the RB of the corresponding existing CBW, the initial guard band on the side with <0.5x extension is extended by the sum of the two sides guard-bands of the corresponding existing CBW. This results in a comfortable guard band that guaranties that the emissions can be met by verifying only the existing CBW supported in the band.

Proposal on less than 0.5x extension:

- For CBW > 15MHz two extensions values of less than 0.5x can be optionally used asymmetrically on top of the 0.5x symmetric and asymmetric cases

- Per CBW, the <0.5x extensions on the two closest CBW from 5/10/15/20MHz extensions for 15KHz SCS and 5/10/20/30MHz extensions for 30kHz SCS that are <CBW/2

- The associated <0.5x NRB extension is the NRB associated with the corresponding existing CBW NRB of the same SCS.

- In many cases the resulting BW with or without 0.5x extension on the other side corresponds to an existing CBW and RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band as our measurements and further relaxed extended guard band suggests.

- The applicable <0.5x extensions is mapped in the following table with the corresponding RB extension.

| SCS [kHz] | UE BW |  | less than 0.5x extension |  |
| --- | --- | --- | --- | --- |
|  | CBW | NRB | BW extension | RB extension |
| 15 | 15 | 79 | 5MHz | 25 |
|  | 20 | 106 | 5MHz | 25 |
|  | 25 | 133 | 5 and 10 MHz | 25 and 52 |
|  | 30 | 160 | 5 and 10 MHz | 25 and 52 |
|  | 35 | 188 | 10 and 15 MHz | NA |
|  | 40 | 216 | 10 and 15 MHz | NA |
|  | 45 | 242 | 15 and 20 MHz | NA |
|  | 50 | 270 | 15 and 20 MHz | NA |
| 30 | 15 | 38 | 5MHz | 11 |
|  | 20 | 51 | 5MHz | 11 |
|  | 25 | 65 | 5 and 10 MHz | 11 and 24 |
|  | 30 | 78 | 5 and 10 MHz | 11 and 24 |
|  | 35 | 92 | 5 and 10 MHz | 11 and 24 |
|  | 40 | 106 | 5 and 10 MHz | 11 and 24 |
|  | 45 | 119 | 10 and 20 MHz | 24 and 51 |
|  | 50 | 133 | 10 and 20 MHz | 24 and 51 |
|  | 60 | 162 | 10 and 20 MHz | 24 and 51 |
|  | 70 | 189 | 20 and 30 MHz | 51 and 78 |
|  | 80 | 217 | 20 and 30 MHz | 51 and 78 |
|  | 90 | 245 | 20 and 30 MHz | 51 and 78 |
|  | 100 | 273 | 20 and 30 MHz | 51 and 78 |

## 2.4 UE capability

As proposed above, the BS signalling needs to encompass the following cases:

- Symmetric or asymmetric extension of 0.5x CBW/NRB

- Asymmetric extension of <0.5x with/without .5x extension on the other side

UEs can declare the support of the capability to apply inner RB MPR to outer RBs according the signalled extension:

- Only for 0.5x symmetric and asymmetric extension

- For 0.5x symmetric and asymmetric extension and <0.5x asymmetric extension

Proposal on signalling:

- Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side extensions with:

  - 2 bits towards UE only supporting 0.5x symmetric and asymmetric extension with the logic table below.

| 0.5x extension side |  | Comment: extension below/above the initial UE CBW |
| --- | --- | --- |
| low | high |  |
| 0 | 0 | No extension |
| 0 | 1 | 0.5x above |
| 1 | 0 | 0.5x below |
| 1 | 1 | 0.5x below and above |

  - 4 bits towards UE supporting 0.5x symmetric and asymmetric extension with <0.5x asymmetric extension with the logic table below.

| <0.5x extension |  | extension side/size |  | Comment: extension below/above the initial UE CBW |
| --- | --- | --- | --- | --- |
| low | high | low | high |  |
| 0 | 0 | 0 | 0 | No extension |
| 0 | 0 | 0 | 1 | 0.5x above |
| 0 | 0 | 1 | 0 | 0.5x below |
| 0 | 0 | 1 | 1 | 0.5x below and above |
| 0 | 1 | 0 | 0 | lowest applicable <0.5x extension above |
| 0 | 1 | 0 | 1 | highest applicable <0.5x extension above |
| 0 | 1 | 1 | 0 | 0.5x extension below and lowest applicable <0.5x extension above |
| 0 | 1 | 1 | 1 | 0.5x extension below and highest applicable <0.5x extension above |
| 1 | 0 | 0 | 0 | lowest applicable <0.5x extension below |
| 1 | 0 | 0 | 1 | highest applicable <0.5x extension below |
| 1 | 0 | 1 | 0 | 0.5x extension above and lowest applicable <0.5x extension below |
| 1 | 0 | 1 | 1 | 0.5x extension above and highest applicable <0.5x extension below |

- Inner and outer allocation can be redefined by the UE using the existing inner/outer allocation equations where:

  - NRB in the inner equations of TS 38.101-1 clause .2.2 is replaced by NRB_UE+RBextL+RBextH based on 0.5x and <0.5x NRB extensions

  - RBstart in the inner equations of TS 38.101-1 clause .2.2 is replaced by RBextL+RBstart_UE

- A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability are needed:

  - One where the UE is only able to apply inner RB MPR to outer/edge RBs based on 0.5x symmetric and asymmetric extension

  - One where the UE is only able to apply inner RB MPR to outer/edge RBs based on 0.5x symmetric and asymmetric or <0.5x asymmetric extension.

# Conclusion

In this contribution, we further explained and refined our proposals from [1] based on last meeting discussions.

Proposal on 2x and 1.5x extension and associated guard band:

- The maximum bandwidth extension is based on adding ceil(NRB/2) symmetrically or asymmetrically to the initial UE CBW NRB:

  - A symmetrically extended NRB (NRBext) = NRB+2*ceil(NRB/2)

  - An asymmetrically extended NRB (NRBext) = NRB+ceil(NRB/2)

- The corresponding extended bandwidth is based on CBW/2 extensions:

  - A symmetrically extended bandwidth (BWext) = 2xCBW

  - An asymmetrically extended bandwidth (BWext) = CBW+CBW/2

- Thus, the resulting minimum guard band on the extended side is:

  - A symmetrically extended minimum guard-band (GBext) = 2xCBW-(NRB+2*Ceil(NRB/2))-SCS/2

  - An asymmetrically extended minimum guard-band (GBext) = 1.5xCBW-(NRB+Ceil(NRB/2))-SCS/2

    - The side that is not extended retains its initial CBWminimum guard-band

- Based on this guard-band approach we propose that RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band as our measurements and relaxed extended guard band suggests.

- In all cases, it is the BS responsibility to ensure that the RB extension enables the use of the extended guard band to define the out-of-band emissions based on BWext and initial CBW based OOBE regions.

  - In cases where a nearby band needs to be protected by the UE, the BS ensures that the RB extension and corresponding guard band is set in accordance with the protected band region by making sure that the protected band is within the spurious emission range (-30dBm/MHz) of the extended BW OOBE region.

Proposal on less than 0.5x extension:

- For CBW > 15MHz two extensions values of less than 0.5x can be optionally used asymmetrically on top of the 0.5x symmetric and asymmetric cases

- Per CBW, the <0.5x extensions on the two closest CBW from 5/10/15/20MHz extensions for 15KHz SCS and 5/10/20/30MHz extensions for 30kHz SCS that are <CBW/2

- The associated <0.5x NRB extension is the NRB associated with the corresponding existing CBW NRB of the same SCS.

- In many cases the resulting BW with or without 0.5x extension on the other side corresponds to an existing CBW and RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band as our measurements and further relaxed extended guard band suggests.

- The applicable <0.5x extensions is mapped in the following table with the corresponding RB extension.

| SCS [kHz] | UE BW |  | less than 0.5x extension |  |
| --- | --- | --- | --- | --- |
|  | CBW | NRB | BW extension | RB extension |
| 15 | 15 | 79 | 5MHz | 25 |
|  | 20 | 106 | 5MHz | 25 |
|  | 25 | 133 | 5 and 10 MHz | 25 and 52 |
|  | 30 | 160 | 5 and 10 MHz | 25 and 52 |
|  | 35 | 188 | 10 and 15 MHz | NA |
|  | 40 | 216 | 10 and 15 MHz | NA |
|  | 45 | 242 | 15 and 20 MHz | NA |
|  | 50 | 270 | 15 and 20 MHz | NA |
| 30 | 15 | 38 | 5MHz | 11 |
|  | 20 | 51 | 5MHz | 11 |
|  | 25 | 65 | 5 and 10 MHz | 11 and 24 |
|  | 30 | 78 | 5 and 10 MHz | 11 and 24 |
|  | 35 | 92 | 5 and 10 MHz | 11 and 24 |
|  | 40 | 106 | 5 and 10 MHz | 11 and 24 |
|  | 45 | 119 | 10 and 20 MHz | 24 and 51 |
|  | 50 | 133 | 10 and 20 MHz | 24 and 51 |
|  | 60 | 162 | 10 and 20 MHz | 24 and 51 |
|  | 70 | 189 | 20 and 30 MHz | 51 and 78 |
|  | 80 | 217 | 20 and 30 MHz | 51 and 78 |
|  | 90 | 245 | 20 and 30 MHz | 51 and 78 |
|  | 100 | 273 | 20 and 30 MHz | 51 and 78 |

Proposal on signalling:

- Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side extensions with:

  - 2 bits towards UE only supporting 0.5x symmetric and asymmetric extension with the logic table below.

| 0.5x extension side |  | Comment: extension below/above the initial UE CBW |
| --- | --- | --- |
| low | high |  |
| 0 | 0 | No extension |
| 0 | 1 | 0.5x above |
| 1 | 0 | 0.5x below |
| 1 | 1 | 0.5x below and above |

  - 4 bits towards UE supporting 0.5x symmetric and asymmetric extension with <0.5x asymmetric extension with the logic table below.

| <0.5x extension |  | extension side/size |  | Comment: extension below/above the initial UE CBW |
| --- | --- | --- | --- | --- |
| low | high | low | high |  |
| 0 | 0 | 0 | 0 | No extension |
| 0 | 0 | 0 | 1 | 0.5x above |
| 0 | 0 | 1 | 0 | 0.5x below |
| 0 | 0 | 1 | 1 | 0.5x below and above |
| 0 | 1 | 0 | 0 | lowest applicable <0.5x extension above |
| 0 | 1 | 0 | 1 | highest applicable <0.5x extension above |
| 0 | 1 | 1 | 0 | 0.5x extension below and lowest applicable <0.5x extension above |
| 0 | 1 | 1 | 1 | 0.5x extension below and highest applicable <0.5x extension above |
| 1 | 0 | 0 | 0 | lowest applicable <0.5x extension below |
| 1 | 0 | 0 | 1 | highest applicable <0.5x extension below |
| 1 | 0 | 1 | 0 | 0.5x extension above and lowest applicable <0.5x extension below |
| 1 | 0 | 1 | 1 | 0.5x extension above and highest applicable <0.5x extension below |

- Inner and outer allocation can be redefined by the UE using the existing inner/outer allocation equations where:

  - NRB in the inner equations of TS 38.101-1 clause .2.2 is replaced by NRB_UE+RBextL+RBextH based on 0.5x and <0.5x NRB extensions

  - RBstart in the inner equations of TS 38.101-1 clause .2.2 is replaced by RBextL+RBstart_UE

- A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability are needed:

  - One where the UE is only able to apply inner RB MPR to outer/edge RBs based on 0.5x symmetric and asymmetric extension

  - One where the UE is only able to apply inner RB MPR to outer/edge RBs based on 0.5x symmetric and asymmetric or <0.5x asymmetric extension.

# References

[1] R4-2503237 Flexible RB extension for improved MPR when NW BW is larger than UE CBW, Skyworks Solutions Inc., R4#114b

[2] R4-2505099 WF on power domain enhancements, Huawei, R4#114b