3GPP TSG-RAN WG4 Meeting #115	R4-2507744

# Malta, Malta,  19th May – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0182 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-5: NS value for additional Transmit OFF state requirement for RA protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Aalyria |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_combinedLband-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NS value for additional Transmit OFF power requirements for radioastronomy (RA) protection in Combined NR-NTN L-bands, according to ETSI EN 301 681. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a new NS value under Clause 6.3.2, specifying additional Transmit OFF power requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not be able to respect the requirements for protection of Radioastronomy (RA) stations in Transmit OFF state, when operating under ETSI regulations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START CHANGED SECTION **********

### 6.3.2	Transmit OFF power

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1 ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.2-1.

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth | (MHz) | 5, 10, 15, 20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| Transmit OFF power | (dBm) | -50 |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

When NS_11N is signaled, the UE shall meet the following additional requirements for Transmit OFF power specified in Table 6.3.2-2, for any channel bandwidth in the bands n253, n251 and n250.

Table 6.3.2-2 Additional EN 301 681 Tranmit OFF power requirements

| NR Band | NS Value | Frequency (MHz) | Emission Limit (dBW) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- | --- | --- |
| n253, n251, n250 | NS_11N | 30 to 1 000 | -87 | 100 kHz | Peak Hold |
|  |  | 1 000 to 1 525 | -87 | 100 kHz | Peak Hold |
|  |  | 1 525 to 1 559 | -975 | 100 kHz | Average |
|  |  | 1 559 to 1 610 | -70 | 1 MHz3 | Average4 |
|  |  | 1 610 to 12 750 | -87 | 100 kHz | Peak Hold |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: Unless otherwise stated, the EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 4: The average measurement method defined in clause 5.2.2.3 of ETSI EN 301 681 shall apply except that a measurement time of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 5: If MES antenna gain is higher than 8 dBi, the unwanted emission limit in the band 1 525 MHz to 1 559 MHz shall be considered to be -90 dBW in 100 kHz. |  |  |  |  |  |

### 6.3.3	Transmit ON/OFF time mask

The requirements for transmit ON/OFF time mask defined in 3GPP TS 38.101-1 [5] clause 6.3.3 shall apply for NTN satellite UE.

********** END CHANGED SECTION **********