3GPP TSG-RAN WG4 Meeting#115	R4-2507748

# St. Julian’s, Malta, 7 – 11 April 2025

Source: 	Qualcomm Incorporated

Title:	A-MPR for n26 PC2 and n1 PC2

Agenda Item:	6.5.3

Document for:	Approval

# 1	Introduction

This contribution provides further updates to n26 PC2 A-MPR. With more simulation and measurement data available small updates proposed, and some corrections are made to tables as in some cases proposed PC2 A-MPR was smaller than PC3 A-MPR. In addition, A-MPR for n1 NS_48 and NS_49 is discussed.

# 2	Discussion

n26 PC2

n26 PC2 A-MPR development started already in rel-18, with some results captured in TR 38.896. The data provided to 3GPP has been focused on QPSK modulation and the channel position closest to the emission requirements. We have performed additional measurements and simulations to check the A-MPR for all modulation orders and all center frequencies. It was observed that in some cases simulations provided slightly optimistic values. Based on those analyses small updates are proposed to the A-MPR tables.

The proposals also include error corrections. For example, for NS_13 A1-region, 256QAM A-MPR was 8 dB for PC3, but was earlier proposed to be only 5 dB for PC2. Similar error was present for NS_14 A3-region. There were also cases where A-MPR for CP-OFDM was smaller than A-MPR for DFT-s-OFDM, e.g. in NS_14 A4-region.

Tables below capture to proposed A-MPR, and updated cells compared to the draftCR in R4-2504186 have been highlighted in yellow. The tables are proposed as common to 1Tx and 2Tx implementations.

Proposal 1: Adopt A-MPR for n26 PC2 as proposed in tables below for NS_12 to NS_15

#### A-MPR for NS_12

Table 6.2.3.21-3: A-MPR regions for NS_12 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A3 |
|  | 817.5 ≤ Fc < 818.5 | ≤0.18 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
|  |  | >1.8 | >1.08 | A2 |
| 10MHz |  | ≤3.6 | >0 | A1 |
|  |  | >3.6, ≤5.04 |  | A2 |

Table 6.2.3.21-4: A-MPR for NS_12 (PC2)

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 4 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4.5 | ≤ 6.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 6.0 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 9 | 5.0 | ≤ 7.5 |
| CP-OFDM 16 QAM | ≤ 9 | 5.0 | ≤ 7.5 |
| CP-OFDM 64 QAM | ≤ 9 | 5.0 | ≤ 7.5 |
| CP-OFDM 256 QAM | ≤ 11.5 | 7.0 | ≤ 7.5 |

#### A-MPR for NS_13

Table 6.2.3.22-3: A-MPR regions for NS_13 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A1 |
|  |  | <0.54, ≤ 1.08 | ≥1.08 | A4 |
|  | 820 ≤ Fc < 821 | ≤0.18 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
|  |  | >1.44 | ≥1.8 | A3 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
|  |  |  | <3.24, ≥2.52 | A4 |

Table 6.2.3.22-4: A-MPR for NS_13 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 7 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 7.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7.5 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 10 | ≤ 10 | ≤ 7 |  |

#### A-MPR for NS_14

Table 6.2.3.23-3: A-MPR regions for NS_14 (Power Class 2)

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
|  | ≥0 | ≥8.1, <9 | A4 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
|  | ≤1.8 | ≥1.8, <3.6 | A4 |
|  | ≥0 | ≥8.1, <9 | A4 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |
|  | ≤3.42 | ≥1.8, <3.6 | A4 |
|  | ≥0 | ≥8.1, <9 | A4 |

Table 6.2.3.23-4: A-MPR for NS_14 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 4.5 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 6.5 | ≤ 6 | ≤ 8 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 5.5 | ≤ 7 | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 5.5 | ≤ 7 | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 5.5 | ≤ 7 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 7.5 | ≤ 8 | ≤ 6.5 |

#### A-MPR for NS_15

Table 6.2.3.24-3: A-MPR regions for NS_15 (Power Class 2)

| Channel BW | Carrier Frequency, Fc MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A2 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
|  | 843.0 < Fc ≤ 844.5 |  | ≥ 1.8 | A5 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
|  |  | <2.53, ≥1.8 | ≥1.44 | A5 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
|  | 837.5 < Fc ≤ 840.5 |  | ≥ 3.0 | A5 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  |  | <4.14, ≥1.8 | ≥2.52 | A5 |
|  |  | <2.52 | >0.36, ≤0.72 | A5 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
|  |  | <7.2, ≥5.22 | <4.32, ≥3.24 | A5 |
|  |  | <1.44 | >0.36, ≤0.72 | A5 |
|  | 829 < Fc ≤ 835 |  | ≥ 5.4 | A6 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  |  | <4.68, ≥1.8 | ≥2.52 | A5 |
|  |  | <3.96 | >0.36, ≤1.08 | A5 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  |  | <2.7 | >0.36, ≤0.72 | A5 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  |  | < 8.46, ≥1.8 | ≥3.6 | A5 |
|  |  | <6.12 | >0.36, ≤1.08 | A5 |
|  |  | <12.24, ≥8.46 | < 5.4, ≥3.6 | A5 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |
|  |  | <4.68 | >0.36, ≤1.08 | A5 |

Table 6.2.3.24-4: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 5 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 |

In addition to these channel bandwidths, work is on-going to also define 7 MHz channel bandwidth to n26. There is no mechanism for UE to indicate that is supports 7 MHz only for PC3. Therefore, PC2 requirements need to be defined also for 7 MHz CBW. There needs to be coordination between the two work items so that combination of having PC2 requirements for other bandwidths and only PC3 requirements for 7 MHz does not happen. If this WI introduces PC2 requirements for other CBW, 7 MHz cannot be added with only PC3 requirements.

Proposal 2: This WI and WI for 7 MHz CBW need to coordinate to ensure combination of having PC2 requirements for other bandwidths and only PC3 requirements for 7 MHz does not happen.

NS_48 and NS_49

Recently an update to NS_48 and NS_49 A-MPR has also been discussed, namely to address the additional allowance given the 2Tx implementation. During the review of the PC2 A-MPR, errors were found in the A-MPR definition. The general principle of PC3 and PC2 output is that PC2 should provide at least as much output power for every RB allocation. As A-MPR is not added on top of MPR, this means that PC2 A-MPR should be at most 3 dB increased compared PC3 A-MPR. In case A-MPR is allowed for PC2 for RB allocations where no A-MPR is specified for PC3, the A-MPR should be at most 3 dB higher than PC3 MPR. There are multiple instances where NS_48 and NS_49 A-MPR definitions violate this principle.

Some examples of the violations are given below, but the list is not exhaustive:

Figure 1: NS_48 A-MPR regions, PC3 on the left and PC2 on the right

Referring to the numbers in the figure above the errors are:

1 and 2: A2 to A1 A-MPR update is 6 dB A-MPR increase

3: Region is extended and covers inner allocations which get no A-MPR for PC3. A4 PC2 A-MPR is more than 3 dB increased from PC3 MPR.

4: In addition to same error as 1 and 2, RB region width is increased and therefore for some allocation A-MPR changes from A4 to A1, resulting in 8 dB A-MPR increase.

There are other similar errors in the NS_48 PC2 definition, even in the figure above.

Unfortunately NS_49 A-MPR has similar bugs.

Figure 2: NS_49 A-MPR regions, PC3 on the left and PC2 on the right

Referring to the numbers in the Figure 2 and just to give two examples:

1: A3 region is expanded, which for PC2 allows 4 dB A-MPR for some inner allocations which have 0 dB MPR for PC3

2: A5 region has changed to A1, and grown in size. PC3 A5 allows 5 dB, whereas PC2 A1 allows up to 13.5 dB.

NS_49 table unfortunately seems to have more these errors than what is reported above.

Observation 1: NS_48 and NS_49 PC2 A-MPR has many instances where PC2 is allowed to transmit less power than PC3.

Fixing these errors drastically reduce the PC2 A-MPR for some cases, and may also result in multiple new A-MPR regions. Still, the update should be done to ensure PC2 transmits at least as much power as PC3. Given that these errors have been in the specification since rel-17, consideration needs to be given also on implementation status.

Our preference would be to align these fixes with the 2Tx allowance update, to avoid multiple changes to the A-MPR in consecutive meetings.

Proposal 3: Correct NS_48 and NS_49 PC2 A-MPR errors which currently allow PC2 UE to transmit less power than PC3 UE and address the 2Tx allowance at the same time.

# 3	Conclusion

In this contribution n26 PC2 A-MPR was discussed and it was proposed to adopt the A-MPR as shown in the tables in this document. The tables are not repeated here.

n26 PC2:

Proposal 1: Adopt A-MPR for n26 PC2 as proposed in tables for NS_12 to NS_15 (see clause 2 for the tables).

Proposal 2: This WI and WI for 7 MHz CBW need to coordinate to ensure combination of having PC2 requirements for other bandwidths and only PC3 requirements for 7 MHz does not happen.

n1 NS_48 and NS_49

Observation 1: NS_48 and NS_49 PC2 A-MPR has many instances where PC2 is allowed to transmit less power than PC3.

Proposal 3: Correct NS_48 and NS_49 PC2 A-MPR errors which currently allow PC2 UE to transmit less power than PC3 UE and address the 2Tx allowance at the same time.