3GPP TSG-RAN WG4 Meeting #115	R4-2507763

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		7.10.3.2.1

Source: 		Qualcomm Inc.

Title:	 	NR NTN HPUE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved. One of the objectives is high power UE operation for NTN. In this contribution UE RF Tx requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

2.1 SAR solution

Fundamentally, SAR can be controlled in the UE by either controlling transmission time or transmission power. Only UE has knowledge of the time and power = energy budget available to meet SAR, as network is not aware of e.g. usage of other radios like WiFi or Bluetooth. The remaining energy is also not constant, as both cellular radio and other radio usage is dynamic over time. Ideally, network would need regularly informed on the available energy headroom to be able to optimize scheduling so that user experience is the least impacted.

Observation 1: Only UE has knowledge of available energy budget to meet SAR.

Observation 2: Energy budget is dynamic and therefore reporting should also be dynamic.

This target can be met by reporting the energy headroom to the network when UE start running close to no available headroom. As SAR measurement window can be very long, unnecessary signaling can be reduced e.g. if there has been a long time with no UL transmissions.

It is expected that in NTN use case most transmissions will be done with high UE output power. Therefore, it is reasonable to use maximum transmit power as a reference for remaining energy. We propose that UE reports to the network that it can transmit X seconds out of next Y seconds with full power. This has the following benefits

- Network has full information on the remaining energy, not only on power or time dimension of it

- Network can scale the time to match the used transmit power, as network also gets PHR

- Network can choose whether to use the budget more quickly or to reduce duty cycle to enable high power transmission over longer time span.

An alternative but equivalent option is to report that UE can transmit at X% duty cycle for next Y seconds.

Proposal 1: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

A reasonable granularity is required for values X and Y. In our view a starting point would be a range from 100 ms to 5s, with 100 ms granularity. Also to share the same understanding on what are the next Y seconds, the counting of the time can be based on the time instant when the report is sent.

Proposal 2: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 3: Starting for the indicated values is the transmission time of the report.

Finally, RAN2 help is needed to realize the reporting. Therefore, an LS should be sent to RAN2 to specify this energy headroom report.

Proposal 4: Send an LS to RAN2 to specify energy headroom reporting.

2.2 Component power hand capability

In addition to maximizing the possibility for high power UE transmissions, also UE RF component feasibility needs to be considered. Specifically, RF filters and their power handling capabilities for PC2 and PC1.5 are designed with 50% duty cycle in mind. There may be instances where UE needs to protect it’s components. To enable re-use of same filter technologies, it would be beneficial not to mandate UE to follow higher than 50% duty cycle for PC2. Unlike the TN solution, this should be optional: UEs with filters with higher power handling capability can transmit longer times.

Proposal 5: For NR NTN PC2 and PC1.5, and 4Tx PC1, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded.

1 Tx PC1 needs further discussion.

2.3 ACLR requirements and MPR for PC1

For PC1, as the difference between TN ACLR and simulation results is much larger. We observed that output power capability for same PA model to reach 31 or 37 dBc ACLR changed almost 2 dB. That is, the components dimensioning would need to be different to sustain same output power with improved ACLR. In case 4x PC2 PAs would be used for PC1 design, MPR would need to increased to meet requirements.

Observation 3: ACLR choice between 31 and 37 dBc has almost 2 dB impact on output power capability of a PA model.

Also, TN PC1 usage has not become widespread, and it is not clear if there are economies of scale to take advantage of. Therefore, it would be useful to relax the PC1 requirements to make implementation possible with 4 PC2 chains and increase the likelihood that designs will appear.

Observation 4: TN PC1 designs are not common.

In RAN4#114bis, -34 dBc was tentatively agreed as the ACLR limit. The ACLR limit has impact on MPR, but how that is observed depends a lot on how the MPR evaluation is done. Typically RAN4 calibrates PA to meet the ACLR with 1 dB MPR with 100 RB DFT-s-OFDM QPSK waveform. If the same calibration is done always the MPR will look quite the same, but the maximum output power for 0 dB MPR will be different. Typically this means there is a constant change in gain, i.e. the PA non-linearity is just captured at the calibration point without caring too much on what is the real output power. Still the maximum output power of the power class stays the same, so this approach in a way defines how to real component needs to be dimensioned.

Another approach to build this from other direction, starting with available components and check that which output power they can reach. In this case we did evaluations for PC1 using a PC2 PA model calibrated to meet 31 dB ACLR with 25 dBm output power. Usage in PC1 was modelled by applying 5 dB more stringent SEM mask. Evaluations were done with 10 MHz CBW. ACLR limits of 31, 34 and 37 dBc were evaluated.

Examples of the results are shown in Figures 1 to 3.

Figure 1: Limiting requirement and MPR for 10 MHz DFTs-QPSK with 37 dBc ACLR limit

Figure 2: Limiting requirement and MPR for 10 MHz DFTs-QPSK with 34 dBc ACLR limit

Figure 3: Limiting requirement and MPR for 10 MHz DFTs-QPSK with 31 dBc ACLR limit

It can be seen that MPR values decrease when ACLR limit changes from 37 dBc to 34 dBc, but MPR does not get smaller anymore when ACLR limit is changed from 34 dBc to 31 dBc. This is explained by allocations becoming SEM limited and ACLR relaxation does not help any longer. The MPR decrease is approximately 0.9 dB for when ACLR is relaxed from 37 to 34 dBc, and only about 0.2 from 34 dBc to 31 dBc.

The set of RB allocations that benefit are only ACLR limited allocations. For narrower channel bandwidths there is less benefit, as larger percentage of allocations are SEM limited.

Observation 5: For 10 MHz and narrower channel bandwidth, ACLR relaxation from 34 to 31 dBc has a very limited benefit on MPR.

37 dBc ACLR is not a good choice for NR NTN, as co-existence evaluations showed that lower ACLR is sufficient, and ACLR start to limit the achievable output power for much larger set of RB allocations. As such, we propose to confirm 34 dBc to be used as PC1 ACLR limit

Proposal 6: Confirm NR NTN UE ACLR as 34 dBc for PC1 NR NTN PC1.

For edge allocations, the MPR is mainly due to SEM, and emissions are a combination of PA non-linearity and linear leakage from baseband. With PC1, it is expected that there would be approximately 2 dB increase for MPR to to meet SEM with 2 dB higher nominal power class compared to PC1.5.

The figures above do not capture the worst case, which happens with narrowest guard between edge RB and SEM. 5..8 RB allocation width at channel edge was still rather challenging and could require more power backoff than other outer allocations. To avoid increase in MPR for all outer allocations, it is beneficial to increase the RB allocation size for edge allocations for PC1

Proposal 7: L_CRB considered for edge allocations for PC1 is updated to [8] RB.

Based on the evaluations made so far, 4 Tx PC1.5 MPR table can be taken as starting point for PC1 MPR. Below the edge RB MPR is further increased by 2 dB compared to 4 Tx PC1.5 to account for the total power increase.

Proposal 8: Adopt the table below as starting point for PC1 MPR definition

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.5] | ≤ [2.0] | ≤ [0.5] |
|  | QPSK | ≤ [10.0] | ≤ [2.5] | ≤ [0.5] |
|  | 16 QAM | ≤ [10.0] | ≤ [3.5] | ≤ [1.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [4.0] | ≤ [3.5] |
| CP-OFDM | QPSK | ≤ [10.0] | ≤ [4.5] | ≤ [2.0] |
|  | 16 QAM | ≤ [10.0] | ≤ [4.5] | ≤ [2.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [5.0] | ≤ [5.0] |

Further evaluations are still needed on the values.

2.4 PC2 A-MPR

NS_02N

For n255 no A-MPR is specified for PC3 based on more than 10 dB filter rejection. Since then, it has been discussed that multiple L-bands could be supported with a single filter, which extends the n255 filter passband, and can reduce the rejection towards GNSS. In one filter only 5 dB rejection in was guaranteed over extreme conditions. The need for A-MPR depends on the filter rejection. With only 5 dB rejection some A-MPR is needed. To make evaluations with same assumptions, RAN4 should agree a filter rejection assumption for GNSS frequencies considering co-banding of multiple L-bands.

Proposal 9: RAN4 to agree filter rejection assumption for GNSS frequencies 1559 – 1610 MHz considering co-banding of multiple L-bands

NS_24

Initial simulation data is provided for NS_24 in Figures 4 to Figure 8 for 10 to 20 MHz channel bandwidths

This simulation data consider’s only PA non-linearity and does not yet include the composite impact of transceiver wideband noise, phase noise and PA noise floor. Only A-MPR is shown in the Figures.

Figure 4: DFT-s-OFDM QPSK, 20 MHz, Fc = 1995 MHz

Figure 5: DFT-s-OFDM QPSK, 20 MHz, Fc = 1990 MHz

Figure 6: DFT-s-OFDM QPSK, 15 MHz, Fc = 1997.5 MHz

Figure 7: DFT-s-OFDM QPSK, 15 MHz, Fc = 1987.5 MHz

Figure 8: DFT-s-OFDM QPSK, 10 MHz, Fc = 2000 MHz

The data is initial at this point of time and it is difficult to make solid conclusions on the required A-MPR before further measurement checks are made the fully confirm the PA model still performs accurately under such high backoffs. When PC3 A-MPR for NS_24 was defined, there were multiple instances where measurements showed larger A-MPR than simulations and also significant different between different PA models. [2]

## 3	Conclusion

In this contribution UE RF Tx requirements for NR NTN were discussed. Following observations and proposals were made.

2.1 SAR solution

Observation 1: Only UE has knowledge of available energy budget to meet SAR.

Observation 2: Energy budget is dynamic and therefore reporting should also be dynamic.

Proposal 1: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

Proposal 2: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 3: Starting for the indicated values is the transmission time of the report.

Proposal 4: Send an LS to RAN2 to specify energy headroom reporting.

2.2 Component power hand capability

Proposal 5: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded.

2.3 ACLR requirements and MPR for PC1

Observation 3: ACLR choice between 31 and 37 dBc has almost 2 dB impact on output power capability of a PA model.

Observation 4: TN PC1 designs are not common.

Observation 5: For 10 MHz and narrower channel bandwidth, ACLR relaxation from 34 to 31 dBc has a very limited benefit on MPR.

Proposal 6: Confirm NR NTN UE ACLR as 34 dBc for PC1 NR NTN PC1.

Proposal 7: L_CRB considered for edge allocations for PC1 is updated to [8] RB.

Proposal 8: Adopt the table below as starting point for PC1 MPR definition

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.5] | ≤ [2.0] | ≤ [0.5] |
|  | QPSK | ≤ [10.0] | ≤ [2.5] | ≤ [0.5] |
|  | 16 QAM | ≤ [10.0] | ≤ [3.5] | ≤ [1.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [4.0] | ≤ [3.5] |
| CP-OFDM | QPSK | ≤ [10.0] | ≤ [4.5] | ≤ [2.0] |
|  | 16 QAM | ≤ [10.0] | ≤ [4.5] | ≤ [2.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [5.0] | ≤ [5.0] |

Further evaluations are still needed on the values.

2.4 PC2 A-MPR

Proposal 9: RAN4 to agree filter rejection assumption for GNSS frequencies 1559 – 1610 MHz considering co-banding of multiple L-bands

## 4	References

- R4-2503006, WF on HPUE UE requirement for IoT-NTN, Vivo

- R4-1812118, AMPR for n65, Qualcomm Incorporated