3GPP TSG-RAN WG4 Meeting #115	R4-2507764

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		7.10.3.3.1

Source: 		Qualcomm Inc.

Title: 		IoT NTN high power UE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN is on-going. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

NB-IoT PC2 RF requirements

To evaluate PC2 MPR, we used a simulation model of a PC2 PA, which was pushed to the maximum output power of the model with 12-tone signal to set a MPR=0 reference point. This point was mapped to correspond to 26 dBm at antenna. We applied PC3 MPR values to this reference point, and verified that NB-IoT mask and UTRA ACLR requirements are met. The waveforms contained DC-leakage and IQ-image at 25 dBc level, and CIM3 was set to 60 dBc.

We only used both 3.75 kHz and 15 kHz SCS.

The evaluation was repeated using a PC3 NR PA. It should be noted that the component ruggedness is not specified for PC2 power levels. Still, with NB-IoT signals the emission levels met the requirements with PC3 MPR applied.

Observation 1: General requirements (SEM, spurious, ACLR) for NB-IoT can be met with PC3 MPR.

Based on this evaluation, we propose to use PC3 MPR also for PC2 at least for 15 kHz SCS.

Proposal 1: PC3 MPR values are re-used for PC2 NB-IoT for all form factors.

NB-IoT PC2 A-MPR

There are a number of additional requirements to consider, shown in the table below.

For NS_04N and NS_05N, PC3 A-MPR has not been concluded. PC3 baseline would be needed before evaluations for NS_04N and NS_05N.

Observation 2: PC3 A-MPR is not complete for NS_04N and NS_05N.

Proposal 2: Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N

For NS_03N there are two requirements, FCC emission mask and GNSS protection. The FCC mask is relative to output power and requirements apply outside authorized bandwidth which is wider than the NB-IoT channel bandwidth. No A-MPR is needed to meet the FCC mask.

The GNSS protection requirements start immediately from the band edge, ramping down to -40 dBm/MHz at 5 MHz offset from band edge. The PA non-linearities visible in simulations are below this level, however we have not completely evaluated the wideband noise levels from phase noise, transceiver noise and PA noise floor, for which RF filtering does not help at this close frequency offset. Based on initial evaluation, no A-MPR is needed, but this needs further confirmation.

Situation is similar with NS_02N, which has similar GNSS protection requirement in addition to FCC mask.

Proposal 3: As as starting point, no A-MPR is needed for NS_03N and NS_02N for NB-IoT PC2

Finally for NS_24 the requirement is -50 dBm/MHz at 5 MHz offset. A-MPR is specified already for PC3. Here the issue with wideband noise sources is much tougher than with NS_02N and NS_03N. In initial evaluations we observed the need for 2 dB additional A-MPR.

Proposal 4: As a starting point, define 2 dB A-MPR increase for NS_24 to support NB-IoT PC2.

Proposal 3 and 4 are made as a starting point as the wideband noise floor needs verification and is not reliable found out from typical RAN4 simulations.

NB-IoT PC2 power boosting

Power boosting to increase output power by [1] dB has been proposed for NB-IoT. Especially if a PC2 NR PA is used, it is feasible to reach 27 dBm output power, ignoring ruggedness of the current components. Last meeting detailed two options, either change in upper power tolerance or increase in nominal power. Defining a new nominal power would be similar to defining a new power class and comes with more specification impacts. Both options are feasible.

Observation 3: Both options for NB-IoT PC2 power boosting are feasible.

For upper power tolerance, it is also important to note that adding a tolerance in power class definition is not sufficient, but the tolerance needs to be also applied to configured maximum output power, for which a separate Pcmax tolerance is defined:

In the equation above, Pcmax_H,c does not include power class tolerance.

Currently TS 36.102 refers to TS 36.101 for configured max output power. However, 36.101 only covers up to PC3 for NB-IoT. Therefore, it would make sense to explicitly write configured maximum output power requirements to TS 36.102, and include also PCmax tolerance for PC2.

Proposal 5: Increase NB-IoT PC2 upper power tolerance by [1] dB.

Proposal 6: Pcmax tolerance needs to be correspondingly updated to enable power boosting.

Proposal 7: With introduction of NB-IoT PC2 requirements, capture configured maximum output power requirements fully in TS 36.102 and no longer reference 36.101

NB-IoT PC1 requirements for handheld and non-handheld devices

For PC1, using -25 dBc IQ-image, there is a risk of needing high MPR/A-MPR to meet general requirements like NB-IoT mask. Therefore, for PC1 we propose to assume in simulation -[34] dBc IQ-image and LO-leakage instead of -25 dBc. Design striving towards PC1 power level could be assumed to be slightly better than lower power classes.

Proposal 8: For NB-IoT PC1, for MPR and A-MPR evaluations assume -[34] dBc IQ-image and LO-leakage and 60 dBc CIM3. Discuss minimum requirement separately.

We made initial evaluations via simulations to NB-IoT PC1. Further measurement verification is still needed. In these simulation-based evaluations we assumed -34 dBc IQ-image, -25 dBc DC-leakage and 60 dBc CIM3. The evaluation framework was simplified to still use PC2 PA model with 31 dBm at antenna. The PA model was pushed to the point where it barely converged to reach 31 dBm at antenna for a single tone transmission with reasonable post-PA loss.

In these initial evaluations, it was observed that additional MPR to support PC1 operation is needed. These evaluations were simplified and based only on simulations, and further measurement confirmation is needed.

The summary of the PC1 MPR is captured in Table 1.

Table 1: Initial proposal for PC1 MPR for NB-IoT, 15 kHz SCS, for all form factors

| Tone positions for 1 Tone allocation | 0-2 | 3-8 |  | 9-11 |
| --- | --- | --- | --- | --- |
| MPR | ≤ [1] dB | 0 dB |  | ≤ [1] dB |
| Tone positions for 3 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [1.5] dB | 0 dB |  | ≤ [1.5] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2] dB |  | ≤ [2] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [4] dB |  |  |  |

Proposal 9: Take NB-IoT PC1 MPR in Table 1 as a starting point for all form factors for 15 kHz SCS. Further evaluate MPR including also 3.75 kHz SCS.

Not having final agreement for MPR should not delay A-MPR evaluations. It is possible to evaluate the total backoff need for additional requirements. The required A-MPR will simply be total backoff – MPR, and this is simple to calculate once the MPR values are solidified.

Proposal 10: For additional emission requirements evaluate total needed power backoff, and trade-off between the MPR and A-MPR so that MPR + A-MPR meets the total backoff need.

Component power handling capability

In previous meeting P-MPR was discussed to be agreed to be added to configured maximum output power. Generally, filter for higher power classes have been designed with 50% duty cycle in mind. There may be some occasions when the UE may need to protect it’s components. Even though NB-IoT is HD-FDD, RAN1 specifications allow up to 256ms contiguous UL transmission, after which UE has to stop to receive DL.

Observation 4: Up to 256ms contiguous UL transmission is possible for NB-IoT.

There may also be UEs with filters which can tolerate high output power. To enable re-use of same filter technologies, it would be beneficial not to mandate UE to follow higher than 50% duty cycle for PC2. Unlike the TN solution for NR, this should be optional.

Further discussion in needed for PC1.

Proposal 11: Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2. Capture this allowance into specification.

Proposal 12: Further evaluate potential specification impact from filter power handling for PC1.

SAR

SAR discussion from NR NTN HPUE contribution is re-produced here as it has been usually handled in NTN IoT discussion thread:

Fundamentally, SAR can be controlled in the UE by either controlling transmission time or transmission power. Only UE has knowledge of the time and power = energy budget available to meet SAR, as network is not aware of e.g. usage of other radios like WiFi or Bluetooth. The remaining energy is also not constant, as both cellular radio and other radio usage is dynamic over time. Ideally, network would need regularly informed on the available energy headroom to be able to optimize scheduling so that user experience is the least impacted.

Observation 5: Only UE has knowledge of available energy budget to meet SAR.

Observation 6: Energy budget is dynamic and therefore reporting should also be dynamic.

This target can be met by reporting the energy headroom to the network when UE start running close to no available headroom. As SAR measurement window can be very long, unnecessary signaling can be reduced e.g. if there has been a long time with no UL transmissions.

It is expected that in NTN use case most transmissions will be done with high UE output power. Therefore, it is reasonable to use maximum transmit power as a reference for remaining energy. We propose that UE reports to the network that it can transmit X seconds out of next Y seconds with full power. This has the following benefits

- Network has full information on the remaining energy, not only on power or time dimension of it

- Network can scale the time to match the used transmit power, as network also gets PHR

- Network can choose whether to use the budget more quickly or to reduce duty cycle to enable high power transmission over longer time span.

An alternative but equivalent option is to report that UE can transmit at X% duty cycle for next Y seconds.

Proposal 13: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

A reasonable granularity is required for values X and Y. In our view a starting point would be a range from 100 ms to 5s, with 100 ms granularity. Also to share the same understanding on what are the next Y seconds, the counting of the time can be based on the time instant when the report is sent.

Proposal 14: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 15: Starting for the indicated values is the transmission time of the report.

Finally, RAN2 help is needed to realize the reporting. Therefore, an LS should be sent to RAN2 to specify this energy headroom report.

Proposal 16: Send an LS to RAN2 to specify energy headroom reporting.

## 3	Conclusion

In this IoT NTN HPUE operation was discussed. Following observations and proposals were made:

NB-IoT PC2

Observation 1: General requirements (SEM, spurious, ACLR) for NB-IoT can be met with PC3 MPR.

Proposal 1: PC3 MPR values are re-used for PC2 NB-IoT for all form factors.

Observation 2: PC3 A-MPR is not complete for NS_04N and NS_05N.

Proposal 2: Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N

Proposal 3: As as starting point, no A-MPR is needed for NS_03N and NS_02N for NB-IoT PC2

Proposal 4: As a starting point, define 2 dB A-MPR increase for NS_24 to support NB-IoT PC2.

Observation 3: Both options for NB-IoT PC2 power boosting are feasible.

Proposal 5: Increase NB-IoT PC2 upper power tolerance by [1] dB.

Proposal 6: Pcmax tolerance needs to be correspondingly updated to enable power boosting.

Proposal 7: With introduction of NB-IoT PC2 requirements, capture configured maximum output power requirements fully in TS 36.102 and no longer reference 36.101

NB-IoT PC1

Proposal 8: For NB-IoT PC1, for MPR and A-MPR evaluations assume [-34] dBc IQ-image and LO-leakage and 60 dBc CIM3. Discuss minimum requirement separately.

Proposal 9: Take NB-IoT PC1 MPR in Table 1 as a starting point for all form factors for 15 kHz SCS. Further evaluate MPR including also 3.75 kHz SCS.

Table 1: Initial proposal for PC1 MPR for NB-IoT, 15 kHz SCS, for all form factors

| Tone positions for 1 Tone allocation | 0-2 | 3-8 |  | 9-11 |
| --- | --- | --- | --- | --- |
| MPR | ≤ [1] dB | 0 dB |  | ≤ [1] dB |
| Tone positions for 3 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [1.5] dB | 0 dB |  | ≤ [1.5] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2] dB |  | ≤ [2] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [4] dB |  |  |  |

Proposal 10: For additional emission requirements evaluate total needed power backoff, and trade-off between the MPR and A-MPR so that MPR + A-MPR meets the total backoff need.

Component power handling capability

Observation 4: Up to 256ms contiguous UL transmission is possible for NB-IoT.

Proposal 11: Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2. Capture this allowance into specification.

Proposal 12: Further evaluate potential specification impact from filter power handling for PC1.

SAR

Note: SAR proposals are exactly the same as in our NR NTN HPUE contribution but included also here as discussion previously takes place in IoT NTN thread.

Observation 5: Only UE has knowledge of available energy budget to meet SAR.

Observation 6: Energy budget is dynamic and therefore reporting should also be dynamic.

Proposal 13: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

Proposal 14: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 15: Starting for the indicated values is the transmission time of the report.

Proposal 16: Send an LS to RAN2 to specify energy headroom reporting.

## 4	References