3GPP TSG-RAN WG4 Meeting RAN4#114bis      				R4-2507772 St. Julian’s, Malta

May 19 - 23, 2025

Agenda Item:	7.1.2.3.4

Source:	Lenovo

Title:	SRS Power Imbalance Handling and PHR Reporting

Document for:  Approval

- Introduction

In previous meetings, some contributions have provided simulation results showing that SRS insertion loss imbalances have little to no impact on system performance and throughput, while other contributions have provided simulation results showing there is a significant impact.  The fact that results differ is perhaps not surprising given that simulation assumptions may not be exactly aligned, and that channel estimation algorithms, scheduling algorithms, and traffic models may all be different.

However, from a theoretical perspective, it is well understood that the scheduler must have an accurate estimate of the channel to optimally choose the MCS for the downlink transmission.  As a result, the SRS insertion losses should either be compensated or known to the gNB. Thus, if the required complexity is not too great, it makes sense to enable the gNB to estimate the uplink channel more accurately in the presence of the SRS power imbalances.

In this contribution, we demonstrate how the signalling overhead associated with indicating the SRS gain imbalances can be minimized.

- Expected UE Behavior for SRS Insertion Losses

The power control equation for SRS from [1] is given by

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i (for SRS resource set qs)

[dBm] .

It can be observed that the power control equation does not depend on the allowed SRS insertion loss ∆TRxSRS other than through the configured maximum power PCMAX,f,c(i).  As a result, it is apparent that at power levels below the maximum configured power for the port PCMAX,f,c(i), the insertion loss ∆TRxSRS should not have any impact on the output power of the SRS port.

Observation 1:	 Since the power control equation for the SRS port does not allow transmit power relaxations below the configured maximum power PCMAX,f,c(i), the UE should compensate the SRS transmit power for the insertion loss ∆TRxSRS at power levels below PCMAX,f,c(i).

Observation 2:	 At some future point in time, it would be beneficial to define a requirement to ensure that the UE compensates for the SRS insertion losses at power levels below PCMAX,f,c(i).

- Handling SRS Insertion Losses with Minimum Signalling

In order to assist the gNB in correctly estimating the downlink channel from the received SRS transmission, the UE should indicate it’s behavior.

Proposal 1:	In order to assist the gNB in correctly estimating the downlink channel from the received uplink SRS transmissions, the UE should indicate whether or not it UE compensates for the SRS insertion losses at power levels below PCMAX,f,c.

Case 1:  The UE signals it does not compensate SRS insertion losses

If the UE does not compensate the SRS insertion losses, then the SRS transmit power as a function of power control is as illustrated in Figure 1. In this figure, Port 1 has the smallest insertion loss, and Port 3 has the largest insertion loss. In Figure 1, the power difference between the SRS ports is fixed and is independent of the power control setting.

Figure 1:  SRS Power Control Behavior without Insertion Loss Compensation

The power difference between the SRS ports is equal to the actual difference in the insertion losses of the different SRS ports.  Let ∆TRxSRS,iA denote the actual insertion loss for the i-th SRS port.  Then all that is needed for the gNB to correctly scale the SRS-based channel measurements are these actual insertion losses.  It can be noted that the signalling overhead can be slightly reduced by signalling the insertion loss difference between each SRS port and a reference SRS port so that it is not necessary to signal the insertion loss of the reference port.  Here we assign index 1 to the SRS port having the smallest insertion loss and signal the insertion loss differences between the remaining SRS ports and this reference port.

Proposal 2:	The UE should signal the actual insertion loss differences between the SRS port with the least insertion loss and the remaining SRS ports.  That is, if there are N SRS ports and port 1 is defined as the SRS port with the smallest insertion loss, then the UE should signal

δ1,j=∆TRxSRS,jA-∆TRxSRS,1A  for  2≤j≤N .

With these values, the gNB can correctly scale the SRS-based channel estimates to compensate for the power differences between the reference symbols on the different ports.  The UE would only need to signal these insertion loss differences initially and when the SRS port mappings change.

Case 2:  The UE signals it compensates SRS insertion losses

If the UE compensates the SRS insertion losses, then the SRS transmit power as a function of power control is as illustrated in Figure 2. As in the previous figure, Port 1 has the smallest insertion loss, and Port 3 has the largest insertion loss. In Figure 2, it can be observed that the transmit power of the SRS ports is equal until the power control exceeds PCMAX,f,c for the SRS port with the largest SRS insertion loss.  In this example, SRS port 3 has the largest insertion loss and SRS port 1 has the smallest insertion loss.

Figure 2:  SRS Power Control Behavior with Insertion Loss Compensation

Observation 3:	 If the UE compensates SRS insertion losses, then the transmit power for all of the SRS ports is equal unless the transmit power for the first antenna port pt,1 exceeds PCMAX,f,c(k) for SRS port k for which the insertion loss ∆TRxSRS,kA is largest.

Observation 4:	 If the UE compensates SRS insertion losses, the power transmitted on all of the SRS ports is equal whenever the transmit power of the first antenna port pt,1 satisfies

pt,1≤miniPCMAX,f,ci,

and in this case, no PHR report is needed to correct the SRS-based channel estimates.

We now consider what signalling is needed in order for the gNB to correct the SRS-based channel estimates in the case that

pt,1>miniPCMAX,f,ci

As above, the SRS ports are numbered such that SRS port 1 has the smallest insertion loss so that

PCMAX,f,c1≥miniPCMAX,f,ci .

The maximum configured power for any other port j can be expressed in terms of PCMAX,f,c1 as

PCMAX,f,cj=PCMAX,f,c1-δ1,j ,

so long as the values δ1,j  2≤j≤N have been signalled from the UE to the gNB (as in Proposal 2 for Case 1 above)

The transmit power for the j-th port pt,j is given by

pt,j=pt,1pt,1≤PCMAX,f,cjPCMAX,f,cjpt,1>PCMAX,f,cj ,

which is equivalent to

pt,j=pt,1pt,1≤PCMAX,f,c1-δ1,jPCMAX,f,c1-δ1,jpt,1>PCMAX,f,c1-δ1,j .			(1)

The power headroom report for the first SRS port gives the power headroom pH,1 defined as

pH,1= PCMAX,f,c (1) - pt,1

where PCMAX,f,c (1) is the configured maximum power on the first antenna port and pt,1 is the transmitted power on the first antenna port.  Optionally, the power headroom can also be configured to include the configured maximum for the first antenna port PCMAX,f,c (1).

From the power headroom pt,1 and the configured maximum power PCMAX,f,c (1) for the first antenna port, the gNB can determine the transmitted power for the first antenna port as

pt,1= PCMAX,f,c (1) - pH,1 ,

and the configured maximum power for the j-th antenna port can be determined as

PCMAX,f,cj=PCMAX,f,c1-δ1,j .				(2)

where the values δ1,j  2≤j≤N have previously been signalled from the UE to the gNB.

Using (1), it can now be observed that the gNB can determine the transmit power for any SRS port using the only the power headroom report for the first SRS port, so long as the first SRS port is defined as the SRS port having the minimum insertion loss and the power headroom report is configured to include the configured maximum power.

Previously (Observation 4), it was observed that if the UE compensates insertion losses, then the power will be equal for all SRS ports so long as transmit power of the first antenna port pt,1 satisfies

pt,1≤miniPCMAX,f,ci .

Using (2), this condition can also be expressed as

pt,1≤PCMAX,f,c1-maxiδ1,j .	            		(3)

From (3), we have the following two observations:

Observation 5:	If the UE compensates SRS insertion losses, and if the power transmitted on SRS port 1 pt,1 satisfies

pt,1≤miniPCMAX,f,ci,

then the transmit power on all of the SRS ports is equal and no PHR report is needed by the gNB to correct the SRS-based channel estimates.

Observation 6:	If the UE compensates SRS insertion losses, and if the power transmitted on SRS port 1 pt,1 satisfies

pt,1>miniPCMAX,f,ci,

the transmit power on at least one SRS ports will be less than the transmit power on the first SRS port.  In this case, the transmit power for an arbitrary SRS port j can be determined using a PHR report for port 1 that includes the optional field PCMAX,f,c1 as

pt,j=pt,1pt,1≤PCMAX,f,c1-δ1,jPCMAX,f,c1-δ1,jpt,1>PCMAX,f,c1-δ1,j .

Based on the above observations, we have the following additional proposal for minimizing the signalling overhead needed to enable the gNB to correct SRS-based channel estimates for the allowed insertion losses

Proposal 3:	If the UE compensates SRS insertion losses, and if the power transmitted on SRS port 1 pt,1 satisfies

pt,1>miniPCMAX,f,ci ,

the transmit power on at least one SRS ports will be less than the transmit power on the first SRS port.  In this case, the UE sends a PHR report only for port 1 that includes the optional field configuredMaxTxPower. From this transmit power for an arbitrary SRS port j can be determined as

pt,j=pt,1pt,1≤PCMAX,f,c1-δ1,jPCMAX,f,c1-δ1,jpt,1>PCMAX,f,c1-δ1,j .

otherwise, if

pt,1≤miniPCMAX,f,ci ,

the power transmitted on the SRS ports is equal, and no power headroom report is needed.

#### Summary

From a theoretical perspective, it is well understood that the gNB scheduler must have an accurate estimate of the channel to optimally choose the MCS for the downlink transmission.  As a result, the SRS insertion losses should either be compensated or known to the gNB. In our view, if the required complexity is not too great, it makes sense to enable the gNB to estimate the uplink channel more accurately in the presence of the SRS power imbalances.

In this contribution, we have discussed how the signalling overhead associated with indicating the SRS gain imbalances can be minimized.

Based on the above discussion, we have the following proposals:

Proposal 1:	In order to assist the gNB in correctly estimating the downlink channel from the received uplink SRS transmissions, the UE should indicate whether or not it UE compensates for the SRS insertion losses at power levels below PCMAX,f,c.

Proposal 2:	The UE should signal the actual insertion loss differences between the SRS port with the least insertion loss and the remaining SRS ports.  That is, if there are N SRS ports and port 1 is defined as the SRS port with the smallest insertion loss, then the UE should signal

δ1,j=∆TRxSRS,jA-∆TRxSRS,1A  for  2≤j≤N .

Proposal 3:	If the UE compensates SRS insertion losses, and if the power transmitted on SRS port 1 pt,1 satisfies

pt,1>miniPCMAX,f,ci ,

the transmit power on at least one SRS ports will be less than the transmit power on the first SRS port.  In this case, the UE sends a PHR report only for port 1 that includes the optional field configuredMaxTxPower. From this transmit power for an arbitrary SRS port j can be determined as

pt,j=pt,1pt,1≤PCMAX,f,c1-δ1,jPCMAX,f,c1-δ1,jpt,1>PCMAX,f,c1-δ1,j .

otherwise, if

pt,1≤miniPCMAX,f,ci ,

the power transmitted on the SRS ports is equal, and no power headroom report is needed.

# References

- 3GPP TS 38.213, “NR; Physical layer procedures for control.”

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2504562, “SRS Power Imbalance Handling,” Lenovo, RAN4#114bis.

- R4-2413306, “SRS Imbalance Determination with Reduced PHR Overhead,” Lenovo, RAN4#112.

- R4-2409736, “SRS Power Imbalance Determination,” Lenovo, Motorola Mobility, RAN4#111.