3GPP RAN WG4 Meeting #115																R4-2507774

Malta, Malta, 19th – 23rd May 2025

Agenda item:	7.10.3.2.2

Source:	Apple

Title:	On RSD requirements for non-handheld devices

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN.

During the previous meetings RAN WG4 agreed to introduce PC1.5 and PC1 power classes for the non-handheld devices, whereupon one of the open issues was the RSD requirement for them. In this discussion paper we present our further considerations on this matter.

# 2	RSD requirements for non-handheld devices

It bears mentioning that 3GPP had only one power class defined for NTN, PC3, which assumes handheld devices. It was agreed to introduce PC2 power class for handheld devices and PC1.5/PC1 power class for the non-handheld devices. During the previous RAN WG4 meeting the following agreements were made for PC1.5 and PC1 RSD.

Issue 1-10: General discussion of RSD analysis

Ad-hoc agreements:

- In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Issue 1-11: General discussion of RSD of PC1.5 and PC1

Ad-hoc agreements:

- Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE. Further check these in next meeting.

Our general understanding that non-handheld devices, e.g. PC1, use different RF components and hardware differentiating them from the handheld devices, for which various "degradations", such as RSD, must be defined. As an example, while we have PC2 RSD defined for both 1TX and 2TX architectures, there is no RSD defined for the PC1 devices operating in the same FDD bands as mobile phones. Thus, following the same logic, no RSD should be defined for the PC1.5/PC1 NTN non-handheld devices operating in the satellite FDD bands.

Proposal:	RSD is 0dB for the NTN non-handheld PC1.5 and PC1 power classes.

# 3	Conclusions

In this discussion paper we have presented our initial view on RSD for the NTN non-handheld power classes PC1.5 and PC1. Since one of the main differentiating factors between "handheld" and "non-handheld" devices is absence of various performance degradations for the latter device class, we suggest following the same principle also for the NTN non-handheld power classes and NTN FDD bands.

Proposal:	RSD is 0dB for the NTN non-handheld PC1.5 and PC1 power classes.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"