3GPP TSG-RAN WG4 Meeting #115									                        R4-2507783

St Julian’s, Malta, May 19 – 23, 2025

Agenda item:			7.24.3

Source:	Ericsson

Title:	On the Asymmetric DL sTRP/UL mTRP for NR MIMO Phase 5

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for the RAN1-led WI on “NR MIMO Phase 5” WI [1], where the objectives of the WI with the identified potential RAN4 UE RF impacts are the following:

RAN1:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution, we continue the discussion on the “Asymmetric DL sTRP/UL mTRP” working area.

# Discussion

In the previous meeting, the discussion on “3 Tx” topic has been finalized. For “Asymmetric DL sTRP/UL mTRP” topic, the following has been captured in the approved WF [2]:

## <Sub-topic 2-1> UL mTRP

<Agreement>: Introduce the beam correspondence requirement when the IE unifiedTCI-StateType is configured as ‘separate’ and different reference signals for QCL-TypeD are configured across the TCI states.

After a long discussion in RAN4, it was finally agreed to introduce the beam correspondence requirements for the scenario where the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

In the previous meeting, a draft CR was submitted [3] with the following text proposal:

## 6.6M Beam correspondence for [asymmetric UL/DL] For a UE that supports RAN1_sTRP_2TA_capability, and the QCL type D reference signals associated with the UL and DL TCI states are different, the requirements of clause 6.6 apply. Side-conditions in clause 6.6 apply to each of the TCI states.

According to the WID[1], the Rel-17/18 unified TCI framework is assumed, fully reusing the legacy QCL/UL spatial relation rules. In RAN1#117, the following was concluded:

Conclusion (RAN1#117) For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

From the previous, for the Rel-18 unified TCI framework for FR2, {one DL TCI state + up to two UL TCI states} can be applied. In the scenario where the UL-only TRP node can transmit the SSB, one UL TCI state is associated with the anchor TRP node (DL/UL TRP node) and the other UL TCI state is associated with the UL-only TRP node.

From the text proposal above, it is not clear which one out of two UL TCI states is meant by when stating: “…and the QCL type D reference signals associated with the UL and DL TCI states are different”.

Observation 1: In the scenario where the UL-only TRP node can transmit the SSB, one UL TCI state is associated with the anchor TRP node and the other UL TCI state is associated with the UL-only TRP node. The beam correspondence requirements should clearly differentiate those two UL TCI states.

In addition, for the legacy sTRP case where the same TRP is used for the DL and the UL, the (DL) TCI-state and TCI-UL-state can have the same SSB as QCL resource, but not necessarily. QCL-source for TCI-UL-state targeting UL-only TRP can be SRS and SSB. Hence, stating that “the UL and DL TCI states are different” does not necessarily imply that the DL and the UL TRP nodes are different.

Observation 2: Stating that “the UL and DL TCI states are different” does not necessarily imply that the DL and the UL TRP nodes are different, as the DL TCI state and the UL TCI states do not have to share the same SSB as QCL resource (QCL-source for TCI-UL-state targeting UL-only TRP can be also SRS).

Finally, the side conditions in clause 6.6 are based on both SSB and CSI-RS and cannot be directly reused, as the assumption in this WI is that the UL-only TRP node can only transmit the SSB (or nothing).

Observation 3: The side conditions in clause 6.6 are based on both SSB and CSI-RS and cannot be directly reused.

For the specification of this feature to be completed in RAN4, the testing aspects should also be agreed on. One of the dilemmas is how to configure 2AoA test set-up, which corresponds to spatially separated anchor DL/UL TRP node and the UL-only TRP node. In the scenario where the UL-only TRP node can transmit SSB, the UE is supposed to track two DL RS (the other one coming from the anchor node). Even though the reception is non-simultaneous, the separation angle between the two DL RS plays an important role for the proper testing of this feature. The UE is using one beam to receive the DL signal from the anchor node and another one to transmit towards the UL-only node. Also, a simplification to a single AoA test set-up by applying the separation angle of zero degrees would mean that both DL RSs are coming from the same direction, and the UE could apply the beam correspondence to the DL RS from the anchor TRP node to configure the beam for the transmission towards the UL-only node, which wouldn’t be correct. The test would have to ensure that the beam correspondence is performed with the DL RS from the UL-only TRP node.

Observation 4: 2AoA test set-up should be specified in order to properly test the asymmetric DL sTRP/UL mTRP feature, where the DL and the UL nodes are non-collocated in general.

Another big problem, especially for the intra-cell multi TRP case, is the question on how the UE can differentiate between the two DL RSs, i.e. how can it know if the DL RS is “coming” from the UL-only TRP node or not. Hence, the UE wouldn’t know to which TRP it should send the RACH.

Observation 5: For the intra-cell multi TRP case, another big problem is the question on how the UE can differentiate between the two DL RS, i.e. how can it know if the DL RS is “coming” from the UL-only TRP node or not.

Given the limited time remaining for this WI and some of the issues mentioned above, it seems more reasonable to postpone introducing this feature to Rel-20 unless the testing part is sorted out, in order to properly specify the RAN4 requirements for it.

Proposal: Unless 2AoA test-up could be sorted out, given the limited time remaining for this WI and some of the issues mentioned above, it seems more reasonable to postpone introducing this feature to Rel-20 in order to properly specify the RAN4 requirements for it.

# Conclusion

Based on the discussion in the previous sections we have made the following observations:

Observation 1: In the scenario where the UL-only TRP node can transmit the SSB, one UL TCI state is associated with the anchor TRP node and the other UL TCI state is associated with the UL-only TRP node. The beam correspondence requirements should clearly differentiate those two UL TCI states.

Observation 2: Stating that “the UL and DL TCI states are different” does not necessarily imply that the DL and the UL TRP nodes are different, as the DL TCI state and the UL TCI states do not have to share the same SSB as QCL resource (QCL-source for TCI-UL-state targeting UL-only TRP can be also SRS).

Observation 3: The side conditions in clause 6.6 are based on both SSB and CSI-RS and cannot be directly reused.

Observation 4: 2AoA test set-up should be specified in order to properly test the asymmetric DL sTRP/UL mTRP feature, where the DL and the UL nodes are non-collocated in general.

Observation 5: For the intra-cell multi TRP case, another big problem is the question on how the UE can differentiate between the two DL RS, i.e. how can it know if the DL RS is “coming” from the UL-only TRP node or not.

We have also made the following proposal:

Proposal: Unless 2AoA test-up could be sorted out, given the limited time remaining for this WI and some of the issues mentioned above, it seems more reasonable to postpone introducing this feature to Rel-20 in order to properly specify the RAN4 requirements for it.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105.

- R4-2505111, “WF on UE RF requirements for MIMO enhancement phase 5”, Samsung, RAN4#114-bis.

- R4-2503173, “dCR on RF requirements for a UE with different TCI states for UL and DL”, Qualcomm, Sony, Nokia, RAN4#114-bis.