3GPP TSG-RAN WG4 Meeting #115									                        R4-2507785

St Julian’s, Malta, May 19 – 23, 2025

Agenda item:			7.10.3.2.2

Source:	Ericsson

Title:	On the UE RF Rx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Rx requirements for NR-NTN HPUE from the approved WF in RAN4#114-bis [2].

# Discussion

In the approved WF [2], some of the open issues which are for further study are the following:

Issue 1-11: General discussion of RSD of PC1.5 and PC1

Ad-hoc agreements:

- Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE.

  - Further check these in next meeting.

Issue 1-14: RSD requirements for PC1.5 and PC1.

Way forward:

- Encourage companies to further discuss the following proposals:

  - Option 1: For PC1.5 2Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

  - Option 2: For PC1 1Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |
| n254 | [3] | [3] | [3] |  |

For TN, the RSD requirements are not specified for PC1.5 and PC1 [3]. FDD bands do not support PC1.5, while PC1 UEs are not targeted for smartphone form factor.

For the NTN, in RAN4#114 meeting it was agreed to define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19 (other cases are still FFS). Since only non-handheld UEs will be considered for PC1.5, even though 2Tx architecture will be assumed, in our view no RSD should be specified because for TN, no RSD was specified for PC1, for which the maximum output power is 2dB larger than for PC1.

For PC1, it was previously agreed only to consider 1Tx architecture and non-handheld UEs for NTN. For such case, no RSD should be specified which is in line with TN, where also 1Tx architecture was assumed and non-handheld UEs only.

Proposal 1: For the NR-NTN, do not to specify RSD requirements for PC1.5 and PC1.

Issue X: Proposal to allow power reduction beyond certain duty cycle

Way forward:

- Encourage companies to further discuss the following proposal:

  - Option 1: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded. (Qualcomm)

If the UE is not mandated to reduce its power when 50% Tx duty cycle in a radio frame is exceeded, in our view the chances that the SAR requirements would not be met will increase. If the UE is e.g. close to the human head and if the proposed relaxation is adopted, the UE would not be mandated to reduce its transmit power and thus would possibly fail the compliance to the SAR requirement. The purpose of 50% Tx duty cycle threshold for PC2 is to facilitate the compliance to SAR requirements.

The proposal in its current form is not agreeable also because for PC1.5, the UE shall be mandated to reduce its transmit power when 25% Tx duty cycle in a radio frame is exceeded. If the ratio is between 25% and 50% it shall fall back to PC2, while if the ratio is larger than 50% the UE shall fall back to PC3.

Proposal 2: For NR NTN PC2 and PC1.5, and IoT NTN PC2, to facilitate compliance to the SAR requirements keep the mandatory requirement for the UE to reduce its transmit power when 50% Tx duty cycle in a radio frame is exceeded (also when 25% Tx duty cycle is exceeded for PC1.5).

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: For the NR-NTN, do not to specify RSD requirements for PC1.5 and PC1.

Proposal 2: For NR NTN PC2 and PC1.5, and IoT NTN PC2, to facilitate compliance to the SAR requirements keep the mandatory requirement for the UE to reduce its transmit power when 50% Tx duty cycle in a radio frame is exceeded (also when 25% Tx duty cycle is exceeded for PC1.5).

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2505113, “WF on HPUE RF requirements for NR-NTN”, Samsung, RAN4#114-bis.

- 3GPP TS 38.101-1 V19.1.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.