3GPP TSG-RAN WG4 Meeting #115									                        R4-2507787

St Julian’s, Malta, May 19 – 23, 2025

Agenda item:			7.10.3.3.2

Source:	Ericsson

Title:	On the UE RF Rx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on the UE RF Rx requirements for IoT-NTN HPUE from the approved WF in RAN4#114-bis [2].

# Discussion

In the approved WF [2], the following has been captured:

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - Reference sensitivity values

    - 256:

      - 0 dB (MTK, Samsung, Ericsson)

      - 1.1dB (Qualcomm)

    - 255:

      - 0.8dB (MTK)

      - 1.2dB (Qualcomm)

  - No differentiation of handheld and non-handheld for RSD

- WF (Ad-hoc agreement):

  - Reference sensitivity values

    - 256: [0.5] dB

    - 255: 1 dB

Similar to NR-NTN, band 256 has a slightly larger duplexing distance than band n1 so at least the same duplexer isolation should be achievable for the two bands. In TS38.101-1 [3], 0dB RSD is specified for n1 for PC2 UE, so the same requirement could be reused for band 256 in our view.

Due to some concerns from other companies, [0.5] dB was proposed in the end as a compromise, which could be acceptable to us.

Proposal 1: For band 256, specify 0.5 dB RSD for PC2 IoT-NTN UEs. 1 dB RSD seems like a reasonable compromise for band 255.

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for 254/255/256 (oppo, Ericsson)

  - Option 2: 0dB for at least band 256 (Samsung)

  - Option 3: non-zero value as following: (MediaTek)

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | [3.7] |
| 255 | [4.7] |
| 254 | [3.7] |

- Recommended WF

  - TBD. Further explanation from proponents of option 3 needed

Since full duplex eMTC based IoT-NTN is considered only for non-handheld UEs with 1Tx, in our view 0dB RSD agreement should be kept for PC1 (for all bands).

Proposal 2: Specify 0 dB RSD for PC1 for full duplex eMTC based non-handheld UEs for IoT-NTN.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: For band 256, specify 0.5 dB RSD for PC2 IoT-NTN UEs. 1 dB RSD seems like a reasonable compromise for band 255.

Proposal 2: Specify 0dB RSD for PC1 for full duplex eMTC based non-handheld UEs for IoT-NTN.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2505114, “WF on HPUE RF requirements for IoT-NTN”, vivo, RAN4#114-bis.

- 3GPP TS 38.101-1 V19.1.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.