3GPP TSG-RAN WG4 Meeting #115	R4-2507814

Malta, MT, May 19 – 23, 2025

Agenda Item:	7.18.3.1

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on the remaining issues in first objective, particularly on optimizing Rx beam sweeping factor for L3 measurement delay reduction when UE support multiple-Rx simultaneous reception.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 discussed the conditions to apply the enhancement of R19 L3 measurement delay reduction (SSB based) in FR2-1 for UE supporting multiple-Rx simultaneous reception. It was agreed that the signal quality of the serving cell can be used as the triggering condition to activate/deactivate multi-Rx for faster beam sweeping in L3 measurement and no additional conditions will be introduced.

Note that, given the agreed scenarios (1/2/3/4/5/7) in RAN4#112 meeting as captured below [3], the above condition will only apply to scenarios (1/2/3/4). It does not apply to scenarios (5/7), i.e., handover and RRC Re-establishment, where multi-Rx is always activated for those events if supported by the UE.

- Scenario 1: SSB based Intra-frequency measurement without MG,

- Scenario 2: SSB based Intra-frequency measurement with MG,

- Scenario 3: SSB based Inter-frequency measurement without MG,

- Scenario 4: SSB based Inter-frequency measurement with MG,

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

Observation 1: The agreed triggering conditions to activate/deactivate multi-Rx for L3 measurement only applies to scenarios (1/2/3/4).

UE power consumption due to activating multiple panels for simultaneous DL reception is an important aspect to discuss and consider in RAN4. Even if UE fulfills the condition to enable FR2-1 SSB based L3 measurement delay reduction by activating multi-Rx, UE should also have the capability to indicate to the NW whenever UE needs to deactivate multi-Rx, for example, due to overheating resulting from activating multiple panels for simultaneous reception for long time. Note that such indication was also introduced in R18 multi-RX as shown in 38.331 spec below, wherein UE can indicate “single” to deactivate multi-Rx or “multiple” to re-enter multi-Rx mode after sending deactivation indication. Therefore, similar indication can be introduced for R19 L3 measurement delay reduction for UE supporting multi-Rx simultaneous reception.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2..

Proposal 1: RAN4 to use same principle as R18 UAI indication for multi-Rx to deactivate/activate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

Regarding the status report issue captured in the WF:

### Issue 1-1-3: Status report of fast beam sweeping Further discuss this issue.

In our view, for UE supporting multi-Rx to achieve fast beam sweeping for L3 measurement delay reduction does not change NW behaviour on the transmission of RS, i.e., NW still need to transmit SSB beams in FR2 for all both UE which support this capability and does not support this capability. Therefore, UE does not need to report to the NW when UE is performing FBS for L3 measurement delay reduction.

Proposal 2: No need for status report for fast beam sweeping for L3 measurement delay reduction.

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: The agreed triggering conditions to activate/deactivate multi-Rx for L3 measurement only applies to scenarios (1/2/3/4).

Proposal 1: RAN4 to use same principle as R18 UAI indication for multi-Rx to deactivate/activate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

Proposal 2: No need for status report for fast beam sweeping for L3 measurement delay reduction.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT.

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple.