3GPP TSG-RAN WG4 Meeting # 115	R4-2507817

Malta, MT, May 19 – May 23, 2025

Agenda Item: 7.24.4.2

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. The WID [1] is divided into two parts to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “Other RRM requirements”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul.

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

## 2.1 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration Agreement: Update the report resolution to (MD = 256, MFO = 256). Moderator’s note: simulation results are collected in R4-2504955, companies can use the table structure in the excel doc to update their results. Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified) RAN4#114 WF [3] Issue 1-1: How to decide on the number of samples used for CJT calibration? Agreement: - RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters - Report resolution (MD = 128, MFO = 128). - Offset range (AD and AFO) - AD = CP - AFO = 0.2 ppm - Time offset between the two TRPs: 0.5 CP - Frequency offset between the two TRPs: 0.1 ppm - Note that report resolution and offset range are network configuration via RRC signalling. - Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

In RAN4#114 meeting, it was agreed that the number of samples for CJT calibration report will be chosen based on the impact on the reported interval (granularity of the report). We provided the simulation result in [4] (also provided in appendix A) based on largest report resolution for both delay and frequency offsets (MD = 256, MFO = 256).

From the TO/FO estimation error of simulation result [4], it can be observed that the accuracy of quantization (reported interval) is improved significantly when the number of samples is increased from 1 to 3. However, there’re quite similar accuracy of quantization level for the number of samples 3 and 5 for both timing offset and frequency offset error estimation. In our view, there’s no strong reason to consider two more samples to define accuracy for 5 samples since there’s not much improvement in the accuracy from number of 3 samples to 5 samples.

Proposal 1: RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

On the other hand, it can be observed that for low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased. Therefore, to guarantee reasonable benefit from the reported intervals of the delay and frequency offsets, higher SNR side condition (3dB) need to be adopted for accuracy requirements.

Observation 1: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased.

Proposal 2: RAN4 to adopt higher SNR side condition for the accuracy requirements of the delay and frequency offsets.

According to WID [1] and last RAN4 agreement, we provided our view on the measurement period and scheduling restriction for FR1 on both FDD and TDD. Since delay offset and frequency offset are measured from periodic CSI-RS (TRS) and aperiodic reporting on PUSCH. The legacy measurement period can be reused as baseline. E.g., measurement period = max(TReport, ceil(M*P)*TCSI-RS) for non-DRX. TReport could be zero since this is aperiodic reporting. The M (number of sample) = 3 based on our proposal 1 and reuse the legacy sharing factor(P).

Proposal 3: Reuse the legacy L1-RSRP CSI-RS measurement period as baseline for delay and frequency offset measurement.

- TReport = 0

- M (number of samples) = 3

- Reuse legacy sharing factor (P)

Since delay and frequency offset measurement is assuming legacy CSI-RS design, the legacy scheduling restriction for FR1 on both FDD and TDD could be reused. We provided the legacy scheduling restriction as reference on the following:

- Scheduling restriction does apply,

  - If UE does not simultaneousRxDataSSB-DiffNumerology and

  - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols in FR1.

- Scheduling restriction does not apply,

  - If UE support simultaneousRxDataSSB-DiffNumerology or

  - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH in FR1.

Proposal 4: Reuse the legacy scheduling restriction in FR1 for delay and frequency offset measurement.

- Scheduling restriction does apply,

  - If UE does not simultaneousRxDataSSB-DiffNumerology and

  - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols.

- Scheduling restriction does not apply,

  - If UE support simultaneousRxDataSSB-DiffNumerology or

  - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH.

## 2.2 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

For single DL TRP and multiple UL TRPs, RAN4 discussion on the related issues are captured in the WF [2] as below:

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2. Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Discuss whether gradual timing adjustment applies to both TAGs or not. - Discuss whether gradual timing adjustment applies to DL/UL TRP or also to UL TRP. Issue 2-3: For Scenario#1, whether other updates are needed for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Discuss whether RACH transmission requirements need to be updated. Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2? Agreement: - RAN4 to define timing requirements for FR1 as well as FR2. Issue 2-5: Whether to define UL TCI state activation requirement based on SRS? Agreement: No RRM impacts on TCI state switching requirement. Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2? Way forward: discuss the following options in the next meeting for Scenario#2 - Option 1: No RRM impacts on TCI state switching requirement. - Option 2: Legacy SSB based UL TCI state activation requirement can be re-used.

For Issue 2-1, RAN1 discussed the previous raised question by RAN4 (in the LS R4-2502667) and the following reply LS (R1-2503091) was sent during the last meeting by RAN1 as shown below:

Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

According to the above LS RAN1 clarifies that the previous agreement “One downlink reference timing is supported and applied to both TAGs” does not apply for the scenario when UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Therefore, RAN4 need to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP based on two DL reference timings.

Proposal 5: For Scenario#2, update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP based on two DL reference timings principle.

For Issue 2-2, the gradual timing requirements for two TAs can be updated similar to legacy requirements for (multi-DCI based multi-TRP operation with two TAs). The gradual timing adjustment applies for each TAG and shall be met for each TAG.

Proposal 6: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

Regarding UL TCI state activation requirement, RAN4 agreed in the last meeting not to define RRM requirements for TCI state activation based on SRS. Therefore, for Issue 2-6, on whether to define UL TCI state activation requirement for Scenario#2, we don’t think there is any RRM impact and legacy requirements can be applied.

Proposal 7: No RRM impacts on TCI state switching requirement.

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Proposal 1: RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

Observation 1: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased.

Proposal 2: RAN4 to adopt higher SNR side condition for the accuracy requirements of the delay and frequency offsets.

Proposal 3: Reuse the legacy L1-RSRP CSI-RS measurement period as baseline for delay and frequency offset measurement.

- TReport = 0

- M (number of samples) = 3

- Reuse legacy sharing factor (P)

Proposal 4: Reuse the legacy scheduling restriction in FR1 for delay and frequency offset measurement.

- Scheduling restriction does apply,

  - If UE does not simultaneousRxDataSSB-DiffNumerology and

  - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols.

- Scheduling restriction does not apply,

  - If UE support simultaneousRxDataSSB-DiffNumerology or

  - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH.

Proposal 5: For Scenario#2, update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP based on two DL reference timings principle.

Proposal 6: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

Proposal 7: No RRM impacts on TCI state switching requirement.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2504903 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2502596, WF on RRM requirements for NR_MIMO_Ph5 (Part2)

[4] R4-2504955 Collection of simulation results for CJT calibration in NR_MIMO_Ph5_Part2

# Appendix A

Simulation assumption and results for CJT calibration:

Table 1. Simulation parameters for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

- Timing offset simulation results:

Table 2. Simulation result for timing offset error estimation.

| SCS | SNR | Timing offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 0.5CP | 29 | 19 | 18 |
| 15KHz | 0 | 0.5CP | 17 | 12 | 10 |
| 15KHz | 3 | 0.5CP | 10 | 7 | 6 |
| 30KHz | -3 | 0.5CP | 30 | 19 | 17 |
| 30KHz | 0 | 0.5CP | 16 | 11 | 10 |
| 30KHz | 3 | 0.5CP | 14 | 9 | 9 |

- Frequency offset simulation results:

Table 3. Simulation result for frequency offset error estimation.

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 65 | 39 | 29 |
| 15KHz | 0 | 350 Hz | 39 | 24 | 18 |
| 15KHz | 3 | 350 Hz | 26 | 16 | 12 |
| 30KHz | -3 | 350 Hz | 133 | 71 | 60 |
| 30KHz | 0 | 350 Hz | 83 | 48 | 37 |
| 30KHz | 3 | 350 Hz | 52 | 31 | 24 |