3GPP TSG-RAN WG4 Meeting #115	R4-2507821

Malta, MT, May 19 – 23, 2025

Agenda Item:	5.25.3

Source:	MediaTek Inc.

Title:	Discussion on Collision between MUSIM Gap and Measurement Gap

Document for:	Discussion

# Introduction

This paper discusses the received LS from RAN2 [1] as captured below, and also try to answer RAN2 question.

1 Overall description On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

# Discussion

According to RAN2 LS, network can configure MUSIM gap and measurement Gap without priority (either one of them or both of them together without priority) and RAN2 ask RAN4 on whether UE requirement is needed for the above collision cases. In our view, RAN4 has already discussed the case when priority is not configured for MUSIM gap or MG, and the following collision handling requirements are defined [2], where the collisions in this case are handled based on the MGRP of the colliding gaps.

9.1.10.5 Collisions between MUSIM gaps and measurement gaps …. Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps: - If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4. - Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

Proposal 1: RAN4 has already defined requirements for collision handling between MUSIM gap and MG when priority is not configured for one or both of them.

Complementary to this paper, we prepared a draft Reply LS (R4-2507822) to answer RAN2 raised question regarding the above issue.

Proposal 2: RAN4 to send a reply LS to RAN2 to address RAN2 question regarding the collision between the MUSIM Gap and Measurement Gap.

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: RAN4 has already defined requirements for collision handling between MUSIM gap and MG when priority is not configured for one or both of them.

Proposal 2: RAN4 to send a reply LS to RAN2 to address RAN2 question regarding the collision between the MUSIM Gap and Measurement Gap.

# References

- R2-2503163, LS on Collision between MUSIM Gap and Measurement Gap, ZTE.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, v19.0.0.