# 3GPP TSG-RAN WG4 Meeting #115	R4-2507822

# Malta, MT, 19th – 23rd May, 2025

Title:	Reply LS on collision handling between MUSIM gaps and MGs

Response to:	R2-2503163

Release:	Rel-18

Work Item:	NR_DualTxRx_MUSIM-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:   Ogeen Toma

E-mail Address:   ogeenhanna.toma@mediatek.com

Attachments:	N/A

# 1. Overall Description:

RAN4 would like to thank RAN2 for the LS on the collision between the MUSIM Gap and Measurement Gap when priority is not configured for one of them or even both. RAN4 had discussed these cases before, and the corresponding requirements were defined in 38.133 spec as captured below, wherein collisions in this case are handled based on the MGRP of the colliding gaps.

9.1.10.5 Collisions between MUSIM gaps and measurement gaps …. Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps: - If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4. - Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

# 2. Actions:

To RAN WG2 group:

RAN4 kindly requests RAN2 to take the above information into consideration for future work.

# 3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #116			   25 – 29 Aug 2025						India

TSG RAN WG4 Meeting #116-bis			13 – 17 Oct 2025						Prague, CZ