3GPP TSG-RAN WG4 Meeting #115	R4-2507824

St Julian’s, Malta, 19th – 23rd May, 2025

Source:	BT

Title:	Simulation Results for SCM Comparison and Alignment

Agenda item:	7.16.3

Document for:	Information

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN4#112, RAN4#112b, RAN4#113, RAN4#114 and RAN4#114b approved way forward agreements [2-6] respectively. This contribution provides simulation results based on agreements in [6].

Simulation Results

Simulation setup

Simulation parameters are provided in Annex A, where the agreements from [6] are integrated in the table. Throughout the simulation results presented in this contribution we use CDL-C channel model based on TR 38.901 and a truncated version of it based on [6]. Perfect channel estimation is used in the simulation results provided in this contribution. In our current simulation method, a constant noise power level is utilized while sweeping across different transmit powers, to achieve the target SNR at the receiver ports. The long-term SNR is calculated using the formula

SU PDSCH and PMI results for alignment

Results in this section provide the minimum SNR values required for the given percentage of maximum throughput, at 30% and 70% for PDSCH, and 70% and 90% for PMI. CDL option 1 is based on CDL-C from TR 38.901 and option 2 is based on Annex A of [6].

Table 1. SU PDSCH Results with Truncated and Original TR 38.901 CDL-C Channel Model

| Channel Model |  | UE Speed/ Doppler | PMI | Test point - % of Rel TP | SNR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Codeword 1 | Codeword 2 |
| 4x4 w/ 4 layers |  |  |  |  |  |  |
| CDL Option 1 | AAV option 3 | 3 kph | Type 1 Random | 30% | 8.2 |  |
|  |  |  | Type 1 Random | 70% | 16.0 |  |
| CDL Option 2 |  |  | Type 1 Random | 30% | 6.3 |  |
|  |  |  | Type 1 Random | 70% | 14.9 |  |
| 8x8 w/ 8 layers |  |  |  |  |  |  |
| CDL Option 1 | AAV option 3 | 3 kph | Type 1 Random | 30% | 6.8 | 9.6 |
|  |  |  | Type 1 Random | 70% | 16.6 | 20.3 |
| CDL Option 2 |  |  | Type 1 Random | 30% | 5.4 | 13.3 |
|  |  |  | Type 1 Random | 70% | 17.9 | 26.3 |

Table 2. SU PMI Results with Truncated CDL-C Channel Model

| Channel Model |  | UE Speed/ Doppler | PMI | Metric (% of Max TP) | SNR |
| --- | --- | --- | --- | --- | --- |
| 8x4 w/ 4 layers |  |  |  |  |  |
| CDL Option 2 | AAV option 3 | 3 kph | Type I Follow | 70% | 11.7 |
|  |  |  |  | 90% | 16.2 |
|  |  |  | eType II Follow | 70% | 14.2 |
|  |  |  |  | 90% | 18.0 |

Comparison results

Simulations for comparison cases using AAV 1Y have not completed at the time of writing and will be provided to RAN4#115 meeting.

Conclusion

This contribution provides SNR data points for the CDL channel model alignment cases in [6]. Further results for the comparison cases will be submitted to RAN4#115 meeting.

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.

- R4-2502378, Way Forward [114][322] NR_SCM, RAN4#114, Feb 2025.

- R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114b, April 2024.

Annex A: Simulation parameters

| Parameter |  |  |  |  | Unit | 8TX | 32TX (for information) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  |  |  | FDD |  |
| TDD DL-UL configurations |  |  |  |  |  | N/A |  |
| MCS |  |  |  |  |  | 13 |  |
| Antenna configuration |  |  |  |  |  | 8 x 4 (N1,N2) = (4,1) | 32 x 4 (N1,N2) = (8,2) |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource Type |  |  |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 32 |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  |  |  | 1 | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  |  | Row 8,(4,6) | Row 17,(2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | Row 8,(5) | Row 17,(5, 12) |
|  |  | CSI-RS interval and offset |  |  | slot | 10, 0 |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  |  | Not configured for eType II Wideband for Type I |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  |  |  | 11111111111111 |  |
| Codebook configuration |  | Codebook Type |  |  |  | - typeII-r16 - typeI-SP |  |
|  |  | eType II CB config | paramCombination-r16 |  |  | 6 (L =4, pν =1/2, β=1/2 ) |  |
|  |  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) | (8,2) |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) | (4,4) |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  |  | RI Restriction (typeII-RI-Restriction-r16) |  |  |  | Rank 2: 0010 Rank 4: 1000 |  |
| Physical channel for CSI report |  |  |  |  |  | Perfect |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 7 |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Channel Geometry (CDL) | LCS UE |  |  | α = 180°, β=0°, γ = 90° |  |  |  |
|  | LCS gNodeB |  |  | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE |  |  | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB |  |  | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation |  |  | (+45, -45) |  |  |  |
|  | BS Radiation Pattern |  |  | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation |  |  | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern |  |  | Omnidirectional |  |  |  |
|  | Antenna Panel Placement |  |  | YZ Plane |  |  |  |