# 3GPP TSG-RAN WG4 Meeting #115	R4-2507832

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR |  | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 36.108: Output power dynamics and Transmitted signal quality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Core |  |  |  |  |  | Date: |  |  | 28-03-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In this Draft CR we provide initial inputs for Output power dynamics and Transmitted signal quality, as per work-split. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initial inputs for conducted requirements: - Output power dynamics, and - Transmitted signal quality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SAN RF requirements would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3C, 6.5C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 6.3C	Output power dynamics for 5G Broadcast over GEO

### 6.3C.1	General

The requirements in clause 6.3C apply during the transmitter ON period. Transmit signal quality (as specified in clause 6.5C) shall be maintained for the output power dynamics requirements of this clause.

Power control is used to limit the interference level.

### 6.3C.2	Total power dynamic range

#### 6.3C.2.1	General

The SAN total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

For SAN type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE:	The upper limit of the dynamic range is the OFDM symbol power for a SAN when transmitting on all RBs at maximum output power. The lower limit of the total power dynamic range is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

#### 6.3C.2.2	Minimum requirement for SAN type 1-H

The downlink (DL) total power dynamic range for each SAN carrier shall be larger than or equal to the level in table 6.3C.2.2-1.

Table 6.3C.2.2-1: Total power dynamic range

| SAN channel | Total power dynamic range (dB) |
| --- | --- |
| bandwidth (MHz) | 15 kHz SCS |
| 10 | 17.1 |

------------------------------ Next modified section -------------------------

## 6.5C	Transmitted signal quality

### 6.5C.1	Frequency error

Frequency error is the measure of the difference between the actual SAN transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

#### 6.5C.1.1	Minimum requirement

The modulated carrier frequency of each E-UTRA carrier configured by the SAN shall be accurate to within the accuracy range of ±0.05 ppm observed over a period of one subframe (1ms).

### 6.5C.2	Modulation quality

The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector. The equaliser parameters are estimated as defined in Annex B. The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent.

The EVM measurement shall be performed for each carrier over all allocated resource blocks and downlink subframes within 10 ms measurement periods for subframe TTI. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries. The EVM value is then calculated as the mean square root of the measured values. The EVM of each carrier for different modulation schemes on PMCH shall be better than the limits in table 6.5C.2-1:

Table 6.5C.2-1: EVM requirements

| Modulation scheme for PMCH | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 |
| 16QAM | 12.5 |

------------------------------ End of modified section -------------------------