# 3GPP TSG-RAN WG4 Meeting #115	R4-2507850

# Malta, MT, 09th – 13th May, 2025

Agenda item:			7.9.1

Source:	Astrum Mobile

Title:	Topic summary for [115][318] 5G_Broadcast_GSO_NTN_band

Document for:	Information

Introduction

In RAN#107, the new WID for New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite was approved in [1] with the following objectives:

- Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

In RAN4#114bis, the issues for UE and SAN RF requirements were discussed and the agreements were captured in WF [2].

This document is to summarize some open issues and CRs for Rel-19 WI on New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite. The summary covers the contributions submitted under the following agendas:

7.9	 New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite	[LTE_band_5G_bcast_GSO]

7.9.4	UE RF core requirements	[LTE_band_5G_bcast_GSO-Core]

7.9.5	SAN RF core requirements [LTE_band_5G_bcast_GSO-Core]

Topic #: UE RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505515 | Astrum Mobile | Title: Draft CR to TS 36.102: New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, Rel-19. Summary of change:Definitions, symbols and abbreviations & General & Operating bands and channel arrangement for New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite are introduced. |
| R4-2505738 | Qualcomm Incorporated | Title: On UE RF requirements for 5G broadcast utilizing a geosynchronous satellite Proposal 1: To only specify 15kHz SCS for 10MHz PMCH for the new band for 5G Broadcast Utilizing Geosynchronous satellite in Rel-19. Proposal 2: The RMC of 15kHz SCS to be specified in TS 36.102 based on the following Table 1: Table 1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) Parameter; PMCH (15 kHz SCS). Unit; Value. Reference channel; R.PMCH.1; R.PMCH.2. PMCH bandwidth; MHz; 10; 10. Allocated resource blocks; 50; 50. Allocated subframes per Radio Frame(Note1); 10; 10. Modulation; QPSK; 16QAM. Target Coding Rate; 1/3; 1/2. Information Bit Payload (Note 2). For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; 3624; 9912. Number of Code Blocks per Sub-Frame (Note 3); 1; 2. Binary Channel Bits Per Subframe. For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; 10200; 20400. MBMS UE Category; ≥ 1; ≥ 1. Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).. Proposal 3: RAN4 to decide which option should be used to define the frequency range of out of band blocking requirements to accommodate band 50 filter design. - Option 1: Reuse legacy frequency range of band 50 out of band blocking requirements - Option 2: Define the Rang 1 starting from 15MHz below and above frequency range of band 246 Proposal 4: The new proposed band for LTE based 5G broadcast utilizing satellite is specified in a release independent manner from Rel-17. |
| R4-2506232 | ZTE Corporation, Sane Chips | Title: Discussion on UE RF requirements for supporting new SDO band for LTE-based 5G Broadcast Proposal 1: To revise the OOBB requirement as follows: Table 7.6C.3-2: Out of band blocking Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pw; dBm; REFSENS + 6 dB. Pinterferer; dBm; -44; -30; -15. 246; Finterferer (CW); MHz; -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85; -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110; 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750. Proposal 2: Four SCSs(15k/2.5k/1.25k/0.37k) are considered for RMC parameters for LTE-based 5G Broadcast for GSO. Proposal 3: To define Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) as Table A.1.3.1-1: Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) Parameter; PMCH (15 kHz SCS). Unit; Value. Reference channel; R.PMCH.1; R.PMCH.2. PMCH bandwidth; MHz; 10; 10. Allocated resource blocks; 50; 50. Allocated subframes per Radio Frame(Note1); 10; 10. Modulation; QPSK; 16QAM. Target Coding Rate; 1/3; 3/4. Information Bit Payload (Note 2). For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; 3624; 15264. Number of Code Blocks per Sub-Frame (Note 3); 1; 3. Binary Channel Bits Per Subframe. For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; 10200; 20400. MBMS UE Category; ≥ 1; ≥ 2. Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).. Proposal 4: To specify the spurious emission requirement for LTE based 5G broadcast utilizing satellite as table 7.7C-1 and table 7.7C-2. Table 7.7C-1: Spurious response parameters Rx parameter; Units; Channel bandwidth. 10 MHz. Power in Transmission Bandwidth Configuration; dBm; REFSENS + channel bandwidth specific value below. 6. NOTE: Reference measurement channel is specified in Annex A.1.3.. Table 7.7C-2: Spurious response Parameter; Unit; Level. PInterferer (CW); dBm; -44. FInterferer; MHz; Spurious response frequencies. |
| R4-2505739 | Qualcomm Incorporated | Title: Draft CR to TS 36.102: UE RF requirements for 5G broadcast for geosynchronous satellite (Reference sensitivity and RMC) Summary of change: Reference sensitivity and corresponding RMC for new LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite are introduced. |
| R4-2505740 | Qualcomm Incorporated | Title: Draft CR to TS 36.102: UE RF requirements for 5G broadcast for geosynchronous satellite (Maximum input level and ACS) Summary of change: Reference sensitivity and corresponding RMC for new LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite are introduced. |
| R4-2506234 | ZTE Corporation, Sane Chips | Title: Draft CR to TS36.102 Introduction of New band for 5G Broadcast Utilizing Geosynchronous Satellite Summary of change: 1.Add a new chapter 7.6.C to support New band for 5G Broadcast Utilizing Geosynchronous Satellite Blocking characteristics. 2.Add a new chapter 7.7.C to support New band for 5G Broadcast Utilizing Geosynchronous Satellite Spurious response characteristics. 3.Add a new chapter 7.8.C to support New band for 5G Broadcast Utilizing Geosynchronous Satellite Intermodulation characteristics. |

Open issues summary

Sub-topic 1-2: UE RF requirements

Issue 1-2-1: 15kHz SCS for 10MHz PMCH

- Proposals

  - Proposal 1 (Qualcomm): To only specify 15kHz SCS for 10MHz PMCH for the new band for 5G Broadcast Utilizing Geosynchronous satellite in Rel-19.

  - Proposal 2(ZTE): Four SCSs(15k/2.5k/1.25k/0.37k) are considered for RMC parameters for LTE-based 5G Broadcast for GSO.

- Recommended WF

  - Specify 15kHz SCS for 10MHz PMCH in Rel-19.

Issue 1-2-2: The RMC of 15kHz SCS

- Proposals

  - Proposal 1 (Qualcomm): The RMC of 15kHz SCS to be specified in TS 36.102 based on the following Table 1:

Table 1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

  - Proposal 2(ZTE): To define Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) as Table A.1.3.1-1:

Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 15264 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 3 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 2 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

- Recommended WF

  - The RMC of 15kHz SCS to be specified in TS 36.102 based on the following Table 1:

Table 1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Issue 1-2-3: Out of band blocking requirements

- Proposals

  - Proposal 1 (Qualcomm): RAN4 to decide which option should be used to define the frequency range of out of band blocking requirements to accommodate band 50 filter design.

    - Option 1: Reuse legacy frequency range of band 50 out of band blocking requirements

    - Option 2: Define the Rang 1 starting from 15MHz below and above frequency range of band 246

  - Proposal 2 (ZTE): To revise the OOBB requirement as follows:

Table 7.6C.3-2: Out of band blocking

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 246 | Finterferer (CW) | MHz | -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85 | -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110 | 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750 |

- Recommended WF

  - Reuse legacy frequency range of band 50 out of band blocking requirements.

Issue 1-2-4:  Spurious emission requirement

- Proposals

  - Proposal (ZTE): To specify the spurious emission requirement for LTE based 5G broadcast utilizing satellite as table 7.7C-1 and table 7.7C-2.

Table 7.7C-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.7C-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

- Recommended WF

  - Proposal is reasonable and agreeable.

Issue 1-2-5: Release Independent

- Proposals

  - Proposal (Qualcomm): The new proposed band for LTE based 5G broadcast utilizing satellite is specified in a release independent manner from Rel-17.

- Recommended WF

  - Proposal is reasonable and agreeable.

CRs

Issue 1-3-1: CR on Definitions, symbols and abbreviations & General & Operating bands and channel arrangement

- R4-2505515

- Recommended WF

  - CR is reasonable and agreeable.

Issue 1-3-2: CR on Reference sensitivity and RMC

- R4-2505739

- Recommended WF

  - CR is reasonable and agreeable.

Issue 1-3-3: CR on Maximum input level and ACS

- R4-2505740

- Recommended WF

  - CR is reasonable and agreeable.

Issue 1-3-4: CR on chapter 7.6.C,7.7C and &7.8.C

- R4-2506234

- Recommended WF

  - CR is reasonable and agreeable.

  - Use Abbreviations “BOG—5G Broadcast over Geosynchronous Satellite” to replace all description related to “5G broadcast over GSO satellite”.

Topic #4: SAN RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505516 | Astrum Mobile | Title: Draft CR to TS 36.108: New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, Rel-19. Summary of change: Scope&Definitions, symbols and abbreviations & General & Operating bands and channel arrangement. |
| R4-2506233 | ZTE Corporation, Sane Chips | Title: Discussion on SAN RF requirements for supporting new SDO band for LTE-based 5G Broadcast Proposal 1: Total power dynamic range requirement is not applied for LTE based 5G broadcast over Satellite. |
| R4-2507831 | Huawei, HiSilicon | Title: Follow-up on SAN RF way forward agreements Issue 4-1-1: Satellite Access Node output power - WF: No specification impact. - SAN output power requirement does apply to NTN SAN (TS 38.108), as well as to IoT NTN SAN (TS 36.108). Therefore, it is suggested to consider this requirement. As WID mentions high power SAN, it would be beneficial for the discussion to know more details of the planned deployments. Issue 4-1-2: RE power control dynamic range - WF: No specification impact. - As RE power control dynamic range is not applicable to PMCH, it does not have to be defined for 5G_Broadcast_GSO_NTN_band. Issue 4-1-3: Total power dynamic range - WF: FFS on whether total power dynamic range requirement applies for LTE based 5G broadcast over Satellite - Total power dynamic range requirement does apply to NTN SAN (TS 38.108), as well as to IoT NTN SAN (TS 36.108). Even if TPDR does not apply to terrestrial broadcast, it is seen valid to reassure that the SAN performance is not worse than existing NTN SAN spec. Therefore, it is suggested to consider TPDR requirement for 10MHz channel (15kHz SCS). Issue 4-1-4: Transmit ON/OFF power - WF: No specification impact. - This requirement is not applicable to SAN. Issue 4-1-5: Transmitted signal quality - WF: No specification impact. - While TAE is not applicable, frequency error shall be considered. EVM is captured below. Issue 4-1-6: Occupied bandwidth - WF: No specification impact. - This requirement shall be applicable. Issue 4-1-7: Adjacent Channel Leakage Power Ratio - WF: Reuse the legacy requirements for GEO - Agree, as it shall not be worse than existing SAN spec. Issue 4-1-8: EVM - WF: To specify the EVM requirements for LTE based 5G broadcast over satellite based on the following table. Table 6.5.2-1: EVM requirements for E-UTRA carrier Modulation scheme; Required EVM (%). QPSK; 17.5. 16QAM; 12.5. - Agree, as it shall not be worse than existing SAN spec. Issue 4-1-9: Out-of-band emissions - WF: Reuse the legacy requirements. - Reuse SAN specification. Issue 4-1-10: Transmitter spurious emissions - WF: Reuse the legacy requirements. - Follow ITU. Issue 4-1-11: Transmitter intermodulation - WF: No specification impact, not applicable to SAN. - Not applicable to SAN, but there may be some impact on spec due to suffix structure. Issue 4-1-12: Window length - WF: To reuse EVM window length for 10MHz defined in 36.104. - Good starting point; may be discussed during Perf part. - Sub-topic 4-2: SAN RF conformance requirements - WF: As per WID: RAN4 to further discuss whether SAN RF conformance requirements and tests will be defined. - To be discussed after Core completion. |
| R4-2506235 | ZTE Corporation, Sane Chips | Title: Draft CR to TS36.108 Introduction of New band for 5G Broadcast Utilizing Geosynchronous Satellite Summary of change: 1.Add 10 MHz channel bandwidth for SAN ACLR limit for GEO class to support New band for 5G Broadcast Utilizing Geosynchronous Satellite 2.Add 10 MHz channel bandwidth for EVM window length to support New band for 5G Broadcast Utilizing Geosynchronous Satellite. |
| R4-2507832 | Huawei, HiSilicon | Title: Draft CR to TS 36.108: Output power dynamics and Transmitted signal quality Summary of change: Initial inputs for conducted requirements: Output power dynamics, and Transmitted signal quality. |

Open issues summary

Sub-topic 2-2: SAN RF requirements

Issue 2-2-1: Total power dynamic range

- Proposals

  - Proposal 1 (ZTE): Total power dynamic range requirement is not applied for LTE based 5G broadcast over Satellite.

  - Proposal 2 (Huawei): Total power dynamic range requirement does apply to NTN SAN (TS 38.108), as well as to IoT NTN SAN (TS 36.108). Even if TPDR does not apply to terrestrial broadcast, it is seen valid to reassure that the SAN performance is not worse than existing NTN SAN spec. Therefore, it is suggested to consider TPDR requirement for 10MHz channel (15kHz SCS).

- Recommended WF

  - Need further discussion on whether to apply Total power dynamic range

Issue 2-2-2: Transmitted signal quality

- Proposals

  - Proposal 1 (Huawei):  While TAE is not applicable, frequency error shall be considered (WF from last meeting: No specification impact.).

- Recommended WF

  - Discussion during this meeting.

Issue 2-2-3: Occupied bandwidth

- Proposals

  - Proposal 1 (Huawei):  This requirement shall be applicable.( WF from last meeting: No specification impact)

- Recommended WF

  - Discussion during this meeting.

Issue 2-2-4: Satellite Access Node output power

- Proposals

  - Proposal 1 (Huawei): It is suggested to consider this requirement. As WID mentions high power SAN, it would be beneficial for the discussion to know more details of the planned deployments.

- Recommended WF

  - To confirm whether the legacy output power requirements in TS 38.108 can be reused.

Issue 2-2-5: Transmitter spurious emissions

- Proposals

  - Follow ITU (WF from last meeting: To reuse legacy requirements).

- Recommended WF

  - Reuse legacy requirements and follow ITU requirements if any

Issue 2-2-6: SAN RF conformance requirements

- Proposals

  - To be discussed after Core completion.

- Recommended WF

  - To be discussed after Core completion

CRs

Issue 2-3-1: CR on Scope&Definitions, symbols and abbreviations & General & Operating bands and channel arrangement

- R4-2505516

- Recommended WF

  - CR is reasonable and agreeable.

Issue 2-3-2: CR on Output power dynamics and Transmitted signal quality

- R4-2507832

- Recommended WF

  - CR is reasonable and agreeable.

  - Use Abbreviations “BOG—5G Broadcast over Geosynchronous Satellite” to replace all description related to “5G broadcast over GSO satellite”.

Issue 2-3-3: CR on SAN ACLR limit and EVM window length

- R4-2506235

- Recommended WF

  - CR is reasonable and agreeable.

  - Use Abbreviations “BOG—5G Broadcast over Geosynchronous Satellite” to replace all description related to “5G broadcast over GSO satellite”.

- 3. References

- RP-250788, “New WID: New band for 5G Broadcast Utilizing Geosynchronous Satellite”, Astrum Mobile,   Qualcomm, RAN#107, March 2025

- R4-2504725, “Way Forward for [114bis][318] 5G_Broadcast_GSO_NTN_band”, Astrum Mobile, Qualcomm, ZTE, Huawei