3GPP TSG-RAN WG4 Meeting #115	R4-2507865

St. Julian’s, Malta, 19th ‒ 23rd May 2025

Agenda item:			7.31.1

Source:	Moderator (Ericsson)

Title:	Topic summary for [115][333] NR_NTN_Ph3_demod

Document for:	Information

# Introduction

This topic summary summarizes the issues on SAN and UE demodulation requirements for Rel-19 WI NTN for NR Phase 3 WI (RP-243300), which contains the following objectives:

- Downlink coverage enhancements

- Uplink capacity/cell throughput enhancements

- Broadcast service

- Regenerative payload

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs

# Topic #1: Broadcast services and Regenerative payload

This topic disucsses whether RAN4 define SAN and/or UE demodulation requirements for NTN broadcase services and/or regenerative payload.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505502 | Nokia | Observation 1: Improvements to DL Link Level are being defined in Rel-19 for NR NTN Proposal 1: RAN4 shall define requirements for PDCCH with repetition Proposal 2: RAN4 shall define requirements for PDSCH with Msg4 via PDSCH repetition Proposal 3: RAN 4 shall define requirements for PDSCH repetitions within 20 ms duration Observation 2: Improvements to UL Link Level are being defined in Rel-19 for NR NTN Proposal 4: RAN4 shall define new requirements for OCC PUSCH |
| R4-2505913 | Apple | Observation 1: There is no UE demodulation requirement impact with the following features introduced in R19 NTN - Uplink Coverage Enhancement - Support for broadcast service - Support for regenerative payload Observation 2: There is potentially impact to UE demodulation requirements with link level enhancements for FR1-NTN. Observation 3: PDCCH repetition enhancements for NTN are for CSS which we typically don’t define requirements for. Proposal 1: RAN4 to discuss if we should introduce requirements for PDCCH repetition for NTN. Observation 4: PDSCH repetition enhancements for NTN are for SIB1 and Msg4 which we don’t define requirements for. Proposal 2: RAN4 to discuss if we should introduce requirements for PDSCH -SIB1 / Msg4 repetition for NTN. Observation 5: RAN4 would need to discuss introducing requirements for NTN features of HARQ enhancement with RedCap/eRedCap features of reduced number of RX, reduce bandwidth, reduced data rate. Proposal 3: Do not introduce PDCCH, PBCH and CSI reporting requirements for Rel-19 NTN for RedCap/eRedCap UE. Proposal 4: Choose a combination NTN features and RedCap/eRedCap features for PDSCH demodulation requirements introduced in Rel-19. Observation 6: The reduced bandwidth for RedCap UEs with NTN is already covered in existing R17 NTN PDSCH demodulation requirements with 2RX. Proposal 5: Introduce requirements with 1RX for NTN in Rel-19. Proposal 6: Introduce requirements with 1RX and 2RX for eRedCap UEs for reduced bandwidth and without reduced bandwidth for NTN. Proposal 7: For PDSCH requirements use the following config- - MCS 4, MCS13; Rank 1 - Antenna config: 1T1R, 1T2R - Channel Model: NTN-TDLA100-200, NTN-TDLC5-200. Proposal 8: Consider the following test cases/ parameters for RedCap with NTN […] Proposal 9: Consider the following test cases/ parameters for eRedCap with NTN […] |
| R4-2505979 | Samsung | Proposal 1: For DL coverage enhancement, introduce PDSCH and PDCCH demodulation requirement in both FR1-NTN and FR2-NTN. Proposal 2: For Uplink Capacity enhancement, introduce PUSCH demodulation performance to support OCC processing based DFT-s-OFDM waveform Proposal 3: There is no performance impact for objective 4: “Signalling of intended service area of a broadcast service” Proposal 4: There is no performance impact for objective 4: “Supporting of regenerative payload” Proposal 5: FFS to introduce new demodulation requirements for RedCap devices to support NTN scenario. Proposal 6: Only consider repetition type A for specifying PUSCH requirement with OCC Proposal 7: Only consider OCC length as 2 for specifying PUSCH requirement with OCC Proposal 8: Consider one OCC group as starting point, the initial RV value can be configured as 0. FFS on considering two OCC groups with number of repetitions as 2 to verify the RV cycling function across OCC groups, the initial RV value for two OCC groups as 0,2. Proposal 9: Configure number of repetitions as 2 for staring point, FFS to configure number of repetitions as 4 to verify the RV cycling function across OCC groups Proposal 10: Only consider the minimum number of channel bandwidth for each SCS for specifying PUSCH requirement with OCC - 15KHz, 5MHz - 30KHz, 10MHz Proposal 11: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC with test applicability rue for different Rx number Proposal 12: Consider the same MCS configured for two UEs for specifying PUSCH requirement with OCC Proposal 13: Consider the same channel model configured for two UEs for specify PUSCH requirement with OCC Proposal 14: Consider there is no frequency offset configured for two UEs as baseline assumption, FFS on frequency offset assumption. Proposal 15: Considering the existing test parameters for PUSCH with DFT-s-OFDM waveform requirement as reference. |
| R4-2506176 | Qualcomm | Proposal 1: Do not define CSI reporting requirements for RedCap UEs over NR-NTN. Proposal 2: Do not define PBCH and PDCCH requirements for RedCap UEs over NR-NTN. Proposal 3: Do not define SDR requirements for RedCap UEs over NR-NTN. Proposal 4: Don’t introduce separate requirements for HD-FDD RedCap UEs. Proposal 5: Discuss the feasibility of defining PDSCH demodulation requirements for RedCap UEs over NR-NTN with 2Rx only. Proposal 6: Do not define CSI reporting requirements for eRedCap UEs over NR-NTN. Proposal 7: Do not define PBCH and PDCCH requirements for eRedCap UEs over NR-NTN. Proposal 8: Discuss the feasibility of defining PDSCH requirements for eRedCap UEs over NR-NTN with 2Rx only. |
| R4-2506633 | Ericsson | Proposal 1: Consider defining SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes. Proposal 2: No SAN impact for the support of eRedCap UEs operating in FR1-NTN bands. Observation 1: Following RAN1 agreements, RAN4 should also consider Hadamard sequences to generate orthogonal cover codes. Intra-symbol pre-DFT OCC need not to considered. Observation 2: Current SAN Requirements for PUSCH repetition Type A are with disabled transform precoding. However, as per the WID, transform precoding may be enabled. Proposal 3: Consider SAN PUSCH repetition type Aassumptions, as a starting point to define requirements with OCC. Observation 3: Inter-OCC group frequency hopping may be considered for SAN PUSCH repetition Type A requirements, considering significant gains observed in RAN1. Observation 4: It may be needed to investigate on: - The impact of CFO drifting on PUSCH performance test. - The impact of starting offset of CFO within OCC group on PUSCH performance test. - The number of repetitions feasible for the considered OCC-length and if there is a performance difference seen, for SAN PUSCH repetition type A requirements. - RV sequence to be used. Observation 5: RAN4 SAN demodulation should wait for conclusion on phase and power consistency within OCC group issue in RAN4 RF discussion. Proposal 4: Consider simulation assumptions in Table 2.3-1 as a starting point for performance requirements of PUSCH repetition type A with OCC. Proposal 5: Define PDSCH requirements for NTN+RedCap and NTN+eRedCap, i.e. […] Proposal 6: Rel-17 RedCap FR1 PDCCH requirements are also applicable for Rel-19 NTN eRedCap UE. Proposal 7: Rel-17 RedCap FR1 PBCH requirements are also applicable for Rel-19 NTN eRedCap UE. Proposal 8: Do not consider SAN requirements for the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands. Proposal 9: Define PDSCH demodulation requirements with the following test setup: Proposal 10: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements. Proposal 11: Do not define new CSI reporting requirement for (e)RedCap UE in Rel-19 NTN enhancement. Proposal 12: Existing CSI reporting requirements for RedCap in Rel-17 and eRedCap in Rel-18 should be applied for Rel-19 NTN access capable (e)RedCap UE according to the UE capability. |
| R4-2507028 | Huawei, HiSilicon | Proposal 1: Do not define requirements for downlink coverage enhancements. Proposal 2: Define one performance requirement with inter-slot OCC length 2. Proposal 3: - Define a set of requirements for (e)RedCap with 1Rx with similar test configurations as the existing PDSCH cases - Define new reference channel for HD-FDD (e)RedCap with same performance requirements as for FDD - Define new performance requirements for eRedCap with reduced baseband - Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband |
| R4-2507116 | MediaTek | Proposal 1: RAN4 to discuss whether to introduce requirements for PDCCH repetition. Proposal 2: Do not define demodulation requirements for “PDSCH with Msg4 via PDSCH repetition” as it may require extensive testing time, given that Msg4 is part of the RACH process. Proposal 3: Do not define demodulation requirements for “PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration” as there is no ACK/NACK feedback from the DUT to the TE to verify the correctness of reception. Proposal 4: Do not define demodulation requirements for “extended periodicity of the half frames with SS/PBCH blocks” as there is no PBCH conformance test in RAN5 specification. Proposal 5: RAN4 to discuss whether to introduce requirements for Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands. |

## Open issues summary

### Sub-topic 1-1 SAN/UE demodulation requirements

Issue 1-1-1: Broadcast service

- Proposals

  - No demodulation performance impact for broadcast service (Samsung)

- Recommended WF

  - Agree with the proposal

Issue 1-1-2: Regenerative payload

- Proposals

  - No demodulation performance impact for regenerative payload (Samsung)

- Recommended WF

  - Agree with the proposal

# Topic #2: SAN demodulation requirements

This topics discusses the SAN demodulation requirements.

## Companies’ contributions summary

See 1.1.

## Open issues summary

### Sub-topic 2-1	RedCap/eRedCap

This subtopic discusses the SAN demodulation requirements for RedCap/eRedCap.

Issue 2-1-1: SAN demodulation requirements for RedCap/eRedCap

- Proposals

  - No SAN demodulation requirements for the support of (e)RedCap UEs (Ericsson)

- Recommended WF

  - Agree with the proposal

### Sub-topic 2-2 Uplink capacity/cell throughput enhancements

This subtopic discusses the SAN demodulation requirements for uplink capacity/cell throughput enhancements.

Issue 2-2-1: PUSCH requirements for UL coverage enhancements

- Proposals

  - Defining SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes (Nokia, Samsung, Ericsson, Huawei)

- Recommended WF

  - Agree with the proposal

Issue 2-2-2: OCC parameters

- Proposals

  - Apply PUSCH type A repetitions (Samsung, Ericsson)

  - Apply inter-slot OCC length 2 (Samsung, Ericsson, Huawei)

    - Apply inter-slot OCC length 4 (Ericsson)

- Recommended WF

  - Apply inter-slot OCC length 2.

    - FFS for OCC length 4.

  - Apply PUSCH type A repetitions

Issue 2-2-3: Detailed PUSCH parameters

- Proposals

  - Option 1 (Samsung):

    - Consider one OCC group (the initial RV value can be configured as 0). FFS on two OCC groups with number of repetitions as 2 (the initial RV value for two OCC groups as 0, 2) (Samsung)

    - Number of repetitions: 2. FFS for 4 repetitions.

    - 15KHz, 5MHz and 30KHz, 10MHz

    - Both 1Rx and 2Rx, with test applicability rue for different Rx number

    - Same MCS for two UEs for PUSCH with OCC

    - Same channel model for two UEs for PUSCH with OCC

    - No frequency offset configured for two UEs as baseline assumption

      - FFS on frequency offset assumption

    - Consider the existing test parameters for PUSCH with DFT-s-OFDM waveform requirement as reference

  - Option 2 (Ericsson):

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled/Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2, n4 and n8 for OCC-length 2 n4, n8 and n16 for OCC-length 4 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled/Enabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

- Recommended WF

  - Need discussion for the detailed simulation assumption.

# Topic #3: UE demodulation requirements

This topics discusses the UE demodulation requirements.

## Companies’ contributions summary

See 1.1.

## Open issues summary

### Sub-topic 3-1 Downlink coverage enhancements

This subtopic discusses the UE demodulation requirements for the downlink coverage enhancements.

Issue 3-1-1: PDCCH with repetition for CSS

- Observations

  - PDCCH repetition enhancements for NTN are for CSS which we typically don’t define requirements for (Apple).

- Proposals

  - Option 1: Define requirements for PDCCH with repetition (Nokia, Samsung)

  - Option 2: Discuss if RAN4 should define requirements for PDCCH repetition (Apple, MediaTek)

  - Option 3: Not define requirements for PDCCH with repetition (Huawei)

- Recommended WF

  - Need discussion considering the observations.

Issue 3-1-2: PDSCH with repetition for Msg4/SIB1

- Observations

  - Apple: PDSCH repetition enhancements for NTN are for SIB1 and Msg4 which we don’t define requirements for.

  - MediaTek: Msg4 is part of the RACH process; it may require extensive testing time

  - MediaTek: There is no ACK/NACK feedback for SIB1

- Proposals

  - Option 1: Define requirements for PDSCH with repetition for Msg4 and SIB1 (Nokia, Samsung)

  - Option 2: Discuss if RAN4 should define requirements for PDSCH repetition (Apple)

  - Option 3: Not define requirements for PDCCH with repetition (Huawei, MediaTek)

- Recommended WF

  - Need discussion considering the observations.

Issue 3-1-3: PBCH periodicity extension

- Proposals

  - Not define requirements for PBCH extended periodicity (MediaTek)

- Recommended WF

  - Agree with the proposal

### Sub-topic 3-2 RedCap/eRedCap

This subtopic discusses the UE demodulation requirements for RedCap/eRedCap.

Issue 3-2-1: PDSCH demodulation requirements for RedCap

- Proposals

  - Option 1: Define PDSCH requirements for both RedCap 1Rx and 2Rx UEs (Ericsson)

  - Option 2: Define PDSCH requirements for RedCap 1Rx only (Apple, Huawei)

  - Option 3: Discuss the feasibility of defining PDSCH requirements for RedCap 2Rx only (Qualcomm)

  - Option 4: Discuss if RAN4 should define PDSCH requirements for RedCap (Samsung, MediaTek)

- Recommended WF

  - Discuss whether to define PDSCH requirements for RedCap over NR-NTN.

    - If defined, it is only for 1Rx only, 2Rx only, or both.

  - Moderator:

    - RedCap PDSCH requirements in TS38.101-4 are applicable for Rel-19 NTN access capable RedCap UE according to the UE capability. RAN4 need discuss whether we need additional requirements with NTN scenario such as channel models and HARQ feedback delay.

    - If defined, for RedCap 2Rx UE, the existing PDSCH requirements in TS 38.101-5 can be applicable for RedCap 2Rx UE because all the FR1-NTN PDSCH requirements for 2Rx in TS 38.101-5 uses 10MHz (52RB)

Issue 3-2-2: PDSCH demodulation requirements for eRedCap

- Proposals

  - Option 1: Define PDSCH requirements for both eRedCap 1Rx and 2Rx UEs (Ericsson, Apple)

  - Option 2: Define PDSCH requirements for RedCap 1Rx only (Huawei)

    - Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

  - Option 3: Discuss the feasibility of defining PDSCH requirements for eRedCap 2Rx only (Qualcomm).

  - Option 4: Discuss if RAN4 should define PDSCH requirements for eRedCap (Samsung, MediaTek)

- Recommended WF

  - Discuss whether to define PDSCH requirements for eRedCap over NR-NTN.

    - If defined, it is only for 1Rx only, 2Rx only, or both.

Issue 3-2-3: PDSCH for HD-FDD (if define PDSCH requirements)

- Proposals

  - Option 1: Define new reference channel for HD-FDD (e)RedCap with same performance requirements as for FDD (Qualcomm, Ericsson, Huawei)

- Recommended WF

  - Agree with option 1 if RAN4 define PDSCH requirements for (e)RedCap.

Issue 3-2-4: Test case of PDSCH for RedCap 1Rx (if defined)

- Proposals

  - Option 1 (Apple):

    - MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

    - MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

    - MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes

  - Option 2 (Ericsson):

    - MCS4, NTN-TDLA100-200, 1x1, 52RBs

    - MCS13, NTN-TDLC5-200, 1x1, 52RBs

  - Option 3 (Huawei): Similar test configurations as the existing PDSCH cases

- Recommended WF

  - Depending on Issues 3-2-1/3-2-2.

  - If defined, discuss whether to consider NTN-specific features such as HARQ disabling and 16/32 HARQ processes, in addition to the NTN channel models.

Issue 3-2-5: Test case of PDSCH for RedCap 2Rx (if defined)

- Proposals

  - Option 1 (Ericsson):

    - MCS4, NTN-TDLA100-200, 1x2, 52RBs

    - MCS13, NTN-TDLC5-200, 1x2, 52RBs

- Recommended WF

  - Depending on Issues 3-2-1/3-2-2.

  - Moderator: The proposal from Ericsson is same as the existing PDSCH requirements in TS 38.101-5 Table 8.2.1.2.2.1.1-3. RAN4 may discuss whether we can reuse these requirements for RedCap 2Rx UE.

Issue 3-2-6: Test case of PDSCH for eRedCap 1Rx (if defined)

- Proposals

  - Option 1 (Apple):

    - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

    - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

    - MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

    - MCS13, NTN-TDLA100-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

  - Option 2 (Ericsson):

    - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RBs)

    - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RBs)

  - Option 3 (Huawei): Similar test configurations as the existing PDSCH cases

- Recommended WF

  - Depending on Issues 3-2-1/3-2-2.

  - If defined, discuss whether to consider NTN-specific features such as HARQ disabling and 16/32 HARQ processes, in addition to the NTN channel models.

Issue 3-2-7: Test case of PDSCH for eRedCap 2Rx (if defined)

- Proposals

  - Option 1 (Apple):

    - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RB), Disabled HARQ

    - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes

    - MCS4, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 32 HARQ processes

    - MCS13, NTN-TDLA100-200, 1x2, without reduced BB BW (40RBs), 16 HARQ processes

  - Option 2 (Ericsson):

    - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RBs)

    - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RBs)

  - Option 3 (Huawei): Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

- Recommended WF

  - Depending on Issues 3-2-1/3-2-2.

  - If defined, discuss whether to consider NTN-specific features such as HARQ disabling and 16/32 HARQ processes, in addition to the NTN channel models.

Issue 3-2-8: Applicability of NGSO channel model

- Proposals

  - Discuss whether to apply the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements (Ericsson).

- Recommended WF

  - Postpone the discussion since NGSO channel modelling is still discussing in another WI.

Issue 3-2-9: PDCCH/PBCH/SDR/CSI requirements

- Proposals

  - Not define PDCCH, PBCH, and CSI reporting requirements for RedCap/eRedCap over NR-NTN (Apple, Ericsson).

  - Not define PDCCH, PBCH, SDR, and CSI reporting requirements for RedCap/eRedCap over NR-NTN (Qualcomm).

  - Existing CSI reporting requirements for RedCap in Rel-17 and eRedCap in Rel-18 are applicable for Rel-19 NTN access capable (e)RedCap UE according to the UE capability (Ericsson).

- Recommended WF

  - Not define PDCCH, PBCH, SDR, and CSI reporting requirements for RedCap/eRedCap UEs over NR-NTN.

  - Existing CSI reporting requirements for RedCap/eRedCap are applicable for Rel-19 NTN access capable RedCap/eRedCap UE according to the UE capability.