3GPP TSG-RAN WG4 Meeting #115	R4-2507875

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.28.5.2

Source:	Qualcomm Incorporated

Title: 		RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

RAN4 has been discussing various scenarios and issues related to RRM requirements for LP WUR/WUS over the past few meetings and achieved many agreements [2]-[8]. Furthermore, RAN4 received an LS from RAN1 [9] on the MR wake-up delay. In this paper, we present our views on RAN1 assumptions on the wake-up delay and other open issues.

# MR wake-up delay

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

The first issue that we want to address is related to MR wake-up delay in IDLE/INACTIVE mode. RAN1’s agreement from the LS [9] is copied below:

It’s clear from RAN1 discussions and agreements that the MR wake-up delay constitutes of two components, MR warm up delay and synchronization. The three candidate values for the wake-up delay assume 10ms, 400ms and 800ms and 3, 5 and 5 SSBs for synchronization delay respectively. RAN4 discussed these values in the last meeting and companies had different opinions on the number of SSBs needed for synchronization

Firstly, we think that the number of SSBs needed for synchronization should be same for all wake-up delays. This is because the synchronization requirements are on the MR and not on the WUR. At-least in Rel-19 we are not talking about different MR implementations, so there is no reason to assume different number of SSBs for different ramp-up times. One of the reason that a few companies gave was that the AGC may need more sample, but this may not be true since it’s safe to assume that the serving (or camped) cell SNR is pretty high and the channel conditions are rather stable for a UE to operate in full offloading or relaxed measurements mode, one of which is assumed to be the case when the WUR is active. In such cases, we don’t expect too much variation in signal power and past AGC states can be used as initial states and perhaps one sample is sufficient to fine-tune AGC. The time/frequency synchronization should also need only 1 SSB at high SNR operating conditions, otherwise, the UE may exit the WUR mode. So, in total 2 SSBs are more than sufficient for time/freq synchronization and AGC tuning.

Observation 1: LP-WUR operation can be assumed to be enabled only during high SNR conditions, where the MR may not need too many samples for AGC or time/freq sync.

Proposal 1: RAN4 to consider a common number of SSBs for synchronization part of the MR wake-up delays. The number of SSBs is 2.

# LS on the RRM measurement metrics for OFDM-based LP-WUR

In RAN1, the following agreements were made on the RRM measurement metrics based on SSS for OFDM-based LP-WUR:

Agreement (RAN1#120)

Confirm the following working assumption with the additional text in red:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

- The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement.

- FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

Agreement (RAN1#120bis)

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

From RAN1 perspective, it was agreed that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell. RAN1 respectfully asks RAN4 to confirm it.

RAN1 sent another LS [10] during the last meeting on RRM measurement metrics to be used by the OFDM-based LP-WUR and the applicability of SMTCs for performing measurements. In short, RAN1 asked RAN4 to confirm the RAN1 agreement that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurements performed by the OFDM-based LP-WUR for the serving cell.

Issue 1-2-2: Periodicity for LP-WUR RRM requirements

Agreement:

Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

In RAN4, the main use-case of the LP-SSS-RSRP/RSSI measurements is to evaluate the entry/exit criteria for LP-WUS monitoring and/or measurement relaxations/offloading. RAN4 has extensively discussed these requirements and has already agreed that the measurement periodicity is based on LO periodicity. So, we think RAN4 should reply to RAN1 LS with the following RAN4 agreement:

Observation 2: RAN4 has already agreed to use LO periodicity as the measurement granularity for OFDM based LP-WUR measurements.

Proposal 2: Send a reply to RAN1 LS on the RRM measurement metrics for OFDM-based LP-WUR (R1-2503103/ R4-2505309) with the following RAN4 agreement:

Periodicity for LP-WUR RRM requirements

Agreement (RAN4#114bis): Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

# LP-WUR operation with eDRX

In last RAN2 meeting, RAN2 discussed LP-WUS with eDRX and had the following agreement.

Agreement: LP-WUS is supported with eDRX, FFS on exact impact if any.

Although RAN4 didn’t receive any LS from RAN2 on this regard, but given the limited amount of time left for this WI, perhaps RAN4 should discuss whether we need to specify any requirements for LP-WUR operation with eDRX.

In our understanding, a UE configured with an eDRX cycle longer than 10.24s monitors the POs only within a Paging Time Window (PTW) configured to the UE. RAN4 specifies serving and neighbor cell measurements and cell-reselection criteria evaluation periods under the assumption that all measurements are performed within the PTW, even for the case when measurements are performed at a relaxed rate. With full offloading, RAN4 may need to discuss how frequently measurements are performed by the WUR.

Furthermore, the power savings gain may not be substantial with long eDRX cycles, and in some cases may see negative power savings if the WUR is required to perform measurements more frequently. Note that even with neighbor cell measurement relaxation scenarios introduced in R16 and 17, RAN4 decided to limit the applicability of relaxations only upto eDRX cycle length of 163.84s.

So, given that the LP-WUS operation with long eDRX cycles (>10.24s) comes with the increased complexity and workload, and provides little to no power savings, we think it makes sense to limit the use of LP-WUS only to eDRX cycles up-to 10.24s.

Observation 3: RAN2 agreed that LP-WUS is supported with eDRX.

Proposal 3: Discuss whether RAN4 needs to introduce RRM requirements for LP-WUR operation with eDRX.

# LP-WUR operation in Connected mode

The next issue is related to WUR implementation. Although 3GPP may assume that the WUR is completely detached from the MR, it’s highly likely that UE may share some RF components between the two radios. So, for CONNECTED mode, when the UE uses measurement gaps for neighbour cell measurements, it may tune away from the serving cell. This typically causes interruptions on the UE where the UE cannot transmit or receive any signal other than those intended for RRM measurements. We think such interruptions may also be needed for WUS monitoring when the WUS MO collides with the MG. Similarly, there are other interruption requirements defined in Clause 8.2 of 38.133 where the UE is not expected to transmit or receive any signal. These interruption requirements should also apply to WUS monitoring by the WUR in Connected mode.

Observation 4: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 4: In the Connected mode, interruptions on the serving cell due to MGs as defined in Clause 9.1.2 also apply to WUS monitoring by the WUR.

Proposal 5: In the Connected mode, interruption requirements specified in Clause 8.2 also apply to WUS monitoring by WUR.

# Conclusion

Observation 1: LP-WUR operation can be assumed to be enabled only during high SNR conditions, where the MR may not need too many samples for AGC or time/freq sync.

Proposal 1: RAN4 to consider a common number of SSBs for synchronization part of the MR wake-up delays. The number of SSBs is 2.

Observation 2: RAN4 has already agreed to use LO periodicity as the measurement granularity for OFDM based LP-WUR measurements.

Proposal 2: Send a reply to RAN1 LS on the RRM measurement metrics for OFDM-based LP-WUR (R1-2503103/ R4-2505309) with the following RAN4 agreement:

Periodicity for LP-WUR RRM requirements

Agreement (RAN4#114bis): Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Observation 3: RAN2 agreed that LP-WUS is supported with eDRX.

Proposal 3: Discuss whether RAN4 needs to introduce RRM requirements for LP-WUR operation with eDRX.

Observation 4: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 4: In the Connected mode, interruptions on the serving cell due to MGs as defined in Clause 9.1.2 also apply to WUS monitoring by the WUR.

Proposal 5: In the Connected mode, interruption requirements specified in Clause 8.2 also apply to WUS monitoring by WUR.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103

[2] R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#110bis

[3] R4-2410296, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#111

[4] R4-2413899, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112

[5] R4-2416861, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112bis

[6] R4-2420100, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#113

[7] R4-2502600, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#114

[8] R4-2504908, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#114bis

[9] R1-2501624, “LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode”, RAN1#120

[10] R1-2503203, “LS on the RRM measurement metrics for OFDM-based LP-WUR”, RAN1#120bis