3GPP TSG-RAN WG4 Meeting #115	R4-2507877

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.30.3

Source:	Qualcomm Incorporated

Title: 		RRM requirements for XR enhancements

Document for:	Discussion

# Introduction

RAN4 achieved multiple agreements over the last few meetings on DCI based MG cancellation mechanism introduced by RAN1. In this contribution paper, we further discuss the remaining few issues identified in the WF [1] from the last meeting.

# Type of measurement gaps

RAN4 has discussed the applicability of the cancellation DCI to various types of MGs for many meetings. Here’s what we agreed during the last meeting:

<Background>: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

Based on the recent discussions, pretty much every company supports Type-1 and Type-2 MGs. After agreeing to not apply MG cancelling DCI to MUSIM and Positioning gaps, we are left with the following ones:

- Gap(s) configured via GapConfig-r17 with preConfigInd-r17 or ncsgInd-r17

- Concurrent MGs

In our understanding, pre-configured MGs are regular MGs which either the UE or network may activate or deactivate on a need basis, e.g., after activation of an SCell or when a new MO is added. When controlled by network using preConfGapStatus, network is aware of whether the pre-configured MG is activated or deactivated, but when the gap status is autonomously determined by the UE, network may not know of the pre-MG status, i.e., whether it’s activated or not. In this case, network may not decide whether to send a DCI or not. So, to avoid such complications, we are okay to preclude MGs configured via GapConfig-r17 with preConfigInd-r17 to be cancelled by the DCI.

Observation 1: Network may not be aware whether a pre-configured MG is activated or deactivated if UE autonomously determines the status.

Proposal 1: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17

Next is MGs configured via GapConfig-r17 with	ncsgInd-r17. The idea behind introduction of NCSG was to reduce the interruption on data to VIL1 and VIL2 portion of the MG, and that no interruption in needed during the ML portion of the MG. Since NCSG do not cause big interruption on the data, so we think it’s okay to deprioritize these as well.

Observation 2: NCSG do not cause big interruption on data and may not impact the XR QoS as much.

Proposal 2: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17

Finally coming to concurrent MGs, if the UE is configured with concurrent MGs, it is prone to more frequent interruptions. So, we think it’s important to support their cancellation as long as none of the colliding MGs are configured with preConfigInd-r17 or ncsgInd-r17. The main issue here is which MG should the DCI cancel when the two MGs collide with each other. We think this is not a big problem as the collision handling rules between concurrent MGs are already specified (TS 38.133, Clause 9.1.8.3). So, it’s straightforward that the DCI cancels the MG that survives (i.e., the MG configured with higher priority) based on the specified collision handling rules. Since the other MG (i.e., the one with low priority) is already dropped, this effectively means that the DCI cancels both of the colliding MGs..

Observation 3: Concurrent MGs may cause more frequent data interruptions at the UE.

Proposal 3: MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

- Note that the other MG is already dropped, so a cancelling DCI would effectively mean that both the colliding MGs are dropped.

Another issue that some companies raised during the last meeting was related to the timeline of cancellation (X) in this case. But that is also not that big of a problem. Since the network already knows which MG has higher priority and is going to survive the collision handling rules, it can transmit DCI X ms before this gap.

Proposal 4: The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules.

# Impact on RRM requirements

Since RAN4 agreed that the MG cancellation applies only to L3 measurements and to follow NR-U-like approach for designing the formulas for measurement delay extension, we now need to discuss the exact formula to extend the L3 measurement requirements. We tend to think that it’s straightforward to use NR-U approach and add the number of skipped MGs to the number of SSB samples for each requirement.

Proposal 5: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

As an example, Table 9.2.6.2-1 can be modified as below:

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, (5+Nskip) x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x (5+Nskip) x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil((5+Nskip) x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |

Some companies also brought various conditions to enable the MG skipping. These included – maximum number of MGs skipped during the measurement period, maximum acceptable time separation between two non-cancelled MGs etc. We are okay to specify the limit on maximum number of skipped MGs within a measurement period and maximum acceptable time separation between two non-cancelled MGs as a part of the requirements but since we agreed to indicate the preferred percentage of MG cancellations within a time period as a UAI, perhaps it’s also okay to not specify such conditions.

Proposal 6: Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

Regarding the applicability of the MG cancelation in CA, we think it’s straightforward that the indication applies to all the carriers in CA mode if the gap is per UE-gap, and the indication applies to all carriers on the FR where the indication is received if the MG is per-FR.

Proposal 7: If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

# Semi-static schemes

Although RAN1 has agreed upon the dynamic indication to enable Tx/Rx in particular gaps, RAN2 may further consider semi-static indication schemes as well, e.g., RRC based MG deactivation pattern. We understand that due to unpredictable nature of the XR data, dynamic indication is better suited from signaling perspective but at the same time, from UE implementation perspective, it may be quite challenging to deactivate a single MG occasion as this needs extensive processing on the modem SW and RF procedures. RRC/MAC-CE based semi-static indications enables the UE to handle MG deactivations in an efficient manner.

Proposal 8: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC or MAC-CE based signaling as well.

# Conclusion

Observation 1: Network may not be aware whether a pre-configured MG is activated or deactivated if UE autonomously determines the status.

Proposal 1: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17

Observation 2: NCSG do not cause big interruption on data and may not impact the XR QoS as much.

Proposal 2: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17

Observation 3: Concurrent MGs may cause more frequent data interruptions at the UE.

Proposal 3: MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

- Note that the other MG is already dropped, so a cancelling DCI would effectively mean that both the colliding MGs are dropped.

Proposal 4: The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules.

Proposal 5: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

Proposal 6: Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

Proposal 7: If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

Proposal 8: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC or MAC-CE based signaling as well.

# References

[1] R4-2504911, “WF on RRM requirements for XR_Ph3”, Nokia, RAN4#114bis