3GPP TSG-RAN WG4 Meeting # 115	R4-2507884

# Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on introduction of FR2 PC8

Source: 	Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC

Agenda item:	5.3.1

Document for:	Discussion

- Background

Due to Ericsson’s continue objection but without any better proposal on how to change the current FR2 industry very depressed situation for almost ten years, WF [1] was still noted finally, although just several options are listed. To promote the FR2 industry prosperous development, we still think that it is beneficial to diversify the UE types, in this contribution, we continue share our consideration and proposal on the necessity and benefit of introduction of FR2 PC8 for the FR2 industry development.

- Discussion

Issues of FR2 PC3

5G Single FR2 handheld UE type PC3

5G is the first generation for FR2 development, handheld UE is one of the most important use cases for mobile communications. Considering e.g., most of the new proposals for UE RF and cross domain in R20 RAN4 led WI for enhancements are for FR1 and commercial deployments in the fields, but 5G FR2 is still on its road to success. One of the key reasons is that the number of commercially available UE is limited. At the same time only one type of FR2 handheld UE, i.e., PC3 was introduced from Release 15 till now. This is unusual and not reasonable, because handheld UE with FR1, its evolution is step by step by starting from lower UE capability, such as the number of antenna, 1Rx is assumed in 2G, larger number of antenna is only considered in later stage, which is consistent with the industry development pace. But handheld UE for FR2 is assumed 2 panels with 1x4 antenna array for each panel from the first release, which is higher request from cost, complexity and power consumption point of view. handheld UE with less number of panel and/or smaller antenna array, such as 1 panel with 1x2 antenna array, should be considered for mmWave in 5G.

Issue #1: High Cost

For any good industry development, cost is one of the key factors that is seriously considered by all vendors involved in this industry, including UE vendors, BS vendors and operators.

As per our evaluation, addition of one FR2 band supporting PC3, the cost is more than 2 times than that for addition one FR1 band. But considering the current limited FR2 network deployment that mainly covers hotspot area, the usage rate is lower compared to that of FR1. “But a new use case will not appear if the implementation/operations cost exceeds the business value of the use case.” [3]

Observation 1: Higher cost and lower usage rate would lead to block the FR2 industry development.

Issue #2: Serious Power Consumption

Due to multiple analog RF chains and complex beam management, power consumption of FR2 PC3 is much higher than that of FR1, which further may lead to overheating issue. But power consumption of FR2 PC8 can be half compared to FR2 PC3. We did complete evaluations on the power consumption of FR2 PC3, with the assumption of FR1 100MHz/30kHz SCS, FR2 200MHz/120kHz SCS and FR2 400MHz/120kHz SCS as shown below:

| DL peak data rate（FR1: 1.6Gbps FR2: 970Mbps/1.8Gbps） | DL fixed data rate（FR1: 100 Mbps FR2: 100 Mbps） |
| --- | --- |
| PDCCH only | UL fixed data rate（FR1: 10 Mbps FR2: 10 Mbps） |

From the above evaluation results, we can know:

Observation 2: Power consumption of FR2 PC3 is 3~4 times of FR1, which further may lead to overheating issue, while that of FR2 PC8 is half compared to that of FR2 PC3.

Issue #3: Complex Beam Management

Other reason that PC3 doesn’t become popular is due to the complex beam management procedure which is not very flexible. Beam acquisition is slow and time-consuming. FR2 PC3 have two AIPs each with 1x4 antenna array + IF (Intermediate frequency). FR2 UE selects one AIP (Panel) out of two, and 8 RF analog chains per each AIP work simultaneously for reception and transmission. With such UE architecture, during the beam management, if we assume each AIP supports 7 Rx beams, BS has 16 SSB Tx beams, then FR2 PC3 UE needs to measure around 160 pairs of Tx-Rx beams for each beam management at least. This complex beam swiping procedure results in large delay and serious power consumption, ultimately can lead to bad user experience and/or discouragement of implementing FR2 PC3 into handheld UEs.

Observation 3: Complex beam management procedure results in large delay and higher power consumption, then bad user experience and/or discouragement of implementing FR2 PC3 into handheld UEs.

Advantage of FR2 PC8

Benefit #1: Low cost

FR2 PC8 can meet the real most application request with reasonable cost, i.e. with the similar cost of addition one FR1 band by assuming one panel with 1x2 antenna array for each panel.

Benefit #2: Low complex beam management

One panel with 1x2 antenna array for each panel, the AIP almost can have the spatial coverage like an omni-directional antenna. Assuming BS has 16 SSB Tx beams, FR2 PC8 UE needs do measurements for less than 20 pairs of Tx-Rx beams for each beam management, which is 1/10 of PC3.

Benefit #3: Low power consumption

Simple beam management and less number of AIP with less pairs of X-pol antenna elements will bring half power consumption of FR2 PC3, which can resolve the overheating issues and bring good user experience.

Benefit #4: Good performance

As per our evaluations, FR2 PC8 can well meet most application requests:

| SLS performance evaluation (UPT) for UL/200MHz | SLS performance evaluation (UPT) for UL/400MHz |
| --- | --- |
| SLS performance evaluation (UPT) for DL/200MHz |  |

Table 1: TP(Mbps)@5%,50% and 95% percentile for UL/DL with FTP traffic model

| CBW | 200MHz, 150m, UL |  |  |  | 400MHz, 150m, UL |  |  |  | 200MHz, 150m, DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50% Spherical coverage(dBm) | 5.5 | 6.5 | 7.5 | 8.5 | 5.5 | 6.5 | 7.5 | 8.5 | 5.5 | 6.5 | 7.5 | 8.5 |
| TP@5% Percentile | 6.58 | 7.59 | 8.95 | 11.8 | 10.69 | 11.91 | 14.86 | 18.33 | 674.4 | 669.4 | 680.9 | 696.3 |
| TP@50% Percentile | 91.0 | 97.58 | 107.4 | 120.5 | 174.48 | 187.75 | 204.67 | 224.26 | 1352.95 | 1355.1 | 1362.4 | 1368.5 |
| TP@95% Percentile | 260.0 | 273.55 | 282.9 | 292.0 | 512.50 | 519.62 | 535.09 | 548.60 | 1518.5 | 1521.7 | 1524.2 | 1523.8 |

Observation 4: FR2 PC8 can well meet health FR2 industry development with low cost, low complex beam management and lower power consumption, which bring good user experience.

FR2 industry development

Status of FR2 industry

3GPP introduced FR2 from 5G first Release 15, i.e. from year 2017. Now the clock is pointing to 2025, 9 years passed. mmWave industry development is still on the way to the originally expected success due to some factors, such as:

- The limited terminal types for handheld UE

- The limited available number of commercial terminals

- The high cost to enable FR2 function in both terminal and network

- The very lower usage rate due to very limited FR2 network deployment

- The very higher power consumption of FR2 PC3 compared to that of FR1.

More vendors’ deep involvement is essential for FR2 industry mature. Now in some regions and/or countries, there are companies that gave up FR2 product design and development due to no clear market request and ROI. As a result, engineers who are engaged in FR2 product transferred to other fields. In order to reverse the negative spiral, we believe that it is time to do something to change the current situation. Some companies don't like FR2 PC8 just because they worry about FR2 PC8 will become popular and will replace FR2 PC3, and then their network can’t show better performance to operators and customers than before. But FR2 PC8 mainly targets for low/mid-end devices and no intention to replace FR2 PC3, because FR2 PC3 is for high-end device. To differentiate UE capability, different power class will be reported to network, and access control scheme can be introduced as did for FR2 PC7 and 2Rx XR which give operators choices to control the access of FR2 PC8. From specification point of view, different UE capabilities should be allowed to cate for real diverse application request instead of closing the door to whom needs PC8.

Observation 5: Suspending market growth would lead to a negative spiral situation such that more device vendors hesitate supporting FR2 or more operators hesitate deploying FR2 network or vice versa

Some companies’ view on FR2 during 6G Workshop

With the lessons learned from the past FR2 industry development in 5G ear, during 6G workshop, some companies acknowledge the meaning to introduce diverse terminal types to cate for different use cases, i.e. low/mid/high-end terminals, to better promote mobile communication industry.

From the 6G workshop contributions, we simply collected some operator’s view on the FR2 utilization:

| Company | Contribution | View |
| --- | --- | --- |
| Deutsche Telekom's | 6GWS-250011 | P4: Spectrum for 6G - > 24GHz: “FR2” and (lower) THz: Lower priority for DT, but 6G assumed to work up to 71 GHz as NR does - {7-24GHz} “FR3”: No potential for Europe currently seen |
| KT Corp. | 6GWS-250021 | P2: 6G spectrum: Upper mid-band（7.125-8.4GHz）to be prioritized for 6G RAN specification developments |
| NTT DoCoMo | 6GWS-250052 | P3: Spectrum aspects - FR2: Despite the huge hard work in NR standard/field, FR2 operation still need improvements in real fields - View: Both FR2 and FR3 should be equally considered for 6G specification work, to achieve real improvements in many regions P8: 6G performance gain – FR2 - FR2 in NR: Expected to improve throughput/capacity, while not successfully deployed - Difficulties for commercialization: Significant propagation loss, LOS-dominant radio - FR2 in 6G: Still very important for throughput/capacity improvement - Availability of 6G new bands (e.g., FR3) is not clear in some regions (e.g., Japan) - Key points for 6G FR2: How to overcome the difficulties? - Direction#1: Usage of more beams and TRPs, to overcome propagation loss and increase LOS probability - Direction#2: Optimize TRP selection, esp. around cell-edge area - Evaluation results show that in FR2, both directions are beneficial (Direction#1=dense (2x TRP), Direction#2=cell free (HO margin=0dB) - For 5%-tile: - Direction#1 with 2x TRPs achieves 78% gain - Direction#2 achieves 20% gain - Direction#1 (with 2x TRPs) + Direction#2 achieves 87% gain P14: FR1: coverage band. Target design: further improved throughput/capacity, utilizing fragmented small spectrum FR3: coverage and capacity band. Target design: better performance than FR1, easier deployment than FR2 FR2: Capacity band. Target design: more practical deployment |
| Verizon | 6GWS-250075 | P11: FR2 optimization for robustness and coverage: Seamless multi-carrier operation cross FR2, FR1 and FR3 P13: FR2 specific: - Advanced antenna system with beam management and PA enhancement - Enhanced multi-frequency operation with FR1 and FR3 |
| Ericsson | 6GWS-250084 | P4: SA only, migration of spectrum with MRSS – highly efficient 6G-5G sharing absolutely essential in (primarily) FR1 and FR2 |
| Orange | 6GWS-250130 | P4: - mmWave (26GHz): still uncertain; business case not better in 6G vs 5G |
| Vodafone | 6GWS-250153 | P2: Key expectations - Works well in the upper 6GHz band - No inter-RAT DC, CA of upper 6GHz with mid&low band is essential for network performance P5: Consider energy consumption for all 6G components and features |
| Rakuten Mobile, Inc | 6GWS-250236 | P5: Enhanced User Experience in 6G RAN Multi-Band Carrier Aggregation → Optimized resource use across FR1, FR2, FR3. |

From the summary above, we can get the following observations.

Observation 6:

- SA v.s. EN-DC: Most operators dislike the 4G-5G inter-RAT DC

- FR2:

  - Some operators set low priority for FR2 considering the lessons learned from 5G FR2 network deployment.

  - FR2 is mainly used for throughput/capacity improvement in the hotspots area.

  - Good user experience is the ultimate target for all feature introduction

  - Energy saving in both UE and BS sides should be considered from Day 1 of 6G

  - CA across FR1, FR2 and FR3 should be considered to ensure optimized resource use

  - More practical deployment: such as with more beams or TRPs, cell-free and etc., to ensure no cell-edge performance degradation

Reduced UL Coverage

The scenario mentioned [5] is under FR1-FR2 NR-DC deployment scenario, UE needs to keep 2 uplinks all the time, 2 uplinks split UE power and limit the UL coverage. So the coverage issue analysed by some companies is not due to the lower UE transmit power, the network deployment scheme also impacts the real UL coverage.

In practice DC is a “switch”, not an aggregator [3]. If it is real “switch”, the total capacity will be impacted.

Observation 7: The 2 uplinks under NR-DC split UE transmit power and reduce the UL coverage.

We highly acknowledge the views from some company for the business success, “Evolution based on real market needs – use cases will not happen if the implementation/operations cost exceeds the business value of the use case.” [3]

We think that FR2 PC8 seriously considers the business value and cost with performance to ensure good user experience.

Observation 8: Introduction of FR2 PC8 is beneficial for mmWave industry development

Compromised solution

If some company still have concern on the value of introduction of FR2 PC8, it is fair and straightforward way to introduce the access control scheme as did for FR2 PC7 and 2Rx XR.

So for the listed options in [1], we are fine with Option 2:

- Option 2: As a compromise, RAN4 introduces the similar access control scheme as 2Rx XR and RedCap for FR2 PC8 with the min peak EIRP = 16.4 dBm and EIRP spherical coverage@50%-ile CDF = 5.5 dBm

  - intraFreqReselectionFR2PC8

  - cellBarredFR2PC8

Proposal 1: Introduce FR2 power class 8 with the access control scheme and with the min peak EIRP = 16.4 dBm and EIRP spherical coverage@50%-ile CDF = 5.5 dBm

- intraFreqReselectionFR2PC8

- cellBarredFR2PC8

- Conclusion

As per our evaluation results shown above, our observations and proposals are summarized as following:

Issues of FR2 PC3:

Observation 1: Higher cost and lower usage rate would lead to block the FR2 industry development.

Observation 2: Power consumption of FR2 PC3 is 3~4 times of FR1, which further may lead to overheating issue, while that of FR2 PC8 is half compared to that of FR2 PC3.

Observation 3: Complex beam management procedure results in large delay and higher power consumption, then bad user experience and/or discouragement of implementing FR2 PC3 into handheld UEs.

Advantage of F2 PC8:

Observation 4: FR2 PC8 can well meet health FR2 industry development with low cost, low complex beam management and lower power consumption, which bring good user experience.

FR2 industry development:

Observation 5: Suspending market growth would lead to a negative spiral situation such that more device vendors hesitate supporting FR2 or more operators hesitate deploying FR2 network or vice versa

Observation 6:

- SA v.s. EN-DC: Most operators dislike the 4G-5G inter-RAT DC

- FR2:

  - Some operators set low priority for FR2 considering the lessons learned from 5G FR2 network deployment.

  - FR2 is mainly used for throughput/capacity improvement in the hotspots area.

  - Good user experience is the ultimate target for all feature introduction

  - Energy saving in both UE and BS sides should be considered from Day 1 of 6G

  - CA across FR1, FR2 and FR3 should be considered to ensure optimized resource use

  - More practical deployment: such as with more beams or TRPs, cell-free and etc., to ensure no cell-edge performance degradation

Observation 7: The 2 uplinks under NR-DC split UE transmit power and reduce the UL coverage.

Observation 8: Introduction of FR2 PC8 is beneficial for mmWave industry development

Compromised Solution:

Proposal 1: Introduce FR2 power class 8 with the access control scheme and with the min peak EIRP = 16.4 dBm and EIRP spherical coverage@50%-ile CDF = 5.5 dBm

- intraFreqReselectionFR2PC8

- cellBarredFR2PC8

- Reference

[1] R4-2505232 WF on FR2 PC8, RAN4#114bis, Huawei, HiSilicon

[2] R4-2504322 Discussion on introduction of FR2 PC8, RAN4#114bis, Huawei, HiSilicon

[3] 6GWS-250084 Overall vision & priorities for RAN in 6G, Ericsson

[4] R4-2503175	On the proposed FR2 PC8 (new power class) UE, RAN4#114bis, Qualcomm