3GPP TSG-RAN WG4 Meeting#115                                                                                                     R4-2507913

Malta , MT, 19th – 23th May, 2025

Agenda item:			7.26.1

Source:	CMCC

Title:	WF on A-IOT device requirements

Document for:	Approval

Introduction

This way forward captures the agreements on A-IOT system parameters, device RF requirements and OTA test method. The summary document for RAN4#115 is R4-2507573.

The way forward agreed in previous RAN4 meeting is R4-2505230.

System parameters

- Topic 2-1: R2D bandwidth

Issue 2-1-1: R2D transmission bandwidth

Agreement in RAN4#115:

- Define symmetric guard band as 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration

- Topic 2-2: D2R bandwidth

Issue 2-2-1: D2R bandwidth

Agreement in RAN4#115:

- Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Assumed 10% SFO for device

  - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered.

- The value of transmission BW and channel BW will be explicitly listed.

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

- The following are for information:

- Figure 1-1 Small frequency shift diagram of FDMA without SFO - Figure 1-2. SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scales the small frequency shift value . - The 2SB transmission BW and small frequency shift _both measured in kHz——with and without SFO are as follows: - Note : BW and Small frequency shift are in kHz;Tc and Tb are in μs 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).

- Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement in RAN4#115:

-	Define 10KHz channel raster

-	FFS on channel raster offset

Device RF requirements

- Topic 3-1: Transmit output power

Issue 3-1-1: Transmit output power

Agreement in RAN4#115:

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

- Topic 3-2: Modulation quality

Issue 3-2-1: SFO requirement

Agreement in RAN4#115:

- No need to directly test SFO if SFO can be indirectly verified via other test.

- FFS whether to explicitly define SFO requirements or not.

Issue 3-2-2: Modulation quality requirements

Agreement in RAN4#115:

- The modulation quality won’t be introduced, and it can be verified by the backscattering loss for OOK or BPSK.

- Topic 3-3: Emission requirements

Issue 3-3-1: SEM requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- The applicable frequency range for SEM: from the CBW edge to [OOB boundary]

- Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

Issue 3-3-1: spurious emission requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- OOB boundary

  - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth

  - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW

  - Other options are not precluded

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

- Topic 3-4: Reference sensitivity

Issue 3-4-1: Reference sensitivity

Agreement in RAN4#115:

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

- FFS for the FRC, consider following as starting point:

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

- Topic 3-5: Others

Issue 3-5-1: Maximum input power

Agreement in RAN4#115:

- Define maximum input level at peak antenna direction, using [-4dBm] as starting point.

Issue 3-5-2: Occupied bandwidth

Agreement in RAN4#115:

- Do not define occupied bandwidth requirements.

OTA test method

- Topic 4-1: General

Issue 4-1-1: testing frequencies

Agreement in RAN4#115:

- RAN4 agreed to test all low, middle and high frequency ranges, and leave RAN5 to determine testing frequency range.

- Topic 4-2: Test setup

Issue 4-2-2: Isolation between CW node and measurement antenna

Agreement in RAN4#115:

- Power difference between received backscatter signal and the CW signal leakage is at least [15dB]

  - FFS on reference point of power difference

Issue 4-2-3: Polarization of CW antenna

Agreement in RAN4#115:

- The CW signal shall be sent in two polarized sequentially, FFS on how to combine two polarization test results.

- Topic 4-5: MU

Issue 4-5-1:MU assessment

Agreement in RAN4#115:

- Use following MU as starting point

Table 1 MU for Tx requirements

| UID | Description of uncertainty contribution | Reuse or not |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of receiver chain | reused |
| 2 | Insertion loss of receiver chain | reused |
| 3 | Influence of the measurement antenna cable | reused |
| 4 | Measurement Receiver: uncertainty of the absolute level | [-0.65] |
| 5 | Measurement distance | reused |
| 6 | Quality of quiet zone | reused |
| 7 | DUT Tx-power drift | FFS |
| 8 | Uncertainty related to the use of phantoms | No needs |
| 9 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 10 | Random uncertainty | reused |
| 11 | Frequency Response | reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | reused |
| 13 | Mismatch of receiver chain | reused |
| 14 | Insertion loss of receiver chain | reused |
| 15 | Mismatch in the connection of calibration antenna | reused |
| 16 | Influence of the calibration antenna feed cable | reused |
| 17 | Influence of the measurement antenna cable | reused |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | reused |
| 19 | Measurement distance | reused |
| 20 | Quality of the Quiet Zone | reused |
| Systematic Errors |  |  |
| 21 | Systematic Error related to TRP grids | reused |

Table 2 MU for Rx requirements

| UID | Description of uncertainty contribution for sensitivity | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | Reused |
| 2 | Insertion loss of transmitter chain | Reused |
| 3 | Influence of the measurement antenna cable | Reused |
| 4 | R2D for sensitivity measurement: uncertainty of the absolute output level | 0.27 |
| 5 | Sensitivity measurement: output level step resolution | Reused |
| 6 | Measurement distance | Reused |
| 7 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| 8 | DUT sensitivity drift | 0.12 |
| 9 | Uncertainty related to the use of phantoms | No needs |
| 10 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 11 | Random uncertainty | Reused |
| 12 | Frequency Response | Reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | Reused |
| 14 | Mismatch of transmitter chain | Reused |
| 15 | Insertion loss of transmitter chain | Reused |
| 16 | Mismatch in the connection of calibration antenna | Reused |
| 17 | Influence of the calibration antenna feed cable | Reused |
| 18 | Influence of the measurement antenna cable | Reused |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | Reused |
| 20 | Measurement distance | Reused |
| 21 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| Systematic Errors |  |  |
| 22 | Systematic Error related to TRS grids | Reused |