3GPP TSG-RAN WG4 Meeting #115	R4-2507933

Malta, Malta, May 19-23, 2025

Agenda item:			7.7.4

Source:	Apple

Title:	WF on UE RF requirements for LB CA via switching

Document for:	Approval

# Way Forward

Issue 1: Agreements related to the time mask requirement

-	A text proposal to TR38.768 in R4-2507932 captures the relevant agreements

-	How to specify the DL time mask:

-	The UE DL behaviour following the switch operation can be discussed in the RRM session

-	Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1

-	RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered.

Issue 2: Agreements related to UE capabilities

-	A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw

-	Related to concurrent Rx operation, the following aspect is included in the feature group:

-	[The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.]

-	Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

-	When network receives the per-BC low band CA via switching capability, the RRC configuration of the switching pattern should be configured.

-	Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us