3GPP TSG-RAN WG4 Meeting #115									                          R4-2507935

St Julian’s, Malta, 19-23 May 2025

Agenda item:			7.1.3

Source:	Skyworks, Murata Manufacturing Co Ltd., Nokia, Samsung, Qualcomm, Apple, Vivo, ZTE, Mediatek, Huawei, Oppo (…)

Title:	WF on new MSD rules

Document for:	Approval

# 1		Introduction

This WF captures the agreements for 2UL MSD simplification reached during ad-hoc meetings and based on proposals [1..10].

# 3		WF on inter-band CA MSD HPUE simplification with 2 UL band 2 UL CC.

Agreement 1 on MSD Look-Up Tables (LUT):

- Scope: In Rel-19, the inter-band CA MSD simplification with 2UL band 2UL CC only applies:

  - To band configurations with 1UL CC per band. 2UL band configurations with 3UL CCs are out of scope, e.g. 1UL CC in one band + 2UL CC in another band,

  - To PC2 and PC1.5 for a total of 2Tx or 3Tx inter-band uplink CA covered up to Rel-19. An example of power class and number of Tx per band in scope of this MSD simplification is captured in the table below from the agreed WF [11].

| Case # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class / #Tx | Type | PC2 | PC2 | PC2 | PC2 | PC2 | PC1.5 | PC1.5 | PC1.5 | PC1.5 |
|  | # TX | 2Tx | 2Tx | 3Tx | 2Tx | 3Tx | 2Tx | 3Tx | 3Tx | 3Tx |
| per band implementation | Band 1 | PC3 | PC3 | PC3 1Tx | PC2 | PC2 1TX | PC2 | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Band 2 | PC3 | PC2 | PC2 2Tx | PC2 | PC2 2Tx | PC2 | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |

2Tx or 3Tx inter-band UL CA combinations where at least one of the constituent band is band n46, n96, or n102 are out of scope.

- For new Rel-19 inter-band CA combinations, and for all valid 2UL band 2UL CC HPUE configurations, specify the 2 band 2 UL CC HPUE MSD requirements due to dual-UL CA intermodulation interference as:

PCx MSD = PC31Tx MSD + MSD

where,

- PCx = PC2, or PC1.5,

- PC31Tx MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1, and,

- MSD is specified in seven LUT columns with a maximum MSD (“MSDmax”) of 6, 9, 12, 15, 18, 24, 30. One example of such LUT is captured below based on [5]. Coarser granularity and other tabulated values are not precluded.

- and where the mapping of the dual-UL IMD order to the “MSDmax” LUT is specified with the following companion table for PC2 and PC1.5. This agreement applies to both 2UL/2DL inter-band CA and for 2UL/3DL inter-band CA for both HPUE achieved with 2Tx or 3Tx. Other table formats are not precluded for CR implementation.

| inter-band CA 2UL band 2CC power-class | MSDmax (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| PC2 | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| PC1.5 |  |  | IMD2 |  | IMD3 | IMD4 | IMD5 |

- It should be noted that for some band combinations, and some RF architectures, and for some dual-UL IMD indexes, these simplification LUTs may represent a tightening of the PC2/PC1.5 MSD requirements, while for other band combinations these LUTs may represent a relaxation of the PC2/PC1.5 MSD requirements. Some examples showing the impact of the dual IMD index on the MSD mapping can be found in the Annex based on [4].

Agreement 2 on handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD:

- For new HPUE band combinations, do not specify the MSD requirements for 2UL band 2CC HPUE PC2 and PC1.5

Agreement 3 on the testing power configuration of IMD MSD for PC1.5 inter-band UL CA:

- For 3Tx UE with PC3 in one band and PC1.5 in the other band, the power configuration is 23dBm + (24.8dBm + 24.8dBm),

- For 2Tx or 3Tx UE with PC2 in each band, or with PC2 in one band and PC1.5 in the other band, the power configuration is 26dBm + 26dBm or 26dBm + (23dBm + 23dBm).

# 3			References

- R4-2505452 On Rel-19 HPUE 2UL IMD MSD, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Apple

- R4-2505321 Discussion on HPUE MSD, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Huawei, HiSilicon.

- R4-2506980 MSD simplification rules for HPUE CA/DC UE, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, MediaTek Korea Inc.

- R4-2505374 Further discussion on HPUE MSD simplification LUTs for 1 UL and 2 UL, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Murata Manufacturing Co Ltd.

- R4-2507071 MSD beyond PC3, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Qualcomm France

- R4-2507118 On HPUE MSD simplification scope and needed corrections for 1UL 2CC and 2UL 3CC, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Skyworks Solutions Inc.

- R4-2507473 Simplification proposal for HPUE 2UL 2CC MSD, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Skyworks Solutions Inc.

- R4-2507699 Finalizing the 1UL HPUE MSD Simplification, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Skyworks Solutions Inc., Nokia, Samsung.

- R4-2506839 Discussion on HPUE MSD simplification, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Vivo.

- R4-2506383 Some general considerations on R19 HPUE IMD MSD rule, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, ZTE Corporation, Sanechips.

- R4-2420380 WF on MSD rule, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, US, Skyworks, Murata, Nokia, Qualcomm.

# 4			Annex-1: 2UL/2DL inter-band CA impact of dual-UL IMD index on MSD.

# Annex-2: 2UL/3DL inter-band CA impact of dual-UL IMD index on MSD.