3GPP TSG-RAN4 Meeting  #115	R4-2507949

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:	7.5.2

Source:	Ericsson, Xiaomi

Title:	TP to TR38.755 on UE indication to the network

Document for:	Discussion

# 1	Introduction

This contribution provides text proposals to the TR on study of means for a UE to inform the network of appropriate CA configuration [1]:

- New bandwidth class conditioned on the frequency separation between two carriers together with several examples of UE indication to the network to support a single Rx chain for the fragmented carriers. [2].

# 2	TP to TR 38.755

<<Start of Change for TR 38.755>>

# 8	Study of means for a UE to inform the network of appropriate CA configuration it can support

## 8.1.4 Indication of fragmented CA alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

The network only configured supported band combination based on the UE capability and the gNB band support, not the RF requirements in 38.101-1. For instance, a slight relaxation of an RF requirement by a few dB on SCell (if necessary) may be considered if this means that a UE would be able to indicate support of a more complex band combination. However, the REFSENS degradation in PCell might increase the risk that the PCell coverage may be lost and lead to radio link failure.

A new indication of bandwidth class conditioned on the frequency separation between two carriers might allow the UE to indicate to the network with the configurations without any REFSENS degradation in PCell

Three example configurations are presented based on the above assumption in the sub-sections below to elaborate on how the UE would indicate to the network to support a fragmented carriers with a single Rx chain.

### 8.1.4.1 Configuration of nXA + nXA with separate RX chains

It is assumed that bands nX and nY share the antenna and have 8 Rx chains available in the frequency range. As shown in Figure 8.1.4.1-1, two carriers in band nX are aggregated, each carrier supports 4 layers (4L). The PCell and SCell are supported by separate Rx chains and no relaxation of PCell REFSENS.

Figure 8.1.4.1-1. Example configuration with PCell (dark grey) and SCell (light grey): (a) nXA + nXA.

### 8.1.4.2 Configuration of nXA”+nXA”+nYA with single RX chains

In case the fragmented carriers in nX are supported by a single RX chain when the frequency span is <=100MHz, an additional band nY can be supported with 4 layers  without increasing the total number of Rx chains. The PCell is configured in band nY as shown in Figure 8.1.4.2-1 assuming that the UE cannot be configured a PCell in band nX due to REFSENS degradation.

This band combination is indicated in the UE capability, but for the network to correctly parse the capability in nX a condition on the maximum frequency separation shall be added. This could be done by specifying a new bandwidth class A only applicable to intra-band NCCA and conditioned on a maximum frequency separation between the lowest and highest carrier edges by 100 MHz. For example, a UE might only indicate 50 MHz bandwidth class on certain FDD band to ensure there is no PCell degradation. A network does not comprehend the new BW class would not configure a band combination containing it, thereby ensuring that this feature does not interfere with the operation of legacy networks.

A limited frequency spacing in band nX will thus enable a single Rx implementation. In this case, a larger number of carriers can be supported with the same 8 Rx chains if the frequency separation of nX is ≤ 50 MHz while maintaining the number of MIMO layers possibly with a slight REFSENS relaxation in SCells. i.e. the spared Rx chains would be used in band nY.

Figure 8.1.4.2-1. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA

### 8.1.4.3 Configuration of nXA”+nXA”+nYA+nZA with single Rx chains and with modified MIMO capability

Furthermore, an additional configuration in Figure 8.1.4.3-1 illustrates that reducing the number of supported layers in one of the bands allows for reducing the number of Rx chains. The Rx chain that are freed-up can then be reallocated to support more CCs. In the example below the fragmented carriers with a single Rx chain are configured only in SCells to avoid PCell degradation.

Figure 8.1.4.3-1. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA + nZA.

As illustrated by the examples above, all supported band combinations, regardless of the UE implementation, must be explicitly included in the UE capability signaling. The UE cannot inform the NW that it has a spare Rx chain and expects the NW’s action based on this information without corresponding capability reporting. The network can not only use RF requirements to configure, release, activate, or deactivate carriers, and the network’s action is based on corresponding capability reporting.

To facilitate this, introducing a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios) could indicate the support for such fragmented carriers with a single Rx chain.

The example frequency separation class table is shown below. The capabilities of the corresponding band entry (i.e. feature set) would then be conditioned on a maximum frequency separation of the carriers.

Table 8.1.4-1: NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| I | 100 MHz | 2 |
| II | 200MHz | 2 |
| III | 200 MHz | 4 |

Another example is to use the currently undefined NR CA bandwidth class (BCS) ‘F’ for indicating a CA configuration with fragmented carrier operation.

Table 8.1.4-2: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the table above, where the newly added parts are highlighted with bold font, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two fragments are shown in Fig 8.1.x-1.

Fig.  8.1.4-3: Example of CA_n25(2F)

<<End of Change for TR 38.755>>

# 4	References

TR 38.755 V0.2.0

R4-2504039, "Discussion on UE capability indication for single Rx chain," Ericsson.