3GPP TSG RAN WG4 Meeting #115		 R4-2507950

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.5.2

Source: 		Huawei, vivo, OPPO, MTK, Apple, ZTE

Title: 	TP for TR38.755 on fall-back considerations

Document for:	Approval

1. Introduction

In RAN4#114 the TR [1] has been approved. In this paper we provide our view for the fall-back considerations about CA configuration and architecture switching.

2. TP

Proposed TP for TR38.755

<Start of TP>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]		R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

[aa]	R4-2503366 On fallback behavior and signaling aspect for fragmented carriers, RAN4#114bis, Samsung

[bb] 	R4-2504213 Discussion on remaining signalling aspects for DL fragmented carriers study, RAN4#114bis, MediaTek

[cc] 	R4-2504450 On methods for reducing the number of UE Rx chains for fragmented carriers, RAN4#114bis, Apple

[dd]	R4-2504260 Views on the usage of the FR1 fragmented carriers, RAN4#114bis, CHTTL

[ee] 	R4-2411310 Views on UE RF architecture and NW deployment assumption for fragmented carriers, RAN4#112, Samsung, TELUS, Bell mobility.

<unchanged part omitted>

8.2	Study on semi-statically switch hardware resources procedure and fall back considerations

When either the in-gap interference or the self-interference is too high, the receiver is hard to maintain the shared receiving state, i.e., one Rx RF chain, and the “fallback” behaviour needs to be studied. In order to make the application of this function more future-oriented, some higher CA combinations are also considered.

In total, two aspects should be involved when discussing fallback behaviour: “CA fallback” and “architecture switching”.

“CA fallback” means releasing at least one SCell in one band of the band combination, while the “architecture switching” means the switching from “One Rx RF chain (aka. full shared Rx RF chain)” to legacy CA architecture  [aa], where legacy CA architecture includes fully separated Rx chain and partially shared Rx chain.When there is only one component carrier in one band, it is nature that UE RF hardware is also configured to legacy architecture for single carrier in the band.

When categorizing “CA fallback”, the following scenarios are involved:

a.) Legacy CA combination falls back to lower order CA

-	Legacy CA combination belongs to mix of inter-band CA and intra-band CA with more than 2CCs, etc.

b.) Legacy CA combination falls back to “non-CA” mode

-	Legacy CA combination belongs to intra-band CA combination with 2CCs.

When categorizing “architecture switching”, the following scenarios are involved:

a.) Architecture switches between “partially shared Rx RF chain” mode and “one Rx RF chain” mode

b.) Architecture switches between “fully separate Rx RF chain” mode and “one Rx RF chain” mode

Based on the analysis above, the fallback behaviour should be totally divided into the following three scenarios:

a.) Scenario 1: CA fallback without architecture switching

b.) Scenario 2: CA fallback with architecture switching

c.) Scenario 3: architecture switching without CA fallback

Below Figure 8.2-1 is an example to illustrate how the UE’s work mode transfers from “one Rx RF chain” to scenarios 1~3. The figure is generated based on Figure 3 in [bb] and Figure 1 in [aa].

Figure 8.2-1: Fall-back behaviour, scenarios 1~3

For scenario 1, it doesn’t require additional UE capability and network configuration. As a default, any UE supports the “one Rx RF chain” will support the Scenario 1 as the legacy CA fallback configuration. From network point of view, if the performance on CC2 is not at satisfactory level, the network could just release the CC2 using existing CA operation process. Based on the evaluation result in section 7, the performance loss of “one Rx RF chain” is comparable with that of the other CA configurations in the RAN4 specification. It’s therefore feasible to handle the CA configuration with “one Rx RF chain” as a normal CA configuration. It is also beneficial for early deployment of the fragmented carrier feature without requiring upgrading the network. On the other hand, as [cc] pointed out, in some cases the Scenario 1 might not be the best choice from network’s point of view if the CC2 has larger bandwidth than CC3, or the CC2 could achieve better channel quality using legacy CA architecture than CC3. The Scenario 2 is thus considered.

For scenario 2, it keeps the CC2 and releases CC3 if the network decides the CC2 could provide what the network expects with better performance than CC3. This Scenario requires architecture switching, which means the RF chains previously used by CC3 need to be retuned to CC2 in case all the Rx chains are used up. However, whether the RF chains for different bands could be flexibly switched across the bands depends on UE implementation and is band combination specific, which means such info, e.g., UE capability, may need to be indicated to the network. Whether and how such information is indicated to network is for further study. The Scenario 2 would be useful especially when the performance of CC2(CC1) could be improved a lot when switching back to legacy CA architecture, such as when there is serious Tx leakage, or the in-gap interference is quite high considering the non-collocated scenario. Additional UE measurement might be needed to assist the network to compare the performance of CC2 and CC3.

For Scenario 3, it doesn’t reduce the number of CCs, but requires architecture switching with reduced DL MIMO layers (Scenario 3a) or the same DL MIMO layers (Scenario 3b). Similar to Scenario 2,whether the switching is feasible depends on UE implementation and is band combination specific, and whether and how such information is indicated to network is for further study. The Scenario 3b assumes there are enough spare Rx chains. [dd] mentions the benefit of adopting “one Rx RF Chain” is not very clear from the RF performance aspect if there are spare Rx chains. On the other hand, the UE might get the benefit of power saving from “one Rx RF Chain” if the relevant conditions could ensure a satisfactory Rx performance.

It needs to be considered whether the transition from “one Rx RF Chain” to Scenario 1~3 is ‘network initiated’ or ‘UE initiated’. ‘Network initiated’ means the network will send a command to the UE to trigger the transition, with or without assistant indication or measurement report from the UE. ‘UE initiated’ means the UE could execute the transition without receiving the network’s command. UE may inform the network after the transition happened.

-	Scenario 1, 2 and 3a involve the change of number of CCs or number of DL MIMO layers, which has to be configured by the network, i.e. ‘network initiated’.

-	For Scenario 2, the network configuration might explicitly indicate the architecture switching, or the switching could be done by the UE autonomously.

-	The Scenario 3b could be transparent to the network if the UE has spare Rx chains. Although some interruption is needed. The network could also intentionally configure the Scenario 3b. Therefore both ‘network initiated’ or ‘UE initiated’ could apply to Scenario 3b.

More details on how the UE’s work mode switching from separate mode to “one RX RF chain” and falling back to scenarios 1~3 are achieved are as follows:

8.2.1	Case1: Partially shared switch to fully shared & fallback behaviour

According to session 5.2, the partially shared architectures contain both internal Rx path split and external Rx path split. These two partially shared architectures are expressed in the same form as below, i.e., the antenna is shared between CC1/2/3, and the other devices before down-conversion such as LNA, BPF are omitted here.

When partially shared architecture switches to fully shared architecture, for band nX CC1 and CC2 would be received by one RX RF chain together instead of each CC taking up one chain. Therefore, there would be one chain being released after each antenna.

According to the utilization of the released chain, there are two sub-scenarios: the released chain is not used (Fig 8.2.1-1), or the released chain is used to aggregate more CCs in the same band (Fig 8.2.1-3). As the new aggregated CCs belong to the same band and there is no new demand in the UE's hardware capability, it could be predicted that there is no new capability needed except for supporting the “one Rx RF chain” architecture.

a.) After sharing, the released chain is not used.

-	Partially shared→ “one Rx RF chain”

Figure 8.2.1-1: Partially shared switch to “one Rx RF chain” mode

As can be seen in Fig 8.2.1-1, before sharing the partially shared architecture composes of one set of RX chain (one main Rx + one diversity Rx). After sharing half of the chain is released and unused which may have the benefit of power saving.

-	Fallback behaviour:

Figure 8.2.1-2: Fallback to lower order CA combination (scenario1), or CA combination unchanged (scenario3)

b.) After one Rx RF chain is enabled, the released chain is used to aggregate more CCs in the same band.

-	Partially shared→ “one Rx RF chain” & Fallback behaviour

The following analysis takes the 4Rx RFchain architecture as an example, and the analysis of the 2Rx RFchain architecture is similar. 4Rx UE contains one main receiver branch and three diversity receiver branches, which could be seen in Fig 8.2.1-3.

Figure 8.2.1-3: 4RX UE, switch to “one Rx RF chain” mode

Figure 8.2.1-4: 4RX UE, fall-back to a lower order CA combination

In Fig 8.2.1-4, for 4Rx UE the fallback behaviour would lead to a lower order CA combination with architecture changed (scenario1) or unchanged (scenario2).

8.2.2	Case2: Fully separate switch to fully shared & fallback behaviour

According to the discussion before, there is another implementation architecture before sharing: fully separate mode, which means each CC has exclusive access to one single Rx RF chain from antenna to ADC.[ee]

The following analysis primarily focuses on the utilization of idle links after switching to “one Rx RF chain” mode and its differences from the partially shared architecture. The fallback behaviour is similar and therefore omitted here.

Figure 8.2.2-1: After switching to “one Rx RF chain” mode, more CC from another band could be aggregated (case A), or MIMO layer could be higher

When fully separate architecture switch to fully shared architecture, some RX chains would also be released and the band combination or MIMO layer could be increased. Since the switching scheme of case A may involve changes in frequency and/or bandwidth of some devices as the part of the architecture would switch to receive the CC3 from another band, it should be supported by additional capability(ies).

<End of TP>

Reference:

- R4-2505158 draft TR 38.755_v020_r1-cl