3GPP TSG RAN WG4 Meeting #115		 R4-2507951

St Julian’s, Malta  19th – 23th May, 2025

Agenda Item:	7.5.2

Source: 	MediaTek Inc., Xiaomi, Nokia, Apple, Ericsson, Huawei, OPPO

Title: 	TP for TR38.755: Summary for NR FR1 DL fragment carriers study

Document for:	Approval

1. Introduction

The SI[1] for Rel-19 NR FR1 DL fragmented carriers has been studied and discussed from RAN4#112 to RAN4#115. Method for reducing number of UE Rx chains, architectures has been discussed. The power spectral density difference between carriers of co-located adjacent channel operators also have been studied. Further more, the potential adjusted RF requirements and signalling indication and UE behaviour were also been studied. The contribution provide summary of these aspects.

2. TP for TR38.755

<Start of TP>

9 Summary for NR FR1 DL fragment carriers study

The intra-band non-contiguous carrier aggregation (Intra-band NC CA) has been introduced in the RAN4 specs for years. The requirements defined in the current specification for intra-band DL NC CA are based on the partially-shared architecture which requires separate Rx RF chains and Rx LOs for receiving each of the non-contiguous carriers that limit the CA capabilities as the maximum supported number of component carriers. In this study, RAN4 has investigated an architecture with one Rx RF chain (fully shared receiver) architecture to reduce the number of Rx chains for receiving intra-band DL NC CA carriers in the case of the frequency span between the DL carriers within 100MHz so that the UE can support receiving more DL carriers with remaining Rx RF chains.

From the study on power spectral density difference between carriers of co-located adjacent channel operators, the in-gap interference to wanted signal ratio (ISR ratio) can range from 20dB to less than 0dB due to different gNB antenna pattern assumptions as shown in system level simulation. The simulation further shows if the network shifts i.e., non-collocated adjacent channel operators, the delta RSRP can range from tens of dBs to negative tens of dBs which implies the in-gap interference rejection capability needs to be as good as possible.

The adjusted receiver RF requirements for one Rx RF chain architecture for example bands have also been evaluated by companies. There may be additional REFSENS degradation for FDD bands in some scenarios in comparison with the existing requirement due to one Rx RF chain mode on both PCC and SCC in the FDD bands due to self-Tx leakage impact and insufficient filter attenuation. The network vendors have expressed strong concerns regarding the degradation of the Pcell when one Rx RF chain mode is configured.

In summary, to manage the impact of Pcell performance degradation, the following operation scenarios can be considered for intra-band non-contiguous DL CA:

a.) FDD band DL fragmented carriers

i. CBW of both DL fragmented carriers + Wgap are small enough

ii. UE located so near to the gNB that UL Tx power level is not too large

iii. For higher order CA combination, both intra-band DL fragmented carriers are SCC (Pcell UL is in another band)

iv. When UE is capable of interference measurement, self-assessment and/or reporting

v. The frequency allocation of PCC and SCC to avoid worst case configuration on DL performance i.e., PCC/SCC assignment

b.) TDD band DL fragmented carriers

There are different opinions about what UE Rx RF requirements should be relaxed for receiving the Intra-band NC CA using one Rx RF chain and, where applicable, how much in what band. Further candidates for a relaxation are e.g. the adjacent channel selectivity and the in-band blocking inside the gap between the NC CCs, in particular in the asymmetric case of different channel bandwidths of the NC CCs because then the image of an in-gap interferer can overlap with the wider of the NC CCs.

The signalling indication has also been discussed and agreed on the following two aspects[22]. The down-selection or final decision can be made during work item phase:

Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

Alternative 2: use the new notation or new bandwidth class for CA configurations with single Rx RF Chain

The UE behaviour regarding CA fallback with/without UE architecture switching has also been discussed. One option is for a UE that is configured with one Rx RF chain mode to fallback to a single CC. As for other fallback configurations, scenario 2 and scenario 3 as illustrated in clause 8.2, Fig 8.2-1, that may happen with architecture switching together and depend on UE implementation, which means such info, e.g., UE capability, may need to be indicated to the network. Whether and how such information is indicated to network is for further study.

The criteria of enabling one Rx RF chain mode have also been discussed. One approach is to rely on existing channel quality reporting and procedures without a new indication from the UE. The measurement can be performed in one Rx RF chain mode or legacy mode (e.g., with partially-shared Rx RF chain). The other approach is for a UE to perform measurements to determine and indicate to the network whether it can support non-contiguous CCs with one Rx RF chain. An exemplary flow of UE state change is also captured in the technical report for reference.

Based on the study outcome, RAN4 concludes that it is feasible under certain conditions for a NR UE to operate one Rx RF chain mode to receive two DL non-contiguous carriers.

<End of TP>