3GPP TSG RAN WG4 Meeting #115		 R4-2507952

St Julian’s, Malta  19th – 23th May, 2025

Agenda Item:	7.5.4

Source: 	MediaTek Inc.

Title: 	Summary on the evaluation results and further discussion on the one Rx RF chain mode applicability

Document for:	Approval

1. Introduction

The performance degradation when one Rx RF chain is enabled had been discussed in previous RAN4 meeting and concern on PCC degradation were raised. A possible alternative, PCC/SCC swapping, was also considered and evaluated. In the contribution, we provide TP for summary on the evaluation results and further discussion on the one Rx RF chain mode applicability.

2. Discussion

TP for TR38.755

<Start of TP>

7.x Summary of evaluation results

The ΔRIBNC evaluation results for example bands are provided by companies and summarized in below tables[24-31]:

7.x.1 ΔRIBNC evaluation on example bands for one Rx RF chain mode

Table 7.x.1-1: Band n2(2A)

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Company | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | ABB LPF Rejection | NF raised |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 5MHz + 5MHz | MediaTek | Wgap = 50.0 | 10 | 12 | 12.8 | 11.8 | 11.6 |
|  |  |  | Wgap = [10] | 25 | 0 | 0 |  |  |
|  |  | Spreadtrum | Wgap = 50.0 | 10 | 10.6 | 11.1 | 8 | 10.6 |
|  |  | Samsung |  |  | 12.2 | 8.1 | 13 | 5 |
|  |  | Huawei | Wgap = 50.0 | 10 | 2.8 | 6.1 | 15.2 | 2 |
|  |  |  |  |  | 0.0/swap | 0.0/swap | 36.1 | 0 |
|  |  |  | Wgap = 30.0 | 10 | 0.0 | 0.0 | 31.7 | 0.0 |
|  |  |  |  |  | 0.0/swap | 0.0/swap | 47.0 | 0 |

Table 7.x.1-2: Band n25(2A)

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Company | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | ABB LPF Rejection | NF raised |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 5MHz + 5MHz | MediaTek | Wgap = 55.0 | 10 | 13.8 | 14.3 | 6.1 | 13.1 |
|  |  |  |  |  | 4.0/swap | 3.4/swap | 22.2 | 4 |
|  |  |  | Wgap = [10 ( 20] | 25 | 0 | 0 |  |  |
|  | 40MHz + 5MHz |  | Wgap = 20.0 | 40 (RBstart = 176) | 6.5 | 24.9 | 6.1 | 13.1 |
|  |  |  |  |  | 2.2/swap | 8/swap | 11.1 | 7.2 |
|  | 5MHz + 5MHz | Spreadtrum | Wgap = 55.0 | 10 | 10.9 | 11.5 | 6 | 10.6 |
|  |  | Apple |  |  | 12 | 12.7 | 12.9 | 11 |
|  |  | Samsung |  |  | 15.4 | 15.9 | 8.6 | 9.2 |
|  |  | ZTE |  |  | 13.8 | 9.5 | 12.6 | 9.5 |
|  | 5MHz + 5MHz | Huawei | Wgap = 55.0 | 10 | 5.7 | 7.7 | 11.9 | 5 |
|  |  |  |  |  | 0.0/swap | 0.0/swap | 47.0 | 0 |
|  | 40MHz + 5MHz |  | Wgap = 20.0 | 40 (RBstart = 176) | 5.0 | 24.6 | 11.9 | 5 |
|  |  |  |  |  | 0.0/swap | 0.2/swap | 22.8 | 0 |

Table 7.x.1-3: Band n3(2A)

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Company | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | ABB LPF Rejection | NF raised |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | 5MHz + 5MHz | MediaTek | Wgap = 65.0 | 12 | 12.2 | 12.9 | 9 | 11.7 |
|  |  |  |  |  | 0 | 0 |  |  |
|  |  | Murata | Wgap = 65.0 | 12 | 9.3 | 10.2 | 13.4 | 10(P)/8(D) |
|  |  | ZTE |  |  | 14.5 | 10.3 | 12.1 | 11 |
|  |  | Spreadtrum |  |  | 10 | 10.7 | 13.4 | 10.6 |
|  |  | Apple |  |  | 11.5 | 12.2 | 13.4 | 10.5 |
|  |  | Xiaomi (set 1) |  |  | 14 | 14.3 | 12 | 10 |
|  |  | Samsung |  |  | 16.3 | 12 | 7.9 | 9.3 |
|  |  | Huawei |  |  | 5.6 | 7.5 | 12.8 | 5 |
|  |  |  |  |  | 0.0/swap | 0.0/swap | 35.0 | 0 |

Table 7.x.1-4: Band n7(2A)

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Company | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | ABB LPF Rejection | NF raised |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7(2A) | 10MHz + 5MHz | MediaTek | Wgap = 55.0 | 32 | 6.4 | 6.4 | 17.2 | 6.1 |
|  |  |  | Wgap = [25] | 25 | 0 | 0 |  |  |
|  |  | ZTE | Wgap = 55.0 | 32 | 8.6 | 10.4 | 17.3 | 6 |
|  |  | Spreadtrum |  |  | 7.6 | 7.7 | 8 | 8.6 |
|  |  | Huawei |  |  | 0.1 | 0.1 | 25 | 0 |
|  |  |  |  |  | 0.0/swap | 0.0/swap | 41.5 | 0 |

Table 7.x.1-5: Band n26(2A)

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Company | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | ABB LPF Rejection | NF raised |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15MHz + 10MHz | MediaTek | Wgap = 10.0 | 5 (RBstart = 74) | 13.9 | 26.2 | 9.7 | 13.3 |
|  |  | Spreadtrum |  |  | 11.7 | 25.7 | 11.5 | 8.6 |
|  |  | Huawei |  |  | 2.3 | 25.2 | 14.6 | 2 |
|  |  |  |  |  | 0.0/swap | 0.0/swap | 29.0 | 0 |

7.x.2 In-Gap Blocking for one Rx RF chain mode

The in-gap blocking performance has also been evaluated and summarized in R4-2500676. It was agreed the requirements can be evaluated in two aspects[33]:

Image of in-gap interference (ACS/IBB) not overlap with wanted CC.

Image of in-gap interference overlap with wanted CC.

Based on companies analysis[19][20], the case A in-gap blocking performance degradation is not serious based on evaluated ΔRIBNC. For the case B, the interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC.

7.x.3 Discussion on applicable scenarios when enabling one Rx RF chain mode

The adjusted RF requirements for example bands have also been evaluated by companies. There would be REFSENS degradation due to one Rx RF chain mode on both PCC and SCC in the FDD bands due to self-Tx leakage impact. The network vendor express strong concerns to the degradation on the Pcell when one Rx RF chain mode is configured. It was agreed in WF[22] to further discuss on the applicable scenarios. Evaluation results also shows when the channel BW of the CCs plus Wgap are small enough, the REFSENS would not be degraded. If the two CCs are configured as SCCs that may be fallback of higher order CA combination when there’s no scheduled self-band UL transmission, the REFSENS are not degraded also. Sor for TDD bands the situation would be similar that REFSENS of CCs would not be degraded if UE is configured in one Rx RF chain mode. One possible approach to reduce ΔRIBNC is PCC/SCC swapping, the degradation may be largely improved to a few dB form the worst case configuration. Another use case is when UE is at the location near the gNB, the received level of wanted CCs can be much higher than the REFSENS level while the configured UL transmission power level does not need to be large, Pcell REFSENS would also not be degraded due to one Rx RF chain configuration or the degradation can be tolerable.

In summary, to manage the impact of Pcell degradation, the following operation scenarios can be considered for intra-band non-contiguous DL CA:

a.) FDD band DL fragmented carriers

i. CBW of both DL fragmented carriers + Wgap are small enough

ii. UE located so near to the gNB that UL Tx power level is not too large

iii. For higher order CA combination, both intra-band DL fragmented carriers are SCC (Pcell UL is in another band)

iv. When UE is capable of interference measurement, self-assessment and/or reporting

v. The frequency allocation of PCC and SCC to avoid worst case configuration on DL performance i.e., PCC/SCC assignment

b.) TDD band DL fragmented carriers

<End of TP>

Proposal 1: Agree on DL fragmented carrier using one Rx RF chain performance evaluation results in the above TP contents for TR38.755

3. Conclusion

Proposal 1: Agree on DL fragmented carrier using one Rx RF chain performance evaluation results in the above TP contents for TR38.755

4. Reference for the contribution:

To be consistent with TR38.755, the reference contributions are based on TR38.755 v0.2.0

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]	R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

[22]	R4-2505109 WF on NR FR1 DL fragmented carriers study, RAN4#114bis, MediaTek, Apple, Qualcomm

[23]	R4-2503556 Procedures for fragmented carriers, RAN4#114bis, Qualcomm

[24]	R4-2501397 Discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114, MediaTek

[25]	R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[26]	R4-2504239 Discussion of impacts on UE RF requirements and DL performance, RAN4#114bis, Spreadtrum, UNISOC

[27]	R4-2506270 TP on RF requirements evaluation for fragmented carriers from company S, RAN4#115, Samsung

[28]	R4-2504451 On UE RF requirements for fragmented carriers, RAN4#114bis, Apple

[29]	R4-2506378 TP to TR38.755 on UE RF requirements impact for fragmented carriers, RAN4#115, ZTE

[30]	R4-2504284 On RF requirements of fragmented carriers, RAN4#114bis, Huanwe

[31]	R4-2503338 Impacts on UE RF requirements and DL performance, RAN4#114bis, Xiaomi

[32]	R4-2502873 WF on NR FR1 DL fragmented carriers study, RAN4#114, Moderator (Mediatek Inc.), OPPO, vivo, ZTE, Nokia, Ericsson, Xiaomi, Apple, Samsung, Huawei

[33]	R4-2500676 Topic summary for [114][124] FS_NR_FR1_DL_Frag_Carrier, RAN4#114, Moderator