3GPP TSG-RAN4 Meeting #115	R4-2507954

# Malta, Malta, 19th May 2025 - 23rd May 2025

Title: 	WF on 7 MHz channel BW

Agenda item:	7.8.1

Source:	Ericsson

Document for:	Approval

# 1- 	UE A-MPR

## NS_12: A-MPR regions

Agreement: The following NS_12 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | [A2] |
|  |  | ≤1.8 | >0.72 | [A1] |
|  |  | >1.8 | >1.44 | [A1] |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

## NS_13: A-MPR regions

Agreement: The following NS_13 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | [A1] |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | [A2] |
|  |  | >1.8 | ≥1.8 | [A7] |

| Modulation/Waveform | A1 | A2 | A7 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 4.5 |

## S_14: A-MPR

Agreement: No A-MPR is needed for NS_14 and 7 MHz channel bandwidth.

## NS_15: A-MPR regions

Agreement: The following NS_14 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | [A1 or A7] |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | [A2] |
|  |  | ≤ 1.44 | ≤ 0.36 | [A3] |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | [A2] |
|  |  | ≥ 5.94 | ≤ 2.7 | [A3 or A8] |

| Modulation/Waveform | A1 | A2 | A3 | A7 | A8 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 7 | ≤ 2.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 8.5 | ≤ 3 |

## NS_12: A-MPR regions for PC2

Agreement: The following PC2 NS_12 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | [A3] |
|  |  | ≤1.8 | >0.72 | [A1] |
|  |  | >1.8 | >1.44 | [A1] |

Table 3: A-MPR for NS_12 (PC2), from [3]

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 3.5 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4 | ≤ 6.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 5.0 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 16 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 64 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 | ≤ 7.5 |

## NS_13: A-MPR regions for PC2

Agreement: The following PC2 NS_13 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | [A1] |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | [A2] |
|  |  | >1.8 | ≥1.8 | [A1] |

Table 6: A-MPR for NS_13 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 7 |  |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

## NS_14: A-MPR for PC2

Agreement: No A-MPR is needed for NS_14 PC2 and 7 MHz channel bandwidth.

## NS_15: A-MPR regions for PC2

Agreement: The following PC2 NS_15 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | [A1] |
|  |  | <4.68, ≥2.52 | ≥2.16 | [A2] |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | [A2] |
|  |  | <1.8 | ≤0.36 | [A3] |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | [A2] |
|  |  | ≥5.94 | <2.7 | [A3] |

Table 10: A-MPR for NS_15 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 |

# BS RF

## Narrowband blocking – interferer offset

Agreement: The narrowband blocking interferer offset for 7MHz channel BW is 400kHz.

## ICS

Agreement: The size of the interferer for ICS and 7MHz channel BW is15 RBs