3GPP TSG-RAN WG4 Meeting #115	R4-2507955

Malta, MT, 19-23 May, 2025

Title: 	WF on PHS protection update

Agenda Item:	4.1.1

Source: 	KDDI

Document for:	Approval

- Introduction

The moderator copies agreements and approved way forwards during the ad-hoc session into this contribution in agenda 4.1.1. On top of that, we capture additional way forwards into this contribution.

Sub-topic 1-1: PHS protection update

NS_05 PC3

< Issue 1-1-1: NS_05 PC3 A-MPR region split and values for PUCCH >

Agreement of ad-hoc session:

- Split the agreed Region A2 into two new regions for NS_05 PC3.

- FFS on the detailed split region definition and corresponding values based on offline discussion.

- Do not assume improved LO leakage or IQ image as baseline for any work in 3GPP.

< Issue 1-1-2: NS_05 PC3 A-MPR region adjustment >

Proposal 1 (R4-2505431 Apple):

- Use the alternative region definition for A2 as it optimises A-MPR.

Way forward:

- Discuss whether this modification is necessary, if not keep the original agreement.

< Issue 1-1-3: A-MPR table format>

Proposal 1 (R4-2505431 Apple):

- Use the more “modern” table layout when writing the CR as proposed in Table 2.

Way forward:

- Use the more “modern” table layout when writing the CR as proposed in Table 2.

NS_05 PC2

< Issue 1-1-4: A-MPR region for NS_05 PC2 >

< Issue 1-1-5: A-MPR values for NS_05 PC2 >

Way forward:

- Define one set AMPR for n1 PC2 NS_05 covering both 1T and 2T.

- Adding 1dB to 1Tx A-MPR values for 2Tx in all configurations for PC2 NS_05.

- Agree with the following A-MPR region and values for NS_05 PC2.

Table 1: A-MPR regions for NS_05 (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 5 | < 1925 | ≥ 3.24(18RB) | ≤ 0.18 | N/A | N/A | N/A | N/A |
| 10 | ≤ 1925 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | < 1.35* RBstart*12*SCS + 4.32 ≥ 1.2* RBstart*12*SCS + 3.6 | ≥ 0 |
| 10 | 1925 < fc < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | N/A | N/A |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs | ≥ 0 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≤ 4.5MHz/12/scs | ≤ 3.6MHz/12/scs | ≤ RBstart*1.2 + 3.96MHz/12/scs ≥ RBstart*12*SCS * + 3.6 > 4.5MHz/12/scs | ≥ 0 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs | ≤ 1.8MHz/12/scs | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | N/A | N/A |
| 20 | < 1945.7 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs | ≥ 0 | ≤ 6.48MHz/12/scs; ≤ RBstart*1.0 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs | N/A | N/A |

Table 2: A-MPR values for NS_05 (Power Class 2, 5MHz, fc<1925MHz)

| Modulation/Waveform |  | A1 (dB) |
| --- | --- | --- |
|  |  | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 |
|  | QPSK | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 |
|  | 64 QAM | ≤ 4.5 |
|  | 256 QAM | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 5.5 |
|  | 16 QAM | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 |
|  | 256 QAM | ≤ 7.5 |

Table 3: A-MPR values for NS_05 (Power Class 2, 10MHz, fc≤ 1925)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | QPSK | ≤ 5.5 | ≤ 4.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 4.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 5.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

Table 4: A-MPR values for NS_05 (Power Class 2, 10MHz, 1925 < fc < 1930.7)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 4.5 | ≤ 3.0 |
|  | QPSK | ≤ 5.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

Table 5: A-MPR values for NS_05 (Power Class 2, 15MHz, 1927.5 ≤ fc <1932.5)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 |
|  | QPSK | ≤ 5.5 | ≤ 4.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.0 | ≤ 7.5 |

Table 6: A-MPR values for NS_05 (Power Class 2, 15MHz, 1932.5 ≤ fc < 1938.2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 3.5 |
|  | QPSK | ≤ 4.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.0 |
|  | 16 QAM | ≤ 5.5 | ≤ 6.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

Table 7: A-MPR values for NS_05 (Power Class 2, 20MHz, fc < 1945.7)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 4.5 | ≤ 4.0 |
|  | QPSK | ≤ 5.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 7.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 6.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.0 |

NS_05U PC3

< Issue 1-1-6: A-MPR region for NS_05U PC3 >

Based on the agreement of ad-hoc session on Issue 1-1-1 for NS_05 PC3, go with the same approach.

Way forward:

- Split the agreed Region A2 into two new regions for NS_05U PC3 same as NS_05 PC3.

Proposal 1 (R4-2507226 Ericsson):

- Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) require some minimum backoff for outer RB allocation when the NS_05U is signalled.

Way forward:

- Need to clarify which region this proposal is targeting.

NS_05U PC2

< Issue 1-1-7: A-MPR region for NS_05U PC2 >

Way forward:

- Harmonize the proposals and discuss offline.

< Issue 1-1-8: A-MPR values for NS_05U PC2 >

Way forward:

- Define one set AMPR for n1 PC2 covering both 1T and 2T for NS_05U.

- Adding [0.5~1] dB to 1Tx A-MPR values for 2Tx in all configurations for PC2 NS_05U.

- Harmonize the proposals and discuss offline.

modifiedMPR-Behaviour

< Issue 1-1-9: How to capture the modifiedMPR-behaviour >

Proposal 1 (R4-2507226 Ericsson):

- For indication of modified MPR behaviour for NS_05/NS_05U, UEs compliant with Rel-19 or later shall set the relevant bit to “1” and thus always include the bitmap for band n1 such that backwards compatibility for modified MPR indication is maintained.

  - Before R19

  - R19 and forward:

Proposal 2 (R4-2507723 Huawei):

- Use the modifiedMPR-Behavior IE in legacy releases to allow a UE to indicate the support of new MPR/A-MPR requirements introduced after the release was frozen. Do not use the modifiedMPR-Behavior IE in open release for the sake of forward-compatibility.

- Add the definition of modifiedMPR-Behavior for band n1 in Rel-16 to Rel-18 specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19. The bit is reserved from Rel-19 onwards.

- Follow the rationale in defining modifiedMPR-Behavior for band n1, align the requirements defined in TS 38.101-1 for other bands.

Proposal 3 (R4-2507006 NTT DoCoMo):

To avoid complications for the network, the modifiedMPR Table for n1 uses "shall" to mandate setting the bit to 1 for Rel-19, and "may" to allow for it to be optional for releases prior to Rel-18.

Agreement of ad-hoc session:

- For Rel-15 to Rel-18 specs, the modifiedMPR bit may be set to 1 to indicate the application of newly defined AMPR values for NS_05 and NS_05U in Rel-19, for Rel-19 the modifiedMPR bit shall be set to 1.

- The detailed wording can be further discussed over CR revision.

Reference

[1] R4-2507540 Topic summary for [115][101] Upto_R16_UERF_maintenance v2-NWM, OPPO, RAN4#115

[2] DraftR4-25xxxx_Ad-hoc minutes for UE RF maintenance, Xiaomi, RAN4#115

[3] R4-115_WF_AMPR region and value for NS_05 PC2_v04.pptx, KDDI, RAN4#115