3GPP TSG-RAN WG4 Meeting #115																			R4-2507957

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	WF on Modified MPR Behavior

Agenda item:	4.1.1

Document for:	Approval

- Background

In TS 38.101-1 Annex L.1 contains the definitions of the bits in the field modifiedMPR-Behaviour. Different approaches are used for different bands when new requirements are defined or a new formfactor is added. There’s no common understanding when to use ‘may’ and when to use ‘shall’ for the bits for different releases. Therefore, companies think it’s necessary to have some discussions on how to clean the existing bits.

- Open issues for next meeting discussion

In next meeting, at least the following issues will be further discussed.

- Check the difference between bit ‘0’ and absent from BS and UE perspective.

- Check if the spec version number can be removed from next release.

- Check if the requirements are changed several times, how to handle the legacy bits.

  - See n41

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

- Check if any good approach on how to handle the bits for different form factors, if dedicated bits should be used to differentiate the form factors, FWA and smart phone, for future new cases.

| 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| --- | --- | --- |

- References

[1] R4-2505680	CR 38.101-1 R17 Correction for ModifiedMPR table [Modified_MPR]	Nokia

[2] R4-2505681	CR 38.101-1 R18 Correction for ModifiedMPR table [Modified_MPR]	Nokia

[3] R4-2505682	CR 38.101-1 R19 Correction for ModifiedMPR table [Modified_MPR]	Nokia

[4] R4-2505757	Discussion on ModifiedMPR-Behavior and spec	Xiaomi

[5] R4-2507723	(PHS_System_NRonly) On the requirements of modifiedMPR-Behavior	Huawei, HiSilicon

[6] R4-2507227	CR to 38.101-1: Modification of additional requirements indicated by NS_05 NS_05U  [NR_newRAT-Core]	Ericsson, Samsung

[7] R4-2507228	CR to 38.101-1 Rel-16. to add indication of modified MPR behavior  [NR_newRAT-Core]	Ericsson, Samsung