3GPP TSG-RAN-WG4 Meeting #115 	R4-2506183

# Saint Julian's, Malta, 19th – 23rd May, 2025

# Title:	Intra-band Contiguous CA Issue and Proposed Resolution

# Source:	T-Mobile USA

# Agenda Item:	5.25.1

# Document for:	Approval

# Introduction

T-Mobile USA has recently discovered some specification issues related to the configuration of carriers for intra-band contiguous CA. This contribution describes the issues and proposes a solution.

# Discussion

The first issue is that the equations for Foffset,high and Foffset,low for intra-band contiguous CA in 38.101-1 do not match the equations in 38.104. 38.101-1 uses the maximum of the minimum guardband for the low carrier bandwidth and the minimum guardband for the high carrier for both the low and high guardband. 38.104 uses the minimum guardband for the low carrier on the low carrier side, and the minimum guardband for the high carrier on the high carrier side:

Figure 1 Foffset equations from 38.101-1

Figure 2 Foffset equations from 38.104

Observation 1: For Foffset,low and Foffset,high for intra-band contiguous CA, 38.101-1 uses the maximum of the minimum guardband for the low carrier bandwidth and the minimum guardband for the high carrier for both the low and high guardband. 38.104 uses the minimum guardband for the low carrier on the low carrier side, and the minimum guardband for the high carrier on the high carrier side.

For some configurations, if Foffset is calculated based on µ=2, there is insufficient guardband for one or both carriers with 30 kHz carriers and the max of the 30 kHz minimum guardbands:

Figure 3 Foffset,low for 60 + 100 MHz intra-band contiguous CA

Figure 4 Foffset,high for 60 + 100 MHz intra-band contiguous CA

Observation 2: For some configurations, if Foffset is calculated based on µ=2, there is insufficient guardband for one or both carriers with 30 kHz carriers and the max of the 30 kHz minimum guardbands.

The guardbands for NR channel bandwidths do not reflect the actual needs, but the available channel bandwidth when the number of RBs and half an SCS are subtracted from the nominal channel bandwidth. From 38.101-1 5.3.3:

NOTE:	The minimum guardbands have been calculated using the following equation: GBChannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1 and GBChannel expressed in kHz.

Observation 3: The guardbands for NR channel bandwidths do not reflect the actual needs, but the available channel bandwidth when the number of RBs and half an SCS are subtracted from the nominal channel bandwidth.

There is no technical reason that a 30, 35, 40, 45, 50, 70 or 80 MHz carrier require a higher minimum guardband than a 100 MHz carrier.

Observation 4: There is no technical reason that a 30, 35, 40, 45, 50, 70 or 80 MHz carrier require a higher minimum guardband than a 100 MHz carrier.

We see the following three options for how to proceed:

- Option 1: Keep the spec as is

  - For some configurations there is not enough room for the 30 kHz guardbands with the 30 kHz RBs

  - Discrepancy between 38.101-1 and 38.104 persists

- Option 2: Copy the formulas for Foffset,low  and Foffset,high from 38.104 to 38.101-1

  - Aligns 38.101-1 and 38.104

  - Compatible with single carrier configurations

  - Doesn’t reflect the max guardband between the two carriers. Not sure if this is necessary, though.

- Option 3: Calculate Foffset,low  and Foffset,high based on the deployed SCS if they are both the same (30 kHz SCS for TDD bands)

  - Different than the single carrier configuration

  - Discrepancy between 38.101-1 and 38.104 persists

Proposal 1: RAN4 to decide between the following options for how to proceed on the intra-band contiguous CA specification issue:

- Option 1: Keep the spec as is

  - For some configurations there is not enough room for the 30 kHz guardbands with the 30 kHz RBs

  - Discrepancy between 38.101-1 and 38.104 persists

- Option 2: Copy the formulas for Foffset,low  and Foffset,high from 38.104 to 38.101-1

  - Aligns 38.101-1 and 38.104

  - Compatible with single carrier configurations

  - Doesn’t reflect the max guardband between the two carriers. Not sure if this is necessary, though.

- Option 3: Calculate Foffset,low  and Foffset,high based on the deployed SCS if they are both the same (30 kHz SCS for TDD bands)

  - Different than the single carrier configuration

  - Discrepancy between 38.101-1 and 38.104 persists

Proposal 2: RAN4 should adopt Option 2, Copy the formulas for Foffset,low  and Foffset,high from 38.104 to 38.101-1.

A CR has been submitted to reflect this change in R4-25xxxxx [1].

# Conclusion

Observation 1: For Foffset,low and Foffset,high for intra-band contiguous CA, 38.101-1 uses the maximum of the minimum guardband for the low carrier bandwidth and the minimum guardband for the high carrier for both the low and high guardband. 38.104 uses the minimum guardband for the low carrier on the low carrier side, and the minimum guardband for the high carrier on the high carrier side.

Observation 2: For some configurations, if Foffset is calculated based on µ=2, there is insufficient guardband for one or both carriers with 30 kHz carriers and the max of the 30 kHz minimum guardbands.

Observation 3: The guardbands for NR channel bandwidths do not reflect the actual needs, but the available channel bandwidth when the number of RBs and half an SCS are subtracted from the nominal channel bandwidth.

Observation 4: There is no technical reason that a 30, 35, 40, 45, 50, 70 or 80 MHz carrier require a higher minimum guardband than a 100 MHz carrier.

Proposal 1: RAN4 to decide between the following options for how to proceed on the intra-band contiguous CA specification issue:

- Option 1: Keep the spec as is

  - For some configurations there is not enough room for the 30 kHz guardbands with the obs30 kHz RBs

  - Discrepancy between 38.101-1 and 38.104 persists

- Option 2: Copy the formulas for Foffset,low  and Foffset,high from 38.104 to 38.101-1

  - Aligns 38.101-1 and 38.104

  - Compatible with single carrier configurations

  - Doesn’t reflect the max guardband between the two carriers. Not sure if this is necessary, though.

- Option 3: Calculate Foffset,low  and Foffset,high based on the deployed SCS if they are both the same (30 kHz SCS for TDD bands)

  - Different than the single carrier configuration

  - Discrepancy between 38.101-1 and 38.104 persists

Proposal 2: RAN4 should adopt Option 2, Copy the formulas for Foffset,low  and Foffset,high from 38.104 to 38.101-1.

# Reference

[1] R4-25xxxxx “CR for 38.101-1: Correction for Contiguous Intra-band CA,” T-Mobile USA