3GPP TSG-RAN WG4 Meeting #115                                                          R4-2507969

Malta, MT, 19th – 23th May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1: the introduction of ATG UE in clause 6.3J and 6.4J |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RF requirement for ATG UE in Rel-19 is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | According to the work split, this CR is to capture the requirement for output dynamic range and transmitter signal quality requirement for ATG UE supporting UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement is not complete for ATG UE in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J and 6.4J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3J	Output power dynamics for ATG

6.3J.1	Minimum output power for ATG

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the sum of the mean power from all antenna connectors or all TAB connectors in at least one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3J.1-1for ATG UE with omni-directional antenna and in Table 6.3J.1-2 for ATG UE with antenna array.

Table 6.3J.1-1: Minimum output power for ATG UE with omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 6.3J.1-2: Minimum output power for ATG UE with antenna array

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -19 | -19+10log10 (BWChannel /20) | -19+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

6.3J.2	Transmit OFF power for ATG

The transmit OFF power is defined as the mean power at each transmit antenna connector or each TAB connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power requirements as specified in clause 6.3.2 are applicable for ATG UE.

6.3J.3	Transmit ON/OFF time mask for ATG

The transmit ON/OFF time mask requirements as specified in clause 6.3.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

6.3J.4	Power control for ATG

The power control requirements specified in clause 6.3.4 are applicable for ATG UE with all transmit antenna connectors or all TAB connectors.

<Start of the change>

6.3Ja	Output power dynamics for ATG UE with UL MIMO

6.3Ja.1	Minimum output power for ATG UE with UL MIMO

For ATG UE with two transmit antenna connectors or [two groups of TAB connectors] within antenna array in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power from all antenna connectors or all TAB connectors in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3J.1-1.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.3J.1 apply when TxD is not indicated.

6.3Ja.2	Transmit OFF power for ATG UE with UL MIMO

The transmit OFF power is defined as the mean power at each transmit antenna connector or each TAB connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power requirements as specified in clause 6.3.2 are applicable for ATG UE with UL MIMO.

6.3Ja.3	Transmit ON/OFF time mask for ATG UE with UL MIMO

The transmit ON/OFF time mask requirements as specified in clause 6.3.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

6.3Ja.4	Power control for ATG UE with UL MIMO

For ATG UE supporting UL MIMO, the power control tolerance applies to the sum of output powers from all transmit antenna connectors of ATG UE with omni-directional antenna or the sum of output powers from all TAB connectors of ATG UE with antenna array .

The power control requirements specified in clause 6.3J.4 apply to ATG UE with all transmit antenna connectors or all TAB connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in clause 6.2J.1.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.3.4 apply when TxD is not indicated.

<Next of the change>

6.4J	Transmit signal quality for ATG

6.4J.1	Frequency error for ATG

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency per layer shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [7].

NOTE 1:	the ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

6.4J.2	Transmit modulation quality for ATG

The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

6.4Ja	Transmit signal quality for ATG UE with UL MIMO

6.4Ja.0	General

For a ATG UE supporting UL MIMO, the requirements in this section are defined per layer or as the sum of emissions from all UE antenna connectors or two groups of TAB connectors to account for the UL MIMO scheme.

Alternatively, when applicable, requirements may be verified per antenna connector or per group of TAB connectors using 2-layer UL MIMO transmission with codebook of, and a configuration defined in Table 6.4J.0-1.

Table 6.4J.0-1: UL MIMO configuration for per connector measurements or per group of TAB connectors

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

6.4Ja.1	Frequency error for ATG UE with UL MIMO

The frequency error requirement as specified in clause 6.4J.1 are per layer applicable for ATG UE.

6.4Ja.2	Transmit modulation quality for ATG UE with UL MIMO

6.4Ja.2.0	General

For UE supporting UL MIMO, the transmit modulation quality requirements are specified based on measurements made at each transmit antenna connector or each group of TAB connectors.

For UE with two transmit antenna connectors or [two groups of TAB connectors] within antenna array in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in clause 6.4.2.1 apply per layer.

For UE with two transmit antenna connectors or [two groups of TAB connectors] within antenna array in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2-1 which is defined in clause 6.4.2.2 apply per layer

For UE with two transmit antenna connectors or [two groups of TAB connectors] in closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.4.2.3-1 which is defined in clause 6.4.2.3 apply at each transmit antenna connector or the summation of per group of TAB connectors.

For UE with two transmit antenna connectors or [two groups of TAB connectors] in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in clause 6.4.2.4 apply per layer.

The requirements shall be met with the UL MIMO configurations specified in Table 6.2Ja.1-2

6.4Ja.3	Time alignment error for UL MIMO

For UE(s) with two antenna connectors or  two groups of TAB connectors supporting UL MIMO, this requirement applies to frame timing differences between transmissions on two transmit antenna connectors or two group of TAB connectors in the closed-loop spatial multiplexing scheme.

The time alignment error (TAE) is defined as the average frame timing difference between the two transmissions on different transmit antenna connectors or different group of TAB connectors.

For UE(s) with two transmit antenna connectors or two groups of TAB connectors, the Time Alignment Error (TAE) shall not exceed 130 ns.

<End of the change>